Embedded Protocol Analyzing Classifier Design Document

Version 1.0.8

April 25, 2007

Technica Corporation

45245 Business Court, Suite 300

Dulles, VA 20166

703.662.2000 phone

703.662.2001 fax

www.technicacorp.com

Table of Contents

1	Introduction	1

2	Design Objectives	2

3	Functional Description	3

3.1	Protocol Memory	4

3.2	Assembler	4

3.3	Length Saver	5

3.3.1	Calculating Length	6

3.4	Protocol Saver	6

3.4.1	Jump Block	6

3.5	Port Block	7

3.5.1	Loading New Port Values	9

4	Summary	9

Acronym List	10

List of Figures

Figure 1.  EmPAC Top Level Design	2

Figure 2.  Internal Architecture of EmPAC	3

Figure 3.  Phy_data Registers	5

Figure 4.  Port Block Functional Description	8

List of Tables

Table 1.  Protocol Memory Signal Description	4

Table 2.  Assembler Signal Description	5

Table 3.  Length Saver Signal Description	5

Table 4.  Length Saver Field Type Requirement	6

Table 5.  Protocol Saver Signal Description	6

Table 6.  Protocol Types and Values	7

Table 7.  Jump Block Signal Description	7

Table 8.  Port Block Signal Description	8

# Introduction

The Embedded Protocol Analyzing Classifier (EmPAC) is designed to perform the task of packet classification through protocol analysis.  Its goal is to take an unclassified byte stream coming from the Ethernet Physical Layer Interface (PHY) and partition and classify the data blocks into corresponding protocol fields.  These include header information such as source and destination address, header and payload sizes, and protocol flags, as well as the payload fields themselves.

The partitioned data fields are output on a 32-bit bus, accompanied simultaneously by a unique 16-bit field type to identify not only the field’s context (its associated protocol), but also its particular relevance (e.g., source address, payload size, and header checksum).  EmPAC will support an interface to modules capable of analyzing certain Transmission Control Protocol (TCP) or User Datagram Protocol (UDP) application layer protocols.  Therefore, EmPAC must verify if the incoming application layer protocol is supported in the design.  When supported, this verification must also be accompanied by the port value of the supported protocol.  These port values are read from the PHY when the TCP or UDP source or destination port value arrives.  Figure 1 depicts the top-level design of EmPAC.  The associated signals are defined as follows:

- Field_data contains the 32-bit partitioned data field.

- Field_type contains the 16-bit field data identifier.

- Data_ready, when asserted, validates the data located on field_type and field_data once the 32-bit partitioned data field is available.

- Port_found, when asserted, the application layer protocol is supported in the design.

- Port_value contains the value of the application layer protocol port number.

- Load, when asserted, indicates that a new application layer protocol is available to be loaded into the design.

- Load_data contains the 16-bit application layer protocol value.

Figure 1.  EmPAC Top Level Design

EmPAC supports the following internet protocols:

- Physical Layer

- Ethernet

- Data Link Layer

- Address Resolution Protocol (ARP)

- Network Layer

- Internet Protocol Version 4 (IPv4)

- Internet Protocol Version 6 (IPv6)

- Transport Layer

- TCP

- UDP

Thousands of application layer protocols appear after the transport layer.  Therefore, EmPAC is designed with re-configurability, allowing the user to update the design with new application layer protocols.

# Design Objectives

Figure 2 illustrates the internal design of the EmPAC first design option.

Figure 2.  Internal Architecture of EmPAC

# Functional Description

EmPAC is capable of taking an incoming Ethernet frame stream and classifying each protocol field into its field type and placing the corresponding data onto the field_data signal.  The field_type and field_data signals are validated by asserting the data_ready signal.  This assures that the field_type and field_data have been completely assembled into a 32-bit vector.  The load and load_data signals are used for reconfiguration when loading new application layer protocols.  When the incoming application layer protocol is found, the port_found signal is asserted, and the corresponding application layer protocol value is placed on the port_value signal.  The following sections explain the functionality of each module in detail.

## Protocol Memory

This module contains the protocol information of all supported protocols listed below.

- Ethernet

- ARP

- IP

- IPv4

- IPv6

- TCP

- UDP

The information contained in this memory is used to determine which field types are expected on the incoming data stream from the PHY.  It will be embedded in the design using case statements to generate the signals listed in Table 1.

Table 1.  Protocol Memory Signal Description

| Signal Name | Direction | Bit Width | Description |
| --- | --- | --- | --- |
| Field_width | OUT | 2 | Contains the width in bytes of each protocol field |
| Branch_indicator | OUT | 1 | Indicates when a branch to a different protocol within the protocol memory is required |
| Protocol_indicator | OUT | 1 | Indicates that a protocol type is available on the incoming packet |
| Length_indicator | OUT | 1 | Indicates that length information is available on the incoming packet |
| Variable_length_field | OUT | 1 | Indicates that the incoming field has a variable length |
| Port_indicator | OUT | 1 | Indicates that the incoming field contains port information |
| Field_type | OUT | 16 | Equivalent to the protocol memory address. It is also used as the protocol field classification values |
| Jump_address | IN | 8 | Address of protocol to jump to when the branch indicator is asserted |

Embedding the data is possible because the protocol information stored will not require any alterations.  This will present a faster frequency than the alternative solution of using a Read Only Memory (ROM) or a Random Access Memory (RAM).  These memories tend to have maximum frequencies associated with them, which may limit the overall frequency of the design.

## Assembler

This component is responsible for assembling the 8-bit data stream from the PHY into a 32-bit data stream.  Bits 0 through 1 of length are used to determine how many bits should be zero padded when the length is less than 4 bytes.  In the event that the length of the current field is greater than 4 bytes, the assembler will then assemble the entire 8-bit data stream into 32 bits without zero pads.  If the length of variable length field is not a multiple of four, the remaining bits required to complete the 32-bit vector will be padded with zeros.  Table 2 describes the ports and their functions.

Table 2.  Assembler Signal Description

| Signal Name | Direction | Bit Width | Description |
| --- | --- | --- | --- |
| Length | IN | 16 | Contains the remaining length in bytes of the incoming protocol field |
| Phy_data | IN | 8 | Contains the incoming packet to be classified |
| Field_data | OUT | 1 | Contains the data corresponding to each protocol field. When the protocol field is less than 32 bits, the remaining bits will be zero padded |
| Data_ready | OUT | 8 | Asserted when the 32-bit field data is complete |

To avoid any possible delays, the incoming data stream is stored in registers.  The assembler uses the concatenation of Q4, Q3, Q2, and Q1, respectively, as the 32 bits to be assembled as illustrated in Figure 3.  The concatenation of D4, D3, D2, and D1 is available to all the other components in the design one cycle prior to assembling so that the necessary branching and length calculations may be determined.

Figure 3.  Phy_data Registers

## Length Saver

The Length Saver is responsible for storing the length of the upcoming variable length field.  Once activated, incoming field_data is stored temporarily.  When the length of the current field type is unknown, the length stored by the Length Saver is used as the length of the current field type in place of the field width.  The input and output signals of this module are listed in Table 3.

Table 3.  Length Saver Signal Description

| Signal Name | Direction | Bit Width | Description |
| --- | --- | --- | --- |
| Length_indicator | IN | 1 | When asserted, the value located on field_data will be stored |
| Field_data | IN | 32 | Contains the assembled packet to be classified |
| Field_type | IN | 8 | Equivalent to the protocol memory address. It is also used as the protocol field classification value |
| Length | OUT | 16 | Contains the length of the upcoming variable length field |

The field_type is used to determine which protocol field is currently available.  This is necessary in order to determine which protocol the length came from so that the proper calculations may be made to generate the correct length of the upcoming variable length field.

### Calculating Length

To calculate the length of the upcoming variable length field, the Length Block uses the following formulas:

- In the case where the network layer header length is greater than 14 bytes

- Length = total packet length – network layer header length

- Otherwise

- Length = total packet length – transport layer header length

The total packet length and the transport layer length are received from the field_data signal based on the value contained on field_type once the length_indicator signal is asserted.  Table 4 illustrates the associated field_type value required to retrieve the data properly.

Table 4.  Length Saver Field Type Requirement

| Data Type | Field_type Value (in hexadecimal) |
| --- | --- |
| Ipv4 Header Length | 18 |
| Total Packet Length | 1A |
| TCP Header Length | 27 |
| UDP Header Length | 2F |
| Ipv6 Header Length | 35 |

## Protocol Saver

The Protocol Saver has the responsibility of saving all protocol information on the incoming packet.  This value is used to determine the location in memory in which the Protocol Memory should branch.  Once activated, this component will begin storing the protocol value located on the incoming data stream.  This protocol value is then delivered to the Jump Block for further processing detailed in the next section.  Table 5 contains the signals to and from the Protocol Saver with the associated descriptions.

Table 5.  Protocol Saver Signal Description

| Signal Name | Direction | Bit Width | Description |
| --- | --- | --- | --- |
| Protocol_indicator | IN | 1 | When asserted, the value located on field_data will be stored |
| Field_data | IN | 32 | Contains the assembled packet to be classified |
| Protocol_type | OUT | 16 | Contains the value of the next protocol type to be expected on the incoming packet |

### Jump Block

Similar to the Protocol Memory, all jump addresses are also embedded into the system via case statements.  When a particular protocol is delivered to this component, it will return the address location of this protocol in the protocol memory.  Table 6 illustrates the supported protocol types and values with their corresponding address location in the protocol memory.  Table 7 represents the signals to and from the Jump Block with the corresponding signal descriptions.

Table 6.  Protocol Types and Values

| Protocol Type | Protocol Value | Protocol Memory Address |
| --- | --- | --- |
| Ehternet | 0000 | 00 |
| ARP | 0806 | 07 |
| Ipv4 | 0800 | 18 |
| Ipv6 | 86DD | 32 |
| TCP | 0006 | 23 |
| UDP | 0011 | 2D |

Table 7.  Jump Block Signal Description

| Signal Name | Direction | Bit Width | Description |
| --- | --- | --- | --- |
| Protocol_type | IN | 16 | Contains the width in bytes of each protocol field |
| Branch_indicator | IN | 1 | Indicates when a branch to a different protocol within the protocol memory is required |
| Jump_address | OUT | 8 | Address of protocol to jump to when the branch indicator is asserted |

## Port Block

Because there are thousands of port values available, it would be infeasible to embed all port values into the design.  Therefore, the Port Block contains dynamically reconfigurable port registers.  These registers contain the values of all supported application layer protocols.  The Port Block will contain 69 preloaded port values, with the possibility of holding up to 128 different port values total.  When the Port_indicator is asserted, this component will store the port value located on the incoming packet.  Port_found will then be asserted only if the port_value has been located within the port registers.  This is accomplished by loading the values of each register into one comparator, as shown in Figure 4.  The comparator will simultaneously compare the value of each register with the value located on the field_data signal.

Figure 4.  Port Block Functional Description

To load new ports into the design, a load signal must be asserted, and the port_value must be placed simultaneously onto the load_data signal.  The new ports will then be loaded sequentially, beginning with the first available address.  If no new addresses are available (i.e., all 128 locations are occupied), new ports will be loaded, beginning with the first address.  
Table 8 descibes the functionality of each signal to and from the Port Block.

Table 8.  Port Block Signal Description

| Signal Name | Direction | Bit Width | Description |
| --- | --- | --- | --- |
| Load | IN | 4 | When asserted, new port values may be loaded |
| Load_data | IN | 1 | Contains the port values to be loaded once load signal is asserted |
| Port_indicator | IN | 1 | Indicates that a port value is available on the incoming packet |
| Port_found | OUT | 1 | Indicates that current application layer protocol is supported |
| Port_value | OUT | 16 | Contains the value of the supported application layer protocol |
| Field_data | IN | 32 | Contains the assembled packet to be classified |

### Loading New Port Values

The load_data value is available to all registers; however, the load signal is connected to each register via the load bus (illustrated in Figure 4).  Because all new port values are loaded into the port registers sequentially, a 7-bit counter is used to select the proper register to be loaded.  For example, when the load signal is asserted, if the value contained in the counter is 1010010 (decimal value 82), only the load signal of the 82nd register will be asserted, allowing this register to take the new port value located on load_data.  Once all port registers have been filled, the counter will reset to zero to overwrite the port values that have been supported the longest.

# Summary

EmPAC is capable of taking an unclassified byte stream coming from the PHY and partition and classify the data blocks into corresponding protocol fields.  It supports the following protocols:

- Physical Layer

- Ethernet

- Data Link Layer

- Address Resolution Protocol (ARP)

- Network Layer

- Internet Protocol Version 4 (IPv4)

- Internet Protocol Version 6 (IPv6)

- Transport Layer

- TCP

- UDP

- EmPAC also supports all application layer protocols appearing after the supported transport layer protocols.  Its reconfiguration ability allows new application layer protocols to also be loaded during runtime.

Acronym List

| Acronym | Definition |
| --- | --- |
| ARP | Address Resolution Protocol |
| EmPAC | Embedded Protocol Analyzing Classifier |
| IP | Internet Protocol |
| IPv4 | Internet Protocol version 4 |
| IPv6 | Internet Protocol version 6 |
| PHY | Physical Layer Interface |
| TCP | Transmission Control Protocol |
| UDP | User Datagram Protocol |