# Aquarius

A Pipelined RISC CPU

# What is Aquarius

Aquarius is a Core IP of pipelined RISC CPU and can execute SuperH-2* instructions. Aquarius and related information are released to OpenCores Organization web site (www.opencores.org). You can freely download and use all resources on the site.*: SuperH is a trademark of Renesas Technology Corp.

# What is SuperH

1. SuperH is a very popular CPU core. The software development environments such as C compiler have been well prepared. The GNU C compiler for SuperH is very famous and easy to get.

2. SuperH-2 is a CPU for MCU (Micro Controller Unit). Then the CPU need not handle complex exception recovering such as memory fault exception from MMU (Memory Managing Unit). This means SuperH-2 has simple structure, easiness to design, and it does not consume many logic gates and power.

3. All SuperH-2 instructions have 16bit length. It also makes the hardware very simple. And most important aspect from 16bit fixed length of instructions is that the object code size compiled from C source programs becomes very small.

# Aquarius Features

1. Instruction set is compatible with SuperH-2.

2. Bus interface is compatible with WISHBONE.

3. Aquarius is written in Verilog RTL codes.

4. Aquarius has verified on a test bench by both vector simulation and FPGA implementation with GNU Assembler and C Compiler.

5. Interrupts and Exceptions are fully supported.

6. Low Power Mode (SLEEP) is supported.

7. Some small applications are provided including debugging monitor for a FPGA board.

8. A complete Document with over 100 pages, describing Usage of Aquarius, FPGA Implementation and Inside Aquarius, is provided.

CPU Block Diagram

# Deliverables

1. Verilog RTL codes for CPU, and Test Bench including modules comprising MCU, such as UART, System Controller, Parallel I/O Interface, and Internal Memories (ROM/RAM).

2. Verification Resources, such as Converter from S-format to Verilog format for ROM coding, and Assemble Source Programs for Vector Simulation.

3. FPGA Resources, such as Circuit Schematics of Interface Board, Converter from S-format to Xilinx BlockRAM INIT statements for RAM initialization, and a sample of User Constraints File.

4. Small Applications written in C Sources including LCD Test Program, Clock using interval interrupt, Debugging Monitor, and Calculation of Circular Constant (Pi).

Aquarius RTL Tree

# Download

Documents, RTL Source codes and related Tools can be downloaded from the OpenCores CVS directory "Aquarius".

# FPGA Verification System

Aquarius has verified by using FPGA Verification System. It consists of Xilinx VirtexE, LCD Display Interface, Key Matrix Interface and RS-232C Interface.	  FPGA System

# Performance

Aquarius CPU core and related peripheral modules have been configured in both Xilinx and Altera. Following table shows their performance.

| FPGA Device | Performance |
| --- | --- |
| Xilinx VirtexE (XCV300E) | 2753 slices @21MHz |
| Altera Stratix (EP1S10) | 7499 cells @31MHz |

# Contact

Thorn Aitch :	thorn_aitch@opencores.org