Color Converter

Authors : Michail Tsvetkov,

Vyacheslav Gulyaev

cms@mail.ru

vv_gulyaev@mail.ru

Rev. 0.9

July 30, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0.8 | 06.17.06 | Vyacheslav Gulaev | Initial release. |
| 0.9 | 07.30.06 | Michael Tsvetkov | Add some text and IO Port description. |

Contents

Introduction	1

Architecture	2

Operation	3

Registers	4

List of Registers	4

Register 1 – Description	5

IO Ports	6

Appendix A	7

Appendix B	8

Index	9

Introduction

The main purpose of the core is a color transform tasks such as CIE XYZ<->RGB, different RGB<->RGB and RGB<->YCbCr operations. The main part of color conversions from one to another color system concludes in 3x3 matrix multiplication with vector addition. Therefore the present core performs true 3x3 matrix multiplication and vectors addition of the input data stream.

In the package ccfactors_pkg.vhd presents matrixes for all of the most popular conversions. You can use these defines or type own factors.

Input DATA_WIDTH is changeable and defines by the GENERIC param of the module. Precision of the matrix factors also is changeable. Default precision – 10-E6. Better than enough for color conversion tasks.

Architecture

The input vector [X1 X2 X3] adds with shift vector [B1x B2x B3x]. The result vector goes to vector multiplier, where it multiplies on matrix of factors Aij. The result of multiply after rounding adds with shift vector [B1y B2y B3y]. The result of conversion goes to output after round. The shift register is necessary for generating RDY signal and easy integration with FIFO.

Figure 1. Block diagram of color converter

(bus widths shown for the 8-bit input and 23-bit factors)

Operation

To correct conversion from one to another color system is necessary to load corresponding matrix of factors and shift vectors in FPGA. Before start of conversion is necessary to reset all registers in ‘0’, by establish the RESET signal in ‘0’. When RESET signal back in ‘1’ it is possible to enter sequences for conversion. The read of result of conversion possible when output signal DOUT_RDY is establish in ‘1’.

Figure 2. The example of operation of color converter.

Registers

Registers will be added in the future with adding wishbone-compliance.

### List of Registers

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |

Table 1: List of registers

### Register 1 – Description

(You shall choose the style of register you prefer. Do not use both options in one and the same document.)

| Bit # | Access | Description |
| --- | --- | --- |

Reset Value:

Reg_Name: 0000h

| 31 | 30 | 29 | 28 | ... | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 2: Description of registers

Reset Value:

Reg_Name: 0000h

IO Ports

All ports, except  rstn, synchronous with rising edge of the clk

| Port | Width | Direction |  | Description |
| --- | --- | --- | --- | --- |
| clk | 1 | Input | Clock Input |  |
| rstn | 1 | Input | Reset Input, active level = ‘0’ |  |
| data_ena | 1 | Input | Signal of the valid input data |  |
| dout_rdy | 1 | Output | Signal of the valid output data |  |
| X1 | DATA_WIDTH | Input | First input vector’s component |  |
| X2 | DATA_WIDTH | Input | Second input vector’s component |  |
| X3 | DATA_WIDTH | Input | Third input vector’s component |  |
| B1x | FACTORS_WIDTH | Input | Input add factor’s component |  |
| B2x | FACTORS_WIDTH | Input | Input add factor’s component |  |
| B3x | FACTORS_WIDTH | Input | Input add factor’s component |  |
| A11 | FACTORS_WIDTH | Input | Matrix factor |  |
| A12 | FACTORS_WIDTH | Input | Matrix factor |  |
| A13 | FACTORS_WIDTH | Input | Matrix factor |  |
| A21 | FACTORS_WIDTH | Input | Matrix factor |  |
| A22 | FACTORS_WIDTH | Input | Matrix factor |  |
| A23 | FACTORS_WIDTH | Input | Matrix factor |  |
| A31 | FACTORS_WIDTH | Input | Matrix factor |  |
| A32 | FACTORS_WIDTH | Input | Matrix factor |  |
| A33 | FACTORS_WIDTH | Input | Matrix factor |  |
| B1y | FACTORS_WIDTH | Input | Output add factor’s component |  |
| B2y | FACTORS_WIDTH | Input | Output add factor’s component |  |
| B3y | FACTORS_WIDTH | Input | Output add factor’s component |  |
| Y1 | DATA_WIDTH | Output | First output vector’s component |  |
| Y2 | DATA_WIDTH | Output | Second output vector’s component |  |
| Y3 | DATA_WIDTH | Output | Third output vector’s component |  |

Table 3: List of IO ports

Different color convertions

This section may be added to outline different specifications.

Matlab verification results

This section may be added to outline different specifications.