R6502 IP Core
Specification

Author: Jens Gutschmidt

scantara2003@yahoo.de

Rev. 0.5

July 30, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 12/18/06 | Jens Gutschmidt | First Draft |
| 0.2 | 01/02/07 | Jens Gutschmidt | Pictures of FSM‘s |
| 0.3 | 08/20/08 | Jens Gutschmidt | - Tables and timing diagrams - Deleting pictures of FSM |
| 0.4 | 10/01/08 | Jens Gutschmidt | - New ideas for timing diagrams |
| 0.5 | 01/02/09 | Jens Gutschmidt | - Textual changes / spell checking - Insert R6502_TC block diagram |

Contents

Error! No table of contents entries found.

1	1

2	2

3	5

ADC	5

AND	7

ASL	7

BCC	9

BCS	9

BEQ	9

BIT	9

BMI	9

BNE	9

BPL	9

BRK	9

BVC	9

BVS	9

CLC	10

CLD	11

CLI	12

CLV	13

CMP	14

CPX	14

CPY	14

DEC	14

DEX	15

DEY	16

EOR	17

INC	17

INX	18

INY	19

JMP	19

JSR	20

LDA	21

LDX	22

LDY	23

LSR	23

NOP	24

ORA	25

PHA	26

PHP	26

PLA	26

PLP	26

ROL	26

ROR	26

RTI	26

RTS	26

SBC	26

SEC	27

SED	28

SEI	29

STA	29

STX	29

STY	29

TAX	30

TAY	31

TSX	32

TXA	33

TXS	34

TYA	35

4	36

List of Registers	36

Register 1 – Description	36

5	37

6	38

Appendix A	39

Appendix B	40

Index	41

List of Figures

Figure (1): 6502 architecture	2

Figure (1): CLC – Timing Diagram	10

Figure (2): CLD – Timing Diagram	11

Figure (3): CLI – Timing Diagram	12

Figure (4): CLV – Timing Diagram	13

Figure (5): DEX – Timing Diagram	15

Figure (6): DEY – Timing Diagram	16

Figure (7): INX – Timing Diagram	18

Figure (8): INY – Timing Diagram	19

Figure (9): JSR – Timing Diagram	20

Figure (10): NOP – Timing Diagram	24

Figure (11): SEC – Timing Diagram	27

Figure (12): SED – Timing Diagram	28

Figure (13): SEI – Timing Diagram	29

Figure (14): TAX – Timing Diagram	30

Figure (15): TAY – Timing Diagram	31

Figure (16): TSX – Timing Diagram	32

Figure (17): TXA – Timing Diagram	33

Figure (18): TXS – Timing Diagram	34

Figure (19): TYA – Timing Diagram	35

List of Tables

Table (1): CLC – Short Reference	5

Table (2): ADC – Short Reference	6

Table (2): AND – Short Reference	7

Table (3): CLC – Short Reference	10

Table (4): CLD – Short Reference	11

Table (5): CLI – Short Reference	12

Table (6): CLV – Short Reference	13

Table (7): DEX – Short Reference	15

Table (8): DEX – Short Reference	16

Table (9): EOR – Short Reference	17

Table (10): INX – Short Reference	18

Table (11): INY – Short Reference	19

Table (12): JSR – Short Reference	20

Table (13): LDA – Short Reference	21

Table (14): LDX – Short Reference	22

Table (15): LDY – Short Reference	23

Table (16): NOP – Short Reference	24

Table (17): OR – Short Reference	25

Table (18): SEC – Short Reference	27

Table (19): CLD – Short Reference	28

Table (20): SEI – Short Reference	29

Table (21): TAX – Short Reference	30

Table (22): TAY – Short Reference	31

Table (23): TSX – Short Reference	32

Table (24): TXA – Short Reference	33

Table (25): TXS – Short Reference	34

Table (26): TYA – Short Reference	35

Introduction

The Central Processing Unit (CPU) 6502 was introduced at 1976 by Commodore Computers. It is an 8 Bit processor which is well known worldwide. The also most known computer system in the 70s/80s was the APPLE and based on this famous 6502.

In this century building of little computer systems based on an 8 Bit architecture is easier than ever before. Many CAD/CAM tool are existing on the market to give you help to do this job.

In the last decade the technology give us more and more possibilities to reach our dreams – higher, faster, wider. FPGA’s (Field Programmable Gate Arrays) and CPLD’s (Complex Programmable Logic Devices) are coming up. They shortening the time of development, simulation and verification of such systems dramatically. On the other hand stands the rising complexility of designs and well knowing of desired description languages – VHDL (Very high scale integration Hardware Description Language) or Verilog.

Many operations and functions of computer tasks aren’t need really the power of “modern” processors today like Intel’s Pentium. Nevertheless it is not usable – some times also impossible - to build simple systems with discrete IC’s. The necessity of flexibility compel us the using of high tech parts and tools...to build a “simple” system.

The using of standard IP’s (Intellectual Properties) is the key to reach that goal.

Now talking about the specification of the 6502 True Cycle Core IP. Written in VHDL but developed with Mentor’s HDL Designer. The 6502 is very easy to handle for people were have experience with other cpu’s. Also suitable for beginners who will learning to build her own first cpu system.

Architecture

The following figure shows the internal architecture of the Commodore’s 6502.

Figure (1): 6502 architecture

First, for more clearance and better understanding of the 6502 True Cycle Core IP let me summarize the most important quality of Commodore’s 6502.

This architecture based on asynch logic. The two phase clock is a special attribute of that. The Data Bus is only active while clock phase PH1 is “1” for reading and writing (PH0 is “0”). All transfers of data are occur at active phase of PH1.

The whole 6502 is fully statically, so all registers holding her values if the clocking will be going to D.C. Internally all operation of 16 Bit are splitted into two 8 Bit busses. This is very interesting and causes one more clock cycle in some operations (see “PCL”, “PCH” -> Program Counter Low/High) which can operate over page boundaries.

Address lines are 16 Bit wide. 65.536 addresses for byte access are possible. There are no differences exist between memory and I/O cycles. All I/O devices must be connected via memory mapped I/O. The mnemonics are also all the same for both. The usage of address space is normally organized form top to the button “ROM – I/O – RAM” (0xFFFF – 0x0000).

Only one pin R/W controls the read and write operations.

The 6502 can hold in every cycle – except write cycles - by applying RDY. So wait states can be generate by using RDY.

There are two interrupts IRQ and NMI. IRQ is mask able, NMI is not. Note that RES is internally handled as an interrupt, but is resetting the 6502 to the starting point of operation at vector address 0xFFFF. Every interrupt has its own vector which can point elsewhere to the 16 Bit address space. The vectors are located from 0xFFFF to 0xFFFA.

The Stack Pointer is 256 Bytes deep and is hard-wired to 0x01FF (internally 0x0200 –0x01FF going to the address bus) for the first memory location and grows downward to 0x0100. Decrementing again will produce a wrap around back to 0x01FF.

Arithmetical operations like ADC and SBC can handle binary values from 0-255 and decimal values from 0-99 without using other op codes. Only one user changeable bit into the Status Register determine the exact arithmetic mode of that operations.

The 6502 generally operates into a pipeline mode. That means that a finish of an OP code falls every time into a fetch cycle of the next OP. This save one clock cycle

Every operation consume between two and seven clock cycles.

To build an useful und backward software and hardware compatible VHDL Core for the 6502 many unavoidable changes may be forced to simulation and verification before any real core will be made.

The requirements:

	Vendor indepent for implementation in any FPGA

	True Cycle for all operations as described in original publications

	No “Mixed Mode” – Only 6502, not 65C02 -> for performance and area

	No fantastic or useful extensions – as like the original 6502 as possible

 	Don’t implement the “undocumented OP’s” at this time -> goodie for future releases

	Only one clock

	Full synch design

	Operating speed up to 60 MHz – or higher

	Easily to change for future requirement – building variants

	The activity on data and address busses since the execution of OP codes is not forced to be like the original – may be or may be not.

The design based on finite state machines (fsm). Every OP code has its own fsm. Some registers are for internally use only and stores temporally values. These registers are not accessible by the programmer.

Some fsm’s own registers for internally use and don’t share this registers with other fsm’s. This help to decrease fan-in and increase performance.

The next picture shows the hierarchical structure of the R6502 IP core.

Figure (2): R6502_TC IP core architecture

Operation

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | CLC | 18 | 1 | 2 | - | - | - | - | - | - | - | 0 |

Table (1): CLC – Short Reference

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Cycle 4 |  | Cycle 5 |  | Cycle 6 |  | Cycle 7 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a_o | adr |  |  |  |  |  |  |  |  |  |  |  |  |  | adr + |  |
| rd_o | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| d_i |  |  |  |  |  |  |  |  |  |  |  |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |  |  |  |  |  |  | X”00” |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |  |  |  |  |  |  | unchanged |  |
| Operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### ADC

Operation:  Add memory or immediate value to accumulator A with carry (A + M + C → A, C)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate | ADC # Oper | 69 | 2 | 2 | √ | √ | - | - | - | - | √ | √ |  |
| Zero Page | ADC Oper | 65 | 2 | 3 | √ | √ | - | - | - | - | √ | √ |  |
| Zero Page, X | ADC Oper, X | 75 | 2 | 4 | √ | √ | - | - | - | - | √ | √ |  |
| Absolute | ADC Oper | 6D | 3 | 4 | √ | √ | - | - | - | - | √ | √ |  |
| Absolute, X | ADC Oper, X | 7D | 3 | 4* | √ | √ | - | - | - | - | √ | √ |  |
| Absolute, Y | ADC Oper, Y | 79 | 3 | 4* | √ | √ | - | - | - | - | √ | √ |  |
| (Indirect, X) | ADC (Oper, X) | 61 | 2 | 6 | √ | √ | - | - | - | - | √ | √ |  |
| (Indirect), Y | ADC (Oper), Y | 71 | 2 | 5* | √ | √ | - | - | - | - | √ | √ |  |
|  |  |  |  | * => Add 1 if page boundary is crossed |  |  |  |  |  |  |  |  |  |

Table (2): ADC – Short Reference

Example Immediate (assumed A=$FE, C=0):

Address	Bytes		Mnemonic

$9000		69 03		ADC #$03	;

$9002		…		next OP	; A is now $01, C=1, N=0, Z=0, V=0

1111 1110 ($FE)

ADC	0000 0011 ($03)

0000 0001 ($01), C=1, N=0, Z=0, V=0

REQ1: Main features

REQ1

This requirement is mandatory at least for Word documents.

REQ1.1: Automatic capture

The tool shall be able to capture semi-automatically the requirements included in a document and/or in a model. “Semi-automatic” means the text has to be formalized beforehand by the user or another dedicated tool.

This requirement is mandatory at least for Word documents.

END_REQ

### AND

Operation: “AND” memory or immediate value with accumulator A (A ∩ M → A)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate | AND # Oper | 29 | 2 | 2 | √ | - | - | - | - | - | √ | - |  |
| Zero Page | AND Oper | 25 | 2 | 3 | √ | - | - | - | - | - | √ | - |  |
| Zero Page, X | AND Oper, X | 35 | 2 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute | AND Oper | 2D | 3 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute, X | AND Oper, X | 3D | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
| Absolute, Y | AND Oper, Y | 39 | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
| (Indirect, X) | AND (Oper, X) | 21 | 2 | 6 | √ | - | - | - | - | - | √ | - |  |
| (Indirect), Y | AND (Oper), Y | 31 | 2 | 5* | √ | - | - | - | - | - | √ | - |  |
|  |  |  |  | * => Add 1 if page boundary is crossed |  |  |  |  |  |  |  |  |  |

Table (3): AND – Short Reference

Example Immediate (assumed A is $C5):

Address	Bytes		Mnemonic

$9000		29 71		AND #$71	;

$9002		…		next OP	; A is now $41, N=0, Z=0

1100 0101 ($C5)

AND	0111 0001 ($71)

0100 0001 ($41), N=0, Z=0

### ASL

| Operation: Shift Left One Bit (Memory or Accumulator) (C ← ← 0) 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accumulator | ASL A | 0A | 1 | 2 | √ | - | - | - | - | - | √ | √ |
| Zero Page | ASL Oper | 06 | 2 | 5 | √ | - | - | - | - | - | √ | √ |
| Zero Page, X | ASL Oper, X | 16 | 2 | 6 | √ | - | - | - | - | - | √ | √ |
| Absolute | ASL Oper | 0E | 3 | 6 | √ | - | - | - | - | - | √ | √ |
| Absolute, X | ASL Oper, X | 1E | 3 | 7 | √ | - | - | - | - | - | √ | √ |

Table (4): ASL – Short Reference

Example Accumulator (assumed A=$55):

Address	Bytes		Mnemonic

$9000		0A		ASL A		;

$9001		…		next OP	; A is now $AA, N=1, Z=0

ASL	0101 0101 ($55)

1010 1010 ($AA), C=0, N=1, Z=0

### BCC

Operation: Branch on Carry Clear (Branch on C = 0)

| Addressing Mode | Assembly Language Form |  | OP CODE |  | No. Bytes |  | No. Cycles |  | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative | BCC Oper |  | 90 |  | 2 |  | 2* |  | - | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  | * => Add 1 if branch occurs to same page * => Add 2 if branch occurs to different page |  |  |  |  |  |  |  |  |

Table (5): BCC – Short Reference

Example Relative (assumed C=1, same page):

Address	Bytes		Mnemonic

$9000		90 10		BCC #$10	; BCC to same page, - NO BRANCH - jumps to $9002

$9002		…		next OP	; C=1 => no branch, 2 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”90” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (3): BCC – Timing Diagram “no branch, same page”

Example Relative (assumed C=1, different page):

Address	Bytes		Mnemonic

$90FF		90 10		BCC #$10	; BCC to different page, - NO BRANCH - jumps to

; $9101

$9101		…		next OP	; C=1 => no branch, 2 cycles (!!!)

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”90” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (4): BCC – Timing Diagram “no branch, different page”

Example Relative (assumed C=0, same page):

Address	Bytes		Mnemonic

$9000		90 10		BCC #$10	; BCC to same page, - BRANCH - jumps to $9002 + $10

…

$9012		…		next OP	; C=0 => branch, 3 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  | $9012 |  |
| rd_o | 1 |  |  |  |  |  |  |  |
| d_i | X”90” |  | X”10” |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | Fetch next OP |  |

Figure (5): BCC – Timing Diagram “branch, same page”

Example Relative (assumed C=0, different page):

Address	Bytes		Mnemonic

$90FF		90 10		BCC #$10	; BCC to different page, - BRANCH - jumps to

…						; $9111

$9111		…		next OP	; C=0 => branch, 4 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Cycle 4 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  | $9011 |  | $9111 |  |
| rd_o | 1 |  |  |  |  |  |  |  |  |  |
| d_i | X”90” |  | X”10” |  | dummy |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | 1. Read dummy 2. Cor- rect PCH |  | Fetch next OP |  |

Figure (6): BCC – Timing Diagram “branch, different page”

### BCS

Operation: Branch on Carry Set (Branch on C = 1)

| Addressing Mode | Assembly Language Form |  | OP CODE |  | No. Bytes |  | No. Cycles |  | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative | BCS Oper |  | B0 |  | 2 |  | 2* |  | - | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  | * => Add 1 if branch occurs to same page * => Add 2 if branch occurs to different page |  |  |  |  |  |  |  |  |

Table (6): BCS – Short Reference

Example Relative (assumed C=0, same page):

Address	Bytes		Mnemonic

$9000		B0 10		BCS #$10	; BCS to same page, - NO BRANCH - jumps to $9002

$9002		…		next OP	; C=0 => no branch, 2 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”B0” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (7): BCS – Timing Diagram “no branch, same page”

Example Relative (assumed C=0, different page):

Address	Bytes		Mnemonic

$90FF		B0 10		BCS #$10	; BCS to different page, - NO BRANCH - jumps to

; $9101

$9101		…		next OP	; C=0 => no branch, 2 cycles (!!!)

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”B0” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (8): BCS – Timing Diagram “no branch, different page”

Example Relative (assumed C=1, same page):

Address	Bytes		Mnemonic

$9000		B0 10		BCS #$10	; BCS to same page, - BRANCH - jumps to $9002 + $10

…

$9012		…		next OP	; C=1 => branch, 3 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  | $9012 |  |
| rd_o | 1 |  |  |  |  |  |  |  |
| d_i | X”B0” |  | X”10” |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | Fetch next OP |  |

Figure (9): BCS – Timing Diagram “branch, same page”

Example Relative (assumed C=1, different page):

Address	Bytes		Mnemonic

$90FF		B0 10		BCS #$10	; BCS to different page, - BRANCH - jumps to

…						; $9111

$9111		…		next OP	; C=1 => branch, 4 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Cycle 4 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  | $9011 |  | $9111 |  |
| rd_o | 1 |  |  |  |  |  |  |  |  |  |
| d_i | X”B0” |  | X”10” |  | dummy |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | 1. Read dummy 2. Cor- rect PCH |  | Fetch next OP |  |

Figure (10): BCS – Timing Diagram “branch, different page”

### BEQ

Operation: Branch on result zero (Branch on Z = 1)

| Addressing Mode | Assembly Language Form |  | OP CODE |  | No. Bytes |  | No. Cycles |  | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative | BEQ Oper |  | F0 |  | 2 |  | 2* |  | - | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  | * => Add 1 if branch occurs to same page * => Add 2 if branch occurs to different page |  |  |  |  |  |  |  |  |

Table (7): BEQ – Short Reference

Example Relative (assumed Z=0, same page):

Address	Bytes		Mnemonic

$9000		F0 10		BEQ #$10	; BEQ to same page, - NO BRANCH - jumps to $9002

$9002		…		next OP	; Z=0 => no branch, 2 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”F0” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (11): BEQ – Timing Diagram “no branch, same page”

Example Relative (assumed Z=0, different page):

Address	Bytes		Mnemonic

$90FF		F0 10		BEQ #$10	; BEQ to different page, - NO BRANCH - jumps to

; $9101

$9101		…		next OP	; Z=0 => no branch, 2 cycles (!!!)

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”F0” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (12): BEQ – Timing Diagram “no branch, different page”

Example Relative (assumed Z=1, same page):

Address	Bytes		Mnemonic

$9000		F0 10		BEQ #$10	; BEQ to same page, - BRANCH - jumps to $9002 + $10

…

$9012		…		next OP	; Z=1 => branch, 3 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  | $9012 |  |
| rd_o | 1 |  |  |  |  |  |  |  |
| d_i | X”F0” |  | X”10” |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | Fetch next OP |  |

Figure (13): BEQ – Timing Diagram “branch, same page”

Example Relative (assumed Z=1, different page):

Address	Bytes		Mnemonic

$90FF		F0 10		BEQ #$10	; BEQ to different page, - BRANCH - jumps to

…						; $9111

$9111		…		next OP	; Z=1 => branch, 4 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Cycle 4 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  | $9011 |  | $9111 |  |
| rd_o | 1 |  |  |  |  |  |  |  |  |  |
| d_i | X”F0” |  | X”10” |  | dummy |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | 1. Read dummy 2. Cor- rect PCH |  | Fetch next OP |  |

Figure (14): BEQ – Timing Diagram “branch, different page”

### BIT

Operation: Bit 6 and 7 are transferred to the status register. If the result of A ∩ M then Z = 1, otherwise Z = 0 (A ∩ M → A, M7 → N, M6 → V)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zero Page | BIT Oper | 24 | 2 | 3 | 7 | 6 | - | - | - | - | √ | - |
| Absolute | BIT Oper | 2C | 3 | 4 | 7 | 6 | - | - | - | - | √ | - |

Table (8): BIT – Short Reference

### BMI

Operation: Branch on result minus (Branch on N = 1)

| Addressing Mode | Assembly Language Form |  | OP CODE |  | No. Bytes |  | No. Cycles |  | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative | BEQ Oper |  | F0 |  | 2 |  | 2* |  | - | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  | * => Add 1 if branch occurs to same page * => Add 2 if branch occurs to different page |  |  |  |  |  |  |  |  |

Table (9): BMI – Short Reference

Example Relative (assumed Z=0, same page):

Address	Bytes		Mnemonic

$9000		30 10		BMI #$10	; BMI to same page, - NO BRANCH - jumps to $9002

$9002		…		next OP	; N=0 => no branch, 2 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”30” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (15): BMI – Timing Diagram “no branch, same page”

Example Relative (assumed Z=0, different page):

Address	Bytes		Mnemonic

$90FF		30 10		BMI #$10	; BMI to different page, - NO BRANCH - jumps to

; $9101

$9101		…		next OP	; N=0 => no branch, 2 cycles (!!!)

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”30” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (16): BMI – Timing Diagram “no branch, different page”

Example Relative (assumed Z=1, same page):

Address	Bytes		Mnemonic

$9000		30 10		BMI #$10	; BMI to same page, - BRANCH - jumps to $9002 + $10

…

$9012		…		next OP	; N=1 => branch, 3 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  | $9012 |  |
| rd_o | 1 |  |  |  |  |  |  |  |
| d_i | X”30” |  | X”10” |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | Fetch next OP |  |

Figure (17): BMI – Timing Diagram “branch, same page”

Example Relative (assumed Z=1, different page):

Address	Bytes		Mnemonic

$90FF		30 10		BMI #$10	; BMI to different page, - BRANCH - jumps to

…						; $9111

$9111		…		next OP	; N=1 => branch, 4 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Cycle 4 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  | $9011 |  | $9111 |  |
| rd_o | 1 |  |  |  |  |  |  |  |  |  |
| d_i | X”30” |  | X”10” |  | dummy |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | 1. Read dummy 2. Cor- rect PCH |  | Fetch next OP |  |

Figure (18): BMI – Timing Diagram “branch, different page”

### BNE

Operation: Branch on result not zero (Branch on Z = 0)

| Addressing Mode | Assembly Language Form |  | OP CODE |  | No. Bytes |  | No. Cycles |  | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative | BNE Oper |  | D0 |  | 2 |  | 2* |  | - | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  | * => Add 1 if branch occurs to same page * => Add 2 if branch occurs to different page |  |  |  |  |  |  |  |  |

Table (10): BNE – Short Reference

Example Relative (assumed Z=1, same page):

Address	Bytes		Mnemonic

$9000		D0 10		BNE #$10	; BNE to same page, - NO BRANCH - jumps to $9002

$9002		…		next OP	; Z=1 => no branch, 2 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”D0” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (19): BNE – Timing Diagram “no branch, same page”

Example Relative (assumed Z=1, different page):

Address	Bytes		Mnemonic

$90FF		D0 10		BNE #$10	; BNE to different page, - NO BRANCH - jumps to

; $9101

$9101		…		next OP	; Z=1 => no branch, 2 cycles (!!!)

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”D0” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (20): BNE – Timing Diagram “no branch, different page”

Example Relative (assumed Z=0, same page):

Address	Bytes		Mnemonic

$9000		D0 10		BNE #$10	; BNE to same page, - BRANCH - jumps to $9002 + $10

…

$9012		…		next OP	; Z=0 => branch, 3 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  | $9012 |  |
| rd_o | 1 |  |  |  |  |  |  |  |
| d_i | X”D0” |  | X”10” |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | Fetch next OP |  |

Figure (21): BNE – Timing Diagram “branch, same page”

Example Relative (assumed Z=0, different page):

Address	Bytes		Mnemonic

$90FF		D0 10		BNE #$10	; BNE to different page, - BRANCH - jumps to

…						; $9111

$9111		…		next OP	; Z=0 => branch, 4 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Cycle 4 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  | $9011 |  | $9111 |  |
| rd_o | 1 |  |  |  |  |  |  |  |  |  |
| d_i | X”D0” |  | X”10” |  | dummy |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | 1. Read dummy 2. Cor- rect PCH |  | Fetch next OP |  |

Figure (22): BNE – Timing Diagram “branch, different page”

### BPL

Operation: Branch on result plus (Branch on N = 0)

| Addressing Mode | Assembly Language Form |  | OP CODE |  | No. Bytes |  | No. Cycles |  | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative | BPL Oper |  | 10 |  | 2 |  | 2* |  | - | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  | * => Add 1 if branch occurs to same page * => Add 2 if branch occurs to different page |  |  |  |  |  |  |  |  |

Table (11): BPL – Short Reference

Example Relative (assumed N=1, same page):

Address	Bytes		Mnemonic

$9000		10 10		BPL #$10	; BPL to same page, - NO BRANCH - jumps to $9002

$9002		…		next OP	; N=1 => no branch, 2 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”10” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (23): BPL – Timing Diagram “no branch, same page”

Example Relative (assumed N=1, different page):

Address	Bytes		Mnemonic

$90FF		10 10		BPL #$10	; BPL to different page, - NO BRANCH - jumps to

; $9101

$9101		…		next OP	; N=1 => no branch, 2 cycles (!!!)

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”10” |  | X”10” |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | Fetch next OP |  |

Figure (24): BPL – Timing Diagram “no branch, different page”

Example Relative (assumed N=0, same page):

Address	Bytes		Mnemonic

$9000		10 10		BPL #$10	; BPL to same page, - BRANCH - jumps to $9002 + $10

…

$9012		…		next OP	; N=0 => branch, 3 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |
| a_o | $9000 |  | $9001 |  | $9002 |  | $9012 |  |
| rd_o | 1 |  |  |  |  |  |  |  |
| d_i | X”10” |  | X”10” |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | Fetch next OP |  |

Figure (25): BPL – Timing Diagram “branch, same page”

Example Relative (assumed N=0, different page):

Address	Bytes		Mnemonic

$90FF		10 10		BPL #$10	; BPL to different page, - BRANCH - jumps to

…						; $9111

$9111		…		next OP	; N=0 => branch, 4 cycles

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Cycle 4 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |  |  |
| a_o | $90FF |  | $9100 |  | $9101 |  | $9011 |  | $9111 |  |
| rd_o | 1 |  |  |  |  |  |  |  |  |  |
| d_i | X”10” |  | X”10” |  | dummy |  | dummy |  | next op |  |
| wr_o | 0 |  |  |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. Decide branch 2. PC + 1 |  | 1. Read dummy 2. Check Page crossing 3. PCL + offset |  | 1. Read dummy 2. Cor- rect PCH |  | Fetch next OP |  |

Figure (26): BPL – Timing Diagram “branch, different page”

### BRK

Operation: Forced Interrupt (PC + 2 ↓, P ↓)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | BRK | 00 | 1 | 7 | - | - | - | - | - | 1 | - | - |

Table (12): BRK – Short Reference

### BVC

Operation: Branch on no overflow (Branch on V = 0)

| Addressing Mode | Assembly Language Form |  | OP CODE |  | No. Bytes |  | No. Cycles |  | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative | BVC Oper |  | 50 |  | 2 |  | 2* |  | - | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  | * => Add 1 if branch occurs to same page * => Add 2 if branch occurs to different page |  |  |  |  |  |  |  |  |

Table (13): BVC – Short Reference

### BVS

Operation: Branch on overflow (Branch on V = 1)

| Addressing Mode | Assembly Language Form |  | OP CODE |  | No. Bytes |  | No. Cycles |  | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Relative | BVS Oper |  | 70 |  | 2 |  | 2* |  | - | - | - | - | - | - | - | - |
|  |  |  |  |  |  |  |  | * => Add 1 if branch occurs to same page * => Add 2 if branch occurs to different page |  |  |  |  |  |  |  |  |

Table (14): BVS – Short Reference

### CLC

Operation: Clear Carry flag (0 → C)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | CLC | 18 | 1 | 2 | - | - | - | - | - | - | - | 0 |

Table (15): CLC – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		18		CLC		;

$9001		…		next OP	; C is now 0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”18” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  |  |  | 0 → C |  |

Figure (27): CLC – Timing Diagram

### CLD

Operation: Clear decimal flag (0 → D)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | CLD | D8 | 1 | 2 | - | - | - | - | 0 | - | - | - |

Table (16): CLD – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		D8		CLD		;

$9001		…		next OP	; D is now 0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”D8” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  |  |  | 0 → D |  |

Figure (28): CLD – Timing Diagram

### CLI

Operation: Clear interrupt disable flag (0 → I)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | CLI | 58 | 1 | 2 | - | - | - | - | - | 0 | - | - |

Table (17): CLI – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		58		CLI		;

$9001		…		next OP	; I is now 0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”58” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  |  |  | 0 → I |  |

Figure (29): CLI – Timing Diagram

### CLV

Operation: Clear overflow flag (0 → O)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | CLV | B8 | 1 | 2 | - | 0 | - | - | - | - | - | - |

Table (18): CLV – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		B8		CLV		;

$9001		…		next OP	; V is now 0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”B8” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  |  |  | 0 → V |  |

Figure (30): CLV – Timing Diagram

### CMP

### CPX

### CPY

### DEC

### DEX

Operation: Decrement index X by one (X - 1 → X)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | DEX | CA | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (19): DEX – Short Reference

Example Implied (assume X=$01):

Address	Bytes		Mnemonic

$9000		CA		DEX		;

$9001		…		next OP	; X is now $00, N=0, Z=1

DEX	0000 0001 ($01)

0000 0000 ($00), N=0, Z=1

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”CA” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. X - 1 2. N & Z |  |

Figure (31): DEX – Timing Diagram

### DEY

Operation: Decrement index Y by one (Y - 1 → Y)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | DEY | 88 | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (20): DEX – Short Reference

Example Implied (assume Y=$00):

Address	Bytes		Mnemonic

$9000		88		DEY		;

$9001		…		next OP	; Y is now $FF, N=1, Z=0

DEY	0000 0000 ($00)

1111 1111 ($FF), N=1, Z=0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”88” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. Y - 1 2. N & Z |  |

Figure (32): DEY – Timing Diagram

### EOR

Operation: “Exclusive-Or” memory or immediate value with accumulator A (A V M → A)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate | EOR # Oper | 49 | 2 | 2 | √ | - | - | - | - | - | √ | - |  |
| Zero Page | EOR Oper | 45 | 2 | 3 | √ | - | - | - | - | - | √ | - |  |
| Zero Page, X | EOR Oper, X | 55 | 2 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute | EOR Oper | 4D | 3 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute, X | EOR Oper, X | 5D | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
| Absolute, Y | EOR Oper, Y | 59 | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
| (Indirect, X) | EOR (Oper, X) | 41 | 2 | 6 | √ | - | - | - | - | - | √ | - |  |
| (Indirect), Y | EOR (Oper), Y | 51 | 2 | 5* | √ | - | - | - | - | - | √ | - |  |
|  |  |  |  | * => Add 1 if page boundary is crossed |  |  |  |  |  |  |  |  |  |

Table (21): EOR – Short Reference

Example Immediate (assumed A is $23):

Address	Bytes		Mnemonic

$9000		49 63		EOR #$63	;

$9002		…		next OP	; A is now $40, N=0, Z=0

0110 0011 ($63)

EOR	0010 0011 ($23)

0100 0000 ($40), N=0, Z=0

### INC

### INX

Operation: Increment index X by one (X + 1 → X)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | INX | E8 | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (22): INX – Short Reference

Example Implied (assume X=$0C):

Address	Bytes		Mnemonic

$9000		E8		INX		;

$9001		…		next OP	; X is now $0D, N=0, Z=0

INX	0000 1100 ($0C)

0000 1101 ($0D), N=0, Z=0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”E8” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. X + 1 2. N & Z |  |

Figure (33): INX – Timing Diagram

### INY

Operation: Increment index Y by one (Y + 1 → Y)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | INY | C8 | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (23): INY – Short Reference

Example Implied (assume Y=$FF):

Address	Bytes		Mnemonic

$9000		C8		INY

$9001		…		next OP

INY	1111 1111 ($FF)

0000 0000 ($00), N=0, Z=1

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”C8” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. Y + 1 2. N & Z |  |

Figure (34): INY – Timing Diagram

### JMP

### JSR

Operation: Jump to subroutine saving return address (PC + 2 ↓, (PC + 1) → PCH, (PC + 2) → PCL)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Absolute | JSR | 20 | 3 | 6 | - | - | - | - | - | - | - | - |

Table (24): JSR – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		20 72 F0	JSR $F072	;

F072		…		next OP

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Cycle 3 |  | Cycle 4 |  | Cycle 5 |  | Cycle 6 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a_o | X”$9000” |  | X”$9001” |  | SP |  | SP - 1 |  | SP - 2 |  | X”$9002” |  | X”F072” |  |
| rd_o | 1 |  |  |  |  |  | 0 |  |  |  |  |  |  |  |
| d_i | X”20” |  | X”72” |  | ??? |  |  |  |  |  | X”F0” |  | next op |  |
| wr_o | 0 |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  | X”90” |  | X”02” |  | X”00” |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |  |  |  |  |  |  | unchanged |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | 1. PC + 2 =X”$9002” 2. read X”72” |  | 1. SP - 1 2. dummy data |  | 1. SP - 2 2. write PCH on stack |  | write PCL onto stack |  | 1. read X”F0” 2. PC ← X”F072” |  | Jump to new lo-cation |  |

Figure (35): JSR – Timing Diagram

### LDA

Operation: Load  accumulator A with memory or immediate value (M → A)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate | LDA # Oper | A9 | 2 | 2 | √ | - | - | - | - | - | √ | - |  |
| Zero Page | LDA Oper | A5 | 2 | 3 | √ | - | - | - | - | - | √ | - |  |
| Zero Page, X | LDA Oper, X | B5 | 2 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute | LDA Oper | AD | 3 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute, X | LDA Oper, X | BD | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
| Absolute, Y | LDA Oper, Y | B9 | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
| (Indirect, X) | LDA (Oper, X) | A1 | 2 | 6 | √ | - | - | - | - | - | √ | - |  |
| (Indirect), Y | LDA (Oper), Y | B1 | 2 | 5* | √ | - | - | - | - | - | √ | - |  |
|  |  |  |  | * => Add 1 if page boundary is crossed |  |  |  |  |  |  |  |  |  |

Table (25): LDA – Short Reference

Example Immediate:

Address	Bytes		Mnemonic

$9000		A9 00		LDA #$00	;

$9002		…		next OP	; A is now $00, N=0, Z=1

### LDX

Operation: Load  index X with memory or immediate value (M → X)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate | LDX # Oper | A2 | 2 | 2 | √ | - | - | - | - | - | √ | - |  |
| Zero Page | LDX Oper | A6 | 2 | 3 | √ | - | - | - | - | - | √ | - |  |
| Zero Page, Y | LDX Oper, Y | B6 | 2 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute | LDX Oper | AE | 3 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute, Y | LDX Oper, Y | BE | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
|  |  |  |  | * => Add 1 if page boundary is crossed |  |  |  |  |  |  |  |  |  |

Table (26): LDX – Short Reference

Example Immediate:

Address	Bytes		Mnemonic

$9000		A2 8F		LDX #$8F	;

$9002		…		next OP	; X is now $8F, N=1, Z=0

### LDY

Operation: Load  index Y with memory or immediate value (M → Y)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate | LDY # Oper | A0 | 2 | 2 | √ | - | - | - | - | - | √ | - |  |
| Zero Page | LDY Oper | A4 | 2 | 3 | √ | - | - | - | - | - | √ | - |  |
| Zero Page, X | LDY Oper, X | B4 | 2 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute | LDY Oper | AC | 3 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute, X | LDY Oper, X | BC | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
|  |  |  |  | * => Add 1 if page boundary is crossed |  |  |  |  |  |  |  |  |  |

Table (27): LDY – Short Reference

Example Immediate:

Address	Bytes		Mnemonic

$9000		A0 02		LDY #$02	;

$9002		…		next OP	; Y is now $02, N=0, Z=0

### LSR

### NOP

Operation: No Operation

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | NOP | EA | 1 | 2 | - | - | - | - | - | - | - | - |

Table (28): NOP – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		EA		NOP		;

$9001		…		next OP	;

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”EA” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  | unchanged |  | unchanged |  |

Figure (36): NOP – Timing Diagram

### ORA

Operation: “Or” memory or immediate value with accumulator A (A V M → A)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate | OR # Oper | 09 | 2 | 2 | √ | - | - | - | - | - | √ | - |  |
| Zero Page | OR Oper | 05 | 2 | 3 | √ | - | - | - | - | - | √ | - |  |
| Zero Page, X | OR Oper, X | 15 | 2 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute | OR Oper | 0D | 3 | 4 | √ | - | - | - | - | - | √ | - |  |
| Absolute, X | OR Oper, X | 1D | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
| Absolute, Y | OR Oper, Y | 19 | 3 | 4* | √ | - | - | - | - | - | √ | - |  |
| (Indirect, X) | OR (Oper, X) | 01 | 2 | 6 | √ | - | - | - | - | - | √ | - |  |
| (Indirect), Y | OR (Oper), Y | 11 | 2 | 5* | √ | - | - | - | - | - | √ | - |  |
|  |  |  |  | * => Add 1 if page boundary is crossed |  |  |  |  |  |  |  |  |  |

Table (29): OR – Short Reference

Example Immediate (assumed A is $8A):

Address	Bytes		Mnemonic

$9000		09 11		OR #$11	;

$9002		…		next OP	; A is now $9B, N=1, Z=0

1000 1010 ($8A)

OR	0001 0001 ($11)

1001 1011 ($9B), N=1, Z=0

### PHA

Operation: Push accumulator on stack (A ↓)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | PHA | 48 | 1 | 3 | - | - | - | - | - | - | - | - |

Table (30): PHA – Short Reference

### PHP

Operation: Push processor status on stack (P ↓)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | PHP | 08 | 1 | 3 | - | - | - | - | - | - | - | - |

Table (31): PHP – Short Reference

### PLA

Operation: Pull accumulator from stack (A ↑)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | PLA | 68 | 1 | 3 | - | - | - | - | - | - | - | - |

Table (32): PLA – Short Reference

### PLP

Operation: Pull processor status from stack (P ↑)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | PLP | 28 | 1 | 3 | From Stack |  |  |  |  |  |  |  |

Table (33): PLP – Short Reference

### ROL

| Operation: Rotate one bit left (memory or accumulator) ( ← C) 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accumulator | ROL A | 2A | 1 | 2 | √ | - | - | - | - | - | √ | √ |
| Zero Page | ROL Oper | 26 | 2 | 5 | √ | - | - | - | - | - | √ | √ |
| Zero Page, X | ROL Oper, X | 36 | 2 | 6 | √ | - | - | - | - | - | √ | √ |
| Absolute | ROL Oper | 2E | 3 | 6 | √ | - | - | - | - | - | √ | √ |
| Absolute, X | ROL Oper, X | 3E | 3 | 7 | √ | - | - | - | - | - | √ | √ |

Table (34): ROL – Short Reference

### ROR

| Operation: Rotate one bit right (memory or accumulator) (C → ) 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Accumulator | ROR A | 6A | 1 | 2 | √ | - | - | - | - | - | √ | √ |
| Zero Page | ROR Oper | 66 | 2 | 5 | √ | - | - | - | - | - | √ | √ |
| Zero Page, X | ROR Oper, X | 76 | 2 | 6 | √ | - | - | - | - | - | √ | √ |
| Absolute | ROR Oper | 6E | 3 | 6 | √ | - | - | - | - | - | √ | √ |
| Absolute, X | ROR Oper, X | 7E | 3 | 7 | √ | - | - | - | - | - | √ | √ |

Table (35): ROR – Short Reference

### RTI

Operation: Return from interrupt (P ↑, PC ↑)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | RTI | 40 | 1 | 6 | From Stack |  |  |  |  |  |  |  |

Table (36): RTI – Short Reference

### RTS

Operation: Return from subroutine (PC ↑, PC + 1 → PC)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | RTS | 60 | 1 | 6 | - | - | - | - | - | - | - | - |

Table (37): RTS – Short Reference

### SBC

Operation:  Substract memory or immediate value from accumulator with borrow (A - M - C → A, C)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Immediate | SBC # Oper | E9 | 2 | 2 | √ | √ | - | - | - | - | √ | √ |  |
| Zero Page | SBC Oper | E5 | 2 | 3 | √ | √ | - | - | - | - | √ | √ |  |
| Zero Page, X | SBC Oper, X | F5 | 2 | 4 | √ | √ | - | - | - | - | √ | √ |  |
| Absolute | SBC Oper | ED | 3 | 4 | √ | √ | - | - | - | - | √ | √ |  |
| Absolute, X | SBC Oper, X | FD | 3 | 4* | √ | √ | - | - | - | - | √ | √ |  |
| Absolute, Y | SBC Oper, Y | F9 | 3 | 4* | √ | √ | - | - | - | - | √ | √ |  |
| (Indirect, X) | SBC (Oper, X) | E1 | 2 | 6 | √ | √ | - | - | - | - | √ | √ |  |
| (Indirect), Y | SBC (Oper), Y | F1 | 2 | 5* | √ | √ | - | - | - | - | √ | √ |  |
|  |  |  |  | * => Add 1 if page boundary is crossed |  |  |  |  |  |  |  |  |  |

Table (38): SBC – Short Reference

### SEC

Operation: Set carry flag (1 → C)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | SEC | 38 | 1 | 2 | - | - | - | - | - | - | - | 1 |

Table (39): SEC – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		38		SEC		;

$9001		…		next OP	; C in now 1

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”38” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  |  |  | 1 → C |  |

Figure (37): SEC – Timing Diagram

### SED

Operation: Set decimal flag (1 → D)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | SED | F8 | 1 | 2 | - | - | - | - | 1 | - | - | - |

Table (40): CLD – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		F8		SED		;

$9001		…		next OP	; D in now 1

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”F8” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  |  |  | 1 → D |  |

Figure (38): SED – Timing Diagram

### SEI

Operation: Set interrupt disable flag (1 → I)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | SEI | 78 | 1 | 2 | - | - | - | - | - | 1 | - | - |

Table (41): SEI – Short Reference

Example:

Address	Bytes		Mnemonic

$9000		78		SEI		;

$9001		…		next OP	; I in now 1

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”78” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  |  |  | 1 → I |  |

Figure (39): SEI – Timing Diagram

### STA

Operation: Store accumulator in memory (A → M)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zero Page | STA Oper | 85 | 2 | 3 | - | - | - | - | - | - | - | - |
| Zero Page, X | STA Oper, X | 95 | 2 | 4 | - | - | - | - | - | - | - | - |
| Absolute | STA Oper | 8D | 3 | 4 | - | - | - | - | - | - | - | - |
| Absolute, X | STA Oper, X | 9D | 3 | 5 | - | - | - | - | - | - | - | - |
| Absolute, Y | STA Oper, Y | 99 | 3 | 5 | - | - | - | - | - | - | - | - |
| (Indirect, X) | STA (Oper, X) | 81 | 2 | 6 | - | - | - | - | - | - | - | - |
| (Indirect), Y | STA (Oper), Y | 91 | 2 | 6 | - | - | - | - | - | - | - | - |

Table (42): STA – Short Reference

### STX

Operation: Store index X in memory (X → M)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zero Page | STX Oper | 86 | 2 | 3 | - | - | - | - | - | - | - | - |
| Zero Page, Y | STX Oper, Y | 96 | 2 | 4 | - | - | - | - | - | - | - | - |
| Absolute | STX Oper | 8E | 3 | 4 | - | - | - | - | - | - | - | - |

Table (43): STX – Short Reference

### STY

Operation: Store index Y in memory (Y → M)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Zero Page | STY Oper | 84 | 2 | 3 | - | - | - | - | - | - | - | - |
| Zero Page, X | STY Oper, X | 94 | 2 | 4 | - | - | - | - | - | - | - | - |
| Absolute | STY Oper | 8C | 3 | 4 | - | - | - | - | - | - | - | - |

Table (44): STY – Short Reference

### TAX

Operation: Transfer accumulator A to index X (A → X)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | TAX | AA | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (45): TAX – Short Reference

Example Implied (assume A=$5D):

Address	Bytes		Mnemonic

$9000		AA		TAX		;

$9001		…		next OP	; X is now $5D, N=0, Z=0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”AA” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. A → X 2. N & Z |  |

Figure (40): TAX – Timing Diagram

### TAY

Operation: Transfer accumulator A to index Y (A → Y)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | TAY | A8 | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (46): TAY – Short Reference

Example Implied (assume A=$89):

Address	Bytes		Mnemonic

$9000		A8		TAY		;

$9001		…		next OP	; Y is now $89, N=1, Z=0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”A8” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. A → Y 2. N & Z |  |

Figure (41): TAY – Timing Diagram

### TSX

Operation: Transfer stack pointer S to index X (S → X)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | TSX | BA | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (47): TSX – Short Reference

Example Implied (assume S=$F2):

Address	Bytes		Mnemonic

$9000		BA		TSX

$9001		…		next OP	; X is now $F2, N=1, Z=0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”BA” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. S → X 2. N & Z |  |

Figure (42): TSX – Timing Diagram

### TXA

Operation: Transfer index X to accumulator A (X → A)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | TXA | 8A | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (48): TXA – Short Reference

Example Implied (assume X=$00):

Address	Bytes		Mnemonic

$9000		8A		TXA

$9001		…		next OP	; A is now $00, N=0, Z=1

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”8A” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. X → A 2. N & Z |  |

Figure (43): TXA – Timing Diagram

### TXS

Operation: Transfer index X to stack pointer S (X → S)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | TXS | 9A | 1 | 2 | - | - | - | - | - | - | - | - |

Table (49): TXS – Short Reference

Example Implied (assume X=$E0):

Address	Bytes		Mnemonic

$9000		9A		TXS

$9001		…		next OP	; S is now $E0 (all flags are uneffected

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”9A” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  |  |  |
| Operation | 1. OP Fetch 2. PC + 1 |  |  |  | 1. X → S |  |

Figure (44): TXS – Timing Diagram

### TYA

Operation: Transfer index Y to accumulator A (Y → A)

| Addressing Mode | Assembly Language Form | OP CODE | No. Bytes | No. Cycles | N | V | E | B | D | I | Z | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Implied | TYA | 98 | 1 | 2 | √ | - | - | - | - | - | √ | - |

Table (50): TYA – Short Reference

Example Implied (assume Y=$14):

Address	Bytes		Mnemonic

$9000		98		TYA		;

$9001		…		next OP	; A is now $14, N=0, Z=0

|  | Cycle 1 OP Fetch |  | Cycle 2 |  | Next OP OP Fetch |  |
| --- | --- | --- | --- | --- | --- | --- |
| clk_clk_i |  |  |  |  |  |  |
| sync_o |  |  |  |  |  |  |
| a_o | $9000 |  |  |  | $9001 |  |
| rd_o | 1 |  |  |  |  |  |
| d_i | X”98” |  |  |  | next op |  |
| wr_o | 0 |  |  |  |  |  |
| d_o | X”00” |  |  |  |  |  |
| FLAGS | unchanged |  |  |  | changed |  |
| Operation | 1. OP Fetch 2. PC + 1 |  | Prepare ALU |  | 1. Y → A 2. N & Z |  |

Figure (45): TYA – Timing Diagram

Registers

This section specifies all internal registers. It should completely cover the interface between the core and the host as seen from the software view.

### List of Registers

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |

Table 1: List of registers

### Register 1 – Description

(You shall choose the style of register you prefer. Do not use both options in one and the same document.)

| Bit # | Access | Description |
| --- | --- | --- |

Reset Value:

Reg_Name: 0000h

| 31 | 30 | 29 | 28 | ... | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 2: Description of registers

Reset Value:

Reg_Name: 0000h

Clocks

This section specifies all the clocks. All clocks, clock domain passes and the clock relations should be described.

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |

Table 3: List of clocks

IO Ports

This section specifies the core IO ports.

| Port | Width | Direction | Description |
| --- | --- | --- | --- |

Table 4: List of IO ports

Name

This section may be added to outline different specifications.

Name

This section may be added to outline different specifications.

This section contains an alphabetical list of helpful document entries with their corresponding page numbers.