# UoS educational processor

Characteristics:

- 8-bit, 4 register processor, Von Neumann architecture

- 3 clock cycle per instruction

- 16-bit instruction set (inspired by x86 ISA) with direct, indirect, immediate, register addressing

- Customizable instructions

- External memory bus (for program/data)

- I/O interface (as in microcontrollers)

- Implemented in VHDL

- Synthesizable

## Registers and flags

There are 4 registers. They are encoded as follows in the instruction:

RA:	00

RB:	01

RC:	10

RD:	11

There are 4 flags, that are set after the "cmp dst, src" instruction.

ZF: Zero flag

SF: Sign flag

CF: Carry flag

OF: Overflow flag (not implemented/used yet)

Flags are used to store the result of a comparison operation. The conditional jump operations check the status of the flag to determine whether to jump or not.

## Generalities

The CPU supports move, ALU, jump and external interface instructions.

The general format of move and ALU instructions are of the format:

instr dst,src

instr is the name of the instruction (e.g. mov, add, sub). All ALU instructions will take the data of dst, apply the operation specified with the value of dst and src, and store the result in dst.

dst is the destination where the result is placed. It is always a register (dst is one of ra,rb,rc,rd) for an ALU instruction. For move instructions the destination can be a register or a memory location contained in a register. The syntax for a mov to a memory location is to put the register in bracket (e.g. [ra]).

src is the source of the data. It is either a register or an immediate. An immediate is a value encoded in the instruction. With move instruction, the source can be directly the register value or the immediate (direct mode) or it can be fetched from memory (indirect mode) at the location indicated by the register or the immediate.

## General instruction format

All instructions are 16 bit and follow this format:

| instr(15..13) | instruction(12..8) | instruction(7..0) |
| --- | --- | --- |
| Opcode | depends on the instruction | src |

The opcode (operation code) indicates the type of operation corresponding to this instruction. The meaning of the remaining fields depend on this opcode.

src contains the source of data for the instruction (or is sometimes unused).

src can be an "immediate" (8-bit of data) that is stored in the instruction or a register, in which case only the 2 least significant bits are used to encode the register.

## Move instruction (opcode 000)

Format:

mov dst, src

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Move | Opcode |  |  | RI | dd#m | sd#m | dreg |  | src (i or rm) |  |  |  |  |  |  |  |
| mov rn, rm | 0 | 0 | 0 | 0 | 0 | 0 | r | r | - | - | - | - | - | - | r | r |
| mov rn, i | 0 | 0 | 0 | 1 | 0 | 0 | r | r | i | i | i | i | i | i | i | i |
| mov rn, [rm] | 0 | 0 | 0 | 0 | 0 | 1 | r | r | - | - | - | - | - | - | r | r |
| mov rn, [i] | 0 | 0 | 0 | 1 | 0 | 1 | r | r | i | i | i | i | i | i | i | i |
| mov [rn], rm | 0 | 0 | 0 | 0 | 1 | 0 | r | r | - | - | - | - | - | - | r | r |
| mov [rn], i | 0 | 0 | 0 | 1 | 1 | 0 | r | r | i | i | i | i | i | i | i | i |

The destination can be a register or a memory location. The memory location is indicated by putting the name of the register in bracket.

The source can be a register, an immediate, or a memory location. If it is a memory location the address of the memory location can be provided in a register or in an immediate. Brackets are used to indicate that the source is a memory location.

dreg indicates the register used in the destination assignment. dd#m indicated whether the destination register is used as the target of the move, (dd#m=0) or provides the memory location  for the move (dd#m=1).

The source src is an immediate providing the data or memory location of the source when RI=1 or a register providing the data or the memory location of the source when RI=0.

The source is read from the memory location indicated by src when sd#m=1, otherwise it is the data specified in src (direct or immediate according to RI).

### Examples:

mov ra, rb

Moves the content of register rb into ra

mov ra, 35h

Moves the immediate 35h into ra

mov rd, [ra]

Moves the data at memory location ra into register rd

mov rd, [12h]

Moves the data at memory location 12h into register rd

mov [rb], rc

Moves the content of register rc to the memory location rb.

mov [rb], 07h

Moves the immediate 07h to the memory location rb.

## ALU operations (opcode 001, 010, 011)

Format:

instruction dst, src			(two operands)

instruction dst				(one operand)

ALU operations include two and one operand arithmetic/logic operations.

The destination dst is a register.

The source src can be a register or an immediate. The source is an immediate when RI=1 or a register when RI=0.

The instructions take dst as the first operand, src as the second operand, and place the result in dst. I.e.:

dst <= instruction(dst,src)

See examples.

### Two operands logic/arithmetic

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ALU 2 op | opcode |  |  | RI | ALU op |  | dreg |  | src (i or rm) |  |  |  |  |  |  |  |
| add rn, rm | 0 | 0 | 1 | 0 | 0 | 0 | r | r | - | - | - | - | - | - | r | r |
| add rn, i | 0 | 0 | 1 | 1 | 0 | 0 | r | r | i | i | i | i | i | i | i | i |
| sub rn, rm | 0 | 0 | 1 | 0 | 0 | 1 | r | r | - | - | - | - | - | - | r | r |
| sub rn, i | 0 | 0 | 1 | 1 | 0 | 1 | r | r | i | i | i | i | i | i | i | i |
| and rn, rm | 0 | 0 | 1 | 0 | 1 | 0 | r | r | - | - | - | - | - | - | r | r |
| and rn, i | 0 | 0 | 1 | 1 | 1 | 0 | r | r | i | i | i | i | i | i | i | i |
| or rn, rm | 0 | 0 | 1 | 0 | 1 | 1 | r | r | - | - | - | - | - | - | r | r |
| or rn, i | 0 | 0 | 1 | 1 | 1 | 1 | r | r | i | i | i | i | i | i | i | i |
| xor rn, rm | 0 | 1 | 0 | 0 | 0 | 0 | r | r | - | - | - | - | - | - | r | r |
| xor rn, i | 0 | 1 | 0 | 1 | 0 | 0 | r | r | i | i | i | i | i | i | i | i |

### Two operands comparison

| Test | opcode |  |  | RI | ALU op |  | dreg |  | immedite or rm |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cmp rn, rm | 0 | 1 | 0 | 0 | 0 | 1 | r | r | - | - | - | - | - | - | r | r |
| cmp rn, i | 0 | 1 | 0 | 1 | 0 | 1 | r | r | i | i | i | i | i | i | i | i |

The comparison is realized by performing a subtraction of dst-src (without storing the result to the destination register). The result of the comparison is stored in "flags"

Therefore:

dst=src:		Zero flag set, Carry flag clear

dst>src:		Zero flag clear, Carry flag clear

dst<src:		Zero flag clear, Carry flag set

The conditional jump instructions check the flag bits to decide whether to perform the jump operation.

### One operand logic/arithmetic

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ALU 1 op | opcode |  |  | ALU op |  |  | dreg |  |  |  |  |  |  |  |  |  |
| not r | 0 | 1 | 1 | 0 | 0 | 0 | r | r | - | - | - | - | - | - | - | - |
| shr r | 0 | 1 | 1 | 0 | 0 | 1 | r | r | - | - | - | - | - | - | - | - |
| ror r | 0 | 1 | 1 | 0 | 1 | 0 | r | r | - | - | - | - | - | - | - | - |
| asr r | 0 | 1 | 1 | 0 | 1 | 1 | r | r | - | - | - | - | - | - | - | - |
| rol r | 0 | 1 | 1 | 1 | 0 | 0 | r | r | - | - | - | - | - | - | - | - |

### Examples:

add	ra, rb

Stores ra+rb in rb

sub	ra,3

stores ra-3 in ra.

and	rd,55

stores the logical AND of rd and 55h in rd

ror	rb

rotates right rb and store the result in rb.

## Jumps (opcode 101)

Format:

jxxx		src

Perform a conditional or unconditional jump (change the content of the instruction pointer) to a new memory location.

The memory location can be an immediate, or the content of a register.

The unconditional jump instruction is jmp.

The conditional jump instructions are: je/jz, jne/jnz.

jz/je:		jump if zero/equal (Zero set)

jne/njz:		jump if not zero/equal (Zero clear)

ja:			jump if above (Zero clear, carry clear)

jb:			jump if below (Zero clear, carry set)

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jump Unsigned: JA, JAE, JB, JBE | opcode |  |  | RI | Jump type |  |  |  | immedite / reg |  |  |  |  |  |  |  |
| jmp rn | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | - | - | - | - | - | - | r | r |
| jmp i | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | i | i | i | i | i | i | i | i |
| je/jz rn | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | - | - | - | - | - | - | r | r |
| je/jz i | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | i | i | i | i | i | i | i | i |
| jne/jnz rn | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | r | r |
| jne/jnz i | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | i | i | i | i | i | i | i | i |
| ja rn | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | - | - | - | - | - | - | r | r |
| ja i | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | i | i | i | i | i | i | i | i |
| jae rn | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | - | - | - | - | - | - | r | r |
| jae i | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | i | i | i | i | i | i | i | i |
| jbe rn | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | - | - | - | - | - | - | r | r |
| jbe i | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | i | i | i | i | i | i | i | i |
| jb rn | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | - | - | - | - | - | - | r | r |
| jb i | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | i | i | i | i | i | i | i | i |
| jc rn | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | - | - | - | - | - | - | r | r |
| jc i | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | i | i | i | i | i | i | i | i |
| jnc rn | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | - | - | - | - | - | - | r | r |
| jnc i | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | i | i | i | i | i | i | i | i |

Not implemented

## External interface (opcode 110)

Syntax:

out	src

in	dst

Reads or write data from the external processor interface

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IO | opcode |  |  | RI | IO type |  | dreg |  | src |  |  |  |  |  |  |  |
| out rn | 1 | 1 | 0 | 0 | 0 | 0 | - | - | - | - | - | - | - | - | r | r |
| out i | 1 | 1 | 0 | 1 | 0 | 0 | - | - | i | i | i | i | i | i | i | i |
| in rn | 1 | 1 | 0 | - | 0 | 1 | r | r | - | - | - | - | - | - | - | - |