Registers and flags

## Legend

rn or rm indicate a register (one of RA, RB, RC, RD) as source or destination. It is encoded by two bits in the instruction encoding.

i indicates an immediate, i.e. a constant specified in the instruction encoding.

RA:	00

RB:	01

RC:	10

RD:	11

Flags are set after "cmp dst, src" and used for conditional jumps:

ZF: Zero flag

SF: Sign flag

CF: Carry flag

OF: Overflow flag (not implemented)

## General instruction format

| instr(15..13) | instruction(12..8) | instruction(7..0) |
| --- | --- | --- |
| Opcode | depends on the instruction | src |

## Move instruction (opcode 000)

Format:

mov dst, src

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Move | Opcode |  |  | RI | dd#m | sd#m | dreg |  | src (i or rm) |  |  |  |  |  |  |  |
| mov rn, rm | 0 | 0 | 0 | 0 | 0 | 0 | r | r | - | - | - | - | - | - | r | r |
| mov rn, i | 0 | 0 | 0 | 1 | 0 | 0 | r | r | i | i | i | i | i | i | i | i |
| mov rn, [rm] | 0 | 0 | 0 | 0 | 0 | 1 | r | r | - | - | - | - | - | - | r | r |
| mov rn, [i] | 0 | 0 | 0 | 1 | 0 | 1 | r | r | i | i | i | i | i | i | i | i |
| mov [rn], rm | 0 | 0 | 0 | 0 | 1 | 0 | r | r | - | - | - | - | - | - | r | r |
| mov [rn], i | 0 | 0 | 0 | 1 | 1 | 0 | r | r | i | i | i | i | i | i | i | i |

## ALU operations (opcode 001, 010, 011)

Format:

instruction dst, src			(two operands)

instruction dst				(one operand)

### Two operands logic/arithmetic

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ALU 2 op | opcode |  |  | RI | ALU op |  | dreg |  | src (i or rm) |  |  |  |  |  |  |  |
| add rn, rm | 0 | 0 | 1 | 0 | 0 | 0 | r | r | - | - | - | - | - | - | r | r |
| add rn, i | 0 | 0 | 1 | 1 | 0 | 0 | r | r | i | i | i | i | i | i | i | i |
| sub rn, rm | 0 | 0 | 1 | 0 | 0 | 1 | r | r | - | - | - | - | - | - | r | r |
| sub rn, i | 0 | 0 | 1 | 1 | 0 | 1 | r | r | i | i | i | i | i | i | i | i |
| and rn, rm | 0 | 0 | 1 | 0 | 1 | 0 | r | r | - | - | - | - | - | - | r | r |
| and rn, i | 0 | 0 | 1 | 1 | 1 | 0 | r | r | i | i | i | i | i | i | i | i |
| or rn, rm | 0 | 0 | 1 | 0 | 1 | 1 | r | r | - | - | - | - | - | - | r | r |
| or rn, i | 0 | 0 | 1 | 1 | 1 | 1 | r | r | i | i | i | i | i | i | i | i |
| xor rn, rm | 0 | 1 | 0 | 0 | 0 | 0 | r | r | - | - | - | - | - | - | r | r |
| xor rn, i | 0 | 1 | 0 | 1 | 0 | 0 | r | r | i | i | i | i | i | i | i | i |

### Two operands comparison

| Test | opcode |  |  | RI | ALU op |  | dreg |  | immedite or rm |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cmp rn, rm | 0 | 1 | 0 | 0 | 0 | 1 | r | r | - | - | - | - | - | - | r | r |
| cmp rn, i | 0 | 1 | 0 | 1 | 0 | 1 | r | r | i | i | i | i | i | i | i | i |

The comparison is realized by performing a subtraction of dst-src and the result is stored in flags; dst is unchanged:

dst=src:		Zero flag set, Carry flag clear

dst>src:		Zero flag clear, Carry flag clear

dst<src:		Zero flag clear, Carry flag set

### One operand logic/arithmetic

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ALU 1 op | opcode |  |  | ALU op |  |  | dreg |  |  |  |  |  |  |  |  |  |
| not rn | 0 | 1 | 1 | 0 | 0 | 0 | r | r | - | - | - | - | - | - | - | - |
| shr rn | 0 | 1 | 1 | 0 | 0 | 1 | r | r | - | - | - | - | - | - | - | - |
| ror rn | 0 | 1 | 1 | 0 | 1 | 0 | r | r | - | - | - | - | - | - | - | - |
| asr rn | 0 | 1 | 1 | 0 | 1 | 1 | r | r | - | - | - | - | - | - | - | - |
| rol rn | 0 | 1 | 1 | 1 | 0 | 0 | r | r | - | - | - | - | - | - | - | - |

## Jumps (opcode 101)

Format:

jxxx		src

Unconditional jump:	jmp

Conditional jump:	je/jz: jump zero/equal, ZF=1

jne/jnz: jump not zero/not equal, ZF=0

ja: jump above (ZF=0, CF=0)

jb: jump below (ZF=0, CF=1)

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Jump Unsigned: JA, JAE, JB, JBE | opcode |  |  | RI | Jump type |  |  |  | immedite or rn |  |  |  |  |  |  |  |
| jmp rn | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | - | - | - | - | - | - | r | r |
| jmp i | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | i | i | i | i | i | i | i | i |
| je/jz rn | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | - | - | - | - | - | - | r | r |
| je/jz i | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | i | i | i | i | i | i | i | i |
| jne/jnz rn | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | - | - | - | - | - | - | r | r |
| jne/jnz i | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | i | i | i | i | i | i | i | i |
| ja rn | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | - | - | - | - | - | - | r | r |
| ja i | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | i | i | i | i | i | i | i | i |
| jb rn | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | - | - | - | - | - | - | r | r |
| jb i | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | i | i | i | i | i | i | i | i |

## External interface (opcode 110)

Format:

out	src

in	dst

| Instructions | instruction(15..8) |  |  |  |  |  |  |  | Instruction(7..0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IO | opcode |  |  | RI | IO type |  | dreg |  | src (i or rn) |  |  |  |  |  |  |  |
| out rn | 1 | 1 | 0 | 0 | 0 | 0 | - | - | - | - | - | - | - | - | r | r |
| out i | 1 | 1 | 0 | 1 | 0 | 0 | - | - | i | i | i | i | i | i | i | i |
| in rn | 1 | 1 | 0 | - | 0 | 1 | r | r | - | - | - | - | - | - | - | - |