DPLL.V

/* Top module */

module dpll(SignalIn, SignalOut, MainClock,

Positive, Negative, Lead, Lag

);

input  SignalIn;                // input signal

input  MainClock;               // reference signal

output SignalOut;               // output

output Positive, Negative;      // internal DPLL signals

output Lead, Lag;               // internal DPLL signals

// phase comparator

phasecomparator inst_ph_cmp(.MainClock(MainClock), .InputSignal(SignalIn),

.OutputSignal(SignalOut), .Lead(Lead), .Lag(Lag)

);

/*

// "Zero-Reset Random Walk Filter"

randomwalkfilter inst_zrwf(.MainClock(MainClock), .Lead(Lead), .Lag(Lag),

.Positive(Positive), .Negative(Negative)

);

*/

// "Variable-Reset Random Walk Filter"

variableresetrandomwalkfilter inst_zrwf(.MainClock(MainClock), .Lead(Lead), .Lag(Lag),

.Positive(Positive), .Negative(Negative)

);

// controlled frequency divider

freqdivider inst_freqdiv(.MainClock(MainClock), .FrequencyOut(SignalOut),

.Positive(Positive), .Negative(Negative)

);

Endmodule

FREQDIVIDER.V

/* frequency divider and phase controller */

module freqdivider(MainClock, Positive, Negative, FrequencyOut);

input MainClock;                 // main clock

input Positive, Negative;    // signals Positive, Negative are synchronous with MainClock

output FrequencyOut;         // output frequency

/* needed counter length */

parameter DividerLength   = 7;

/*  controlled prescaler, after this prescales the "divider by 2" installed,     */

/*  so composite divide coefficient will be equivalent of 96 (in this example) - */

/*  it's necessary for work DPLL on frequency 192kHz with oscillator             */

/*  frequency 18432kHz                                                           */

/* additional divider by 2 used for getting output signal with duty factor of 2  */

parameter DividerMaxValue = 48;

reg [DividerLength-1 : 0] DividerCounter;

reg FrequencyOut;        // registered output

/* Process of freq. division according to  signals from Random  Deviations Filter:  */

/* if "lag" then counter will incremented by 2                                                                          */

/* if "lead" then counter will not changed                                                                                */

/* if there is no phase lead or lag then counter normally incremented by 1                          */

always @(posedge MainClock)

begin

if(DividerCounter >= (DividerMaxValue - 1))

DividerCounter <= 0;

else if(Negative)       DividerCounter <= DividerCounter + 2;

else if(Positive) DividerCounter <= DividerCounter;

else        DividerCounter <= DividerCounter + 1;

if(DividerCounter == 0) FrequencyOut <= ~FrequencyOut;           // additional divider by 2 - for producing 50% duty factor of the output signal

end

endmodule

PHASECOMPARATOR.V

/* phase comparator */

module phasecomparator(InputSignal, OutputSignal, MainClock, Lead, Lag);

input InputSignal, OutputSignal;    // PLL input(reference) and output(dejittered clock) signals

input MainClock;                    // System Clock

output Lead, Lag;                   // Lead and Lag signals

reg [1:0] InputSignalEdgeDet;       // detector of the rising edge

always @(posedge MainClock)

begin

InputSignalEdgeDet <= { InputSignalEdgeDet[0], InputSignal };

end

/* this signal checked at rising edge of MainClock.       */

/* It's simple detector of the Input signal rising edge - */

/* When it detected then we check the level of the output.*/

/* There is possible to place additional 2 registers for  */

/* output signal for eliminatig  the cmp. constant phase error */

wire InputSignalEdge = (InputSignalEdgeDet == 2'b01);

/* "Lead" signal will be generate in case of output==1 during input rising edge*/

reg Lead, Lag;                   // outputs "Lead", "Lag" are registered

always @(posedge MainClock)

begin

Lag  <= ((InputSignalEdge == 1'b1)  && (OutputSignal == 1'b0));

Lead <= ((InputSignalEdge == 1'b1)  && (OutputSignal == 1'b1));

end

endmodule

RANDOMWALKFILTER.V

/* Random Walk Filter with reset value of 0*/

module randomwalkfilter(MainClock, Lead, Lag, Positive, Negative);

input  MainClock, Lead, Lag;    // System Clock and Phase Comparator signals

output Positive, Negative;      // "positive shift" and "negative shift" outputs

/* some parametere are accessible from outside */

parameter FilterLength      = 8;

parameter FilterResetValue  = 4;

parameter FilterMaxValue    = FilterResetValue;

parameter FilterMinValue    = 256 - FilterResetValue;

/* reversive counter */

reg [FilterLength-1 : 0] FilterCounter;

/* calculation of output pulses synchrinized with MainClock */

always @(posedge MainClock)

begin

if((FilterCounter == FilterMaxValue) || (FilterCounter == FilterMinValue))

FilterCounter <= 0;

else

begin

if(Lead) FilterCounter <= FilterCounter + 1;

if(Lag)  FilterCounter <= FilterCounter - 1;

end

end

/* making "Lead" and "Lag" signals when  */

/* counter reached max or min levels     */

reg Positive, Negative;

always @(posedge MainClock)

begin

Positive <= (FilterCounter == FilterMaxValue);

Negative <= (FilterCounter == FilterMinValue);

end

endmodule

VARIABLERESETRANDOMWALKFILTER.V

/* random-walk filter with variable reset */

module variableresetrandomwalkfilter(MainClock, Lead, Lag, Positive, Negative);

input  MainClock, Lead, Lag; // System Clock and Phase Comparator signals

output Positive, Negative;   // "positive shift" and "negative shift" outputs

parameter N_FilterLength      = 8;

parameter N_FilterResetValue  = 8;

parameter N_FilterMaxValue    = N_FilterResetValue;

/* 256=2_PWR_8(counter length). Use this value because unsigned arithmetic */

parameter N_FilterMinValue    = 256 - N_FilterResetValue;

/* the counter length of reset scheme must be short */

parameter ResetterCounterLength   = 4;

parameter ResetterCounterMaxValue = 3;

/* 16=2_PWR_4 */

parameter ResetterCounterMinValue = 16 - 3;

/* counter "N - RandomWalkFilter" */

reg [N_FilterLength-1 : 0] N_FilterCounter;

/* connections of "M - RandomWalkFilter" */

wire Up, Down;

randomwalkfilter inst_M_Filter(.MainClock(MainClock), .Lead(Lead), .Lag(Lag),

.Positive(Up), .Negative(Down));

defparam inst_M_Filter.FilterResetValue = 32;  // length "M-RWF" = 32

/* Reset Scheme. This counter changes on "M-RWF" counter */

reg [ResetterCounterLength-1 : 0] ResetterCounter;

always @(posedge MainClock)

begin

if(Up)

begin

if((ResetterCounter < ResetterCounterMaxValue) || (ResetterCounter >= ResetterCounterMinValue))

ResetterCounter <= ResetterCounter + 1;

end

else if(Down)

begin

if((ResetterCounter <= ResetterCounterMaxValue) || (ResetterCounter > ResetterCounterMinValue))

ResetterCounter <= ResetterCounter - 1;

end

if((ResetterCounter > ResetterCounterMaxValue) && (ResetterCounter < ResetterCounterMinValue))

ResetterCounter <= 0;

end

/* Look-Up Table between ResetterCounter value and reset state of "N-RWF" */

reg [N_FilterLength-1 : 0] ResetterValue;

always @(1)

begin

case(ResetterCounter)

16 - 3:  ResetterValue = 256 - 7;

16 - 2:  ResetterValue = 256 - 6;

16 - 1:  ResetterValue = 256 - 4;

0:       ResetterValue = 0;

1:       ResetterValue = 4;

2:       ResetterValue = 6;

3:       ResetterValue = 7;

default: ResetterValue = 0;

endcase

end

/* "N-RWF" Filter has different reset states */

/* in accordance ResetterCounter value       */

always @(posedge MainClock)

begin

if((N_FilterCounter == N_FilterMaxValue) || (N_FilterCounter == N_FilterMinValue))

N_FilterCounter <= ResetterValue;

else

begin

if(Lead) N_FilterCounter <= N_FilterCounter + 1;

if(Lag)  N_FilterCounter <= N_FilterCounter - 1;

end

end

/* making "Lead" and "Lag" signals when  */

/* counter reached max or min levels     */

reg Positive, Negative;

always @(posedge MainClock)

begin

Positive <= (N_FilterCounter == N_FilterMaxValue);

Negative <= (N_FilterCounter == N_FilterMinValue);

end

endmodule