I2S Interface
Specification

Author: Geir Drange

gedra@opencores.org

Rev. 1.0

July 31, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 28/7/04 | Geir Drange | Specification, first version |
| 1.1 | 17/1/05 | Geir Drange | RATIO equation correction |

Contents

1	1

2	2

2.1 I2S Receiver	4

2.2 I2S Transmitter	5

3	6

3.1 Receiver	6

3.1.1	Resetting	6

3.1.2	Transferring data	6

3.2 Transmitter	6

3.2.1	Resetting	6

3.2.2	Selecting transmit data rate	6

3.2.3	Transferring data	7

4	8

4.1 Generics for both transmitter and receiver	8

5	9

5.1 I2S Receiver	9

5.1.1	Receiver registers - overview	9

5.1.2	RxVersion – Description	9

5.1.3	RxConfig – Description	10

5.1.4	RxIntMask – Description	10

5.1.5	RxIntStat – Description	10

5.1.6	Receive sample data – Description	11

5.2 I2S Transmitter	11

5.2.1	Transmitter registers - overview	11

5.2.2	TxVersion – Description	11

5.2.3	TxConfig – Description	12

5.2.4	TxIntMask – Description	12

5.2.5	TxIntStat – Description	13

5.2.6	Transmit sample data – Description	13

6	14

7	15

7.1 Wishbone interface signals (receiver & transmitter)	15

7.2 I2S Receiver signals, slave mode	16

7.3 I2S Receiver signals, master mode	16

7.4 I2S Transmitter signals, slave mode	16

7.5 I2S Transmitter signals, master mode	16

Introduction

The I2S-bus (Inter-IC Sound bus) is a serial link for transmitting stereo audio between devices in a system. Typical devices that use this bus are: ADC’s, DAC’s, DSP’s, CPU’s etc. The I2S bus was invented by Philips Semiconductor, but is now widely used by several semiconductor manufacturers.

The I2S interface core allows a Wishbone master to stream stereo audio to and from I2S capable devices.

Architecture

The I2S interface consists of two separate cores, a transmitter and a receiver. Both can operate in either master or slave mode. The I2S bus has 3 signals:

- SCK – clock

- WS – word select (left/right channel)

- SD – data

The master generates SCK and WS signals.

SD SCK WS Transmitter (Master) Receiver (Slave) SD SCK WS Transmitter (Slave) Receiver (Master)

Figure 1: I2S Signal Direction

I2S signal timing:

word n+1 right channel word n left channel word n-1 right channel MSB LSB MSB LSB WS SCK SD

Figure 2: I2S Signal Timing

## I2S Receiver

The receiver architecture is shown below.

SD WS Wishbone Bus Interrupt SCK Registers Serial-to-parallel converter Sample buffer

Figure 3: I2S Receiver Block Diagram

The serial to parallel converter receives serial data and stores audio data in the sample buffer. A set of registers define the operating mode of the receiver.

The size of the sample buffer is determined by the Wishbone address bus width. Minimum sample buffer size is 16bytes. The sample buffer is addressed by setting the most significant address bit to ‘1’. The sample buffer is divided in two equal parts, lower and upper, and the user will be notified when either is filled with audio data.

Master or slave mode is selected by a generic during compile time.

## I2S Transmitter

The transmitter architecture is shown below.

SD WS Wishbone Bus Interrupt SCK Registers Parallel-to-serial converter Sample buffer

Figure 4: I2S Transmitter Block Diagram

A parallel to serial converter reads audio data from the sample buffer and transmits serial data on the SD line. A set of registers define the operating mode of the transmitter.

The size of the sample buffer is determined by the Wishbone address bus width. Minimum sample buffer size is 16bytes. The sample buffer is addressed by setting the most significant address bit to ‘1’. The sample buffer is divided in two equal parts, lower and upper, and the user will be notified when either is emptied of audio data.

Master or slave mode is selected by a generic during compile time.

Operation

This chapter contains operational guidelines for the cores.

## Receiver

#### Resetting

The receiver is reset by a Wishbone bus reset or by clearing bit RXEN in RxConfig register.

#### Transferring data

If the receiver is master, set the RATIO bits in RxConfig to required I2S bit rate. Set RXEN to 1 to enable receiver.

## Transmitter

#### Resetting

The transmitter is reset by a Wishbone bus reset or by clearing bit TXEN in TxConfig register.

#### Selecting transmit data rate

In master mode, the RATIO bits in TxConfig must be set to required bit rate.

bitrate=wishbonebusclock2⋅(RATIO+2)

The sample rate depends on the RES (data resolution) and bit rate:

samplerate=bitrateRES⋅2

RATIO bits can then be calculated by:

RATIO=wishbonebusclock−samplerate⋅RES⋅8samplerate⋅RES⋅4

#### Transferring data

Fill up the sample buffer with sample data then set the TXEN bit in TxConfig to start data transfer. Using interrupts, the lower or higher sample buffer can be filled up with new data as soon as its content has been transmitted.

Generics

The I2S interface has a number of generics that can be used to tailor the interface for various needs.

## Generics for both transmitter and receiver

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| DATA_WIDTH | Integer | 16/32 | Wishbone data bus width. If using 16bit bus, some functionality is lost. |
| ADDR_WIDTH | Integer | 5 - 32 | Wishbone addresses bus width. The sample buffer occupies half the address range. |

Table 1: Generics for transmitter and receiver

Registers

This section specifies all internal registers of the I2S interface.

## I2S Receiver

#### Receiver registers - overview

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |
| RxVersion | 0x00 | 16/32 | R | Version register |
| RxConfig | 0x01 | 16/32 | RW | Configuration register |
| RxIntMask | 0x02 | 16/32 | RW | Interrupt mask register |
| RxIntStat | 0x03 | 16/32 | RW | Interrupt status register |

Table 2: Receiver registers

#### RxVersion – Description

The version register allows the SW to read out all the parameters that were used to generate the receiver.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-13 |  | - | Unused |
| 12-6 |  | ADRW | The value of ADDR_WIDTH |
| 5 |  | MAST | 0: The receiver is slave 1: The receiver is master |
| 4 |  | DATW | 0: DATA_WIDTH is 16bit 1: DATA_WIDTH is 32bit |
| 3-0 |  | VER | I2S version number = 1 |

Reset Value:

RxVersion: Depends on generics

#### RxConfig – Description

The configuration register controls the operation of the receiver. Note that only 16bit data resolution is supported when DATA_WIDTH=16.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-22 | R | - | Unused |
| 21-16 | RW | RES | Sample data resolution. Number of bits that are stored in each audio word in the sample buffer. Valid range is 16 to 32. If the received signal has fewer bits than set by RES, zero padding of LSB’s is used. If the received signal has more bits than set by RES, LSB’s are truncated. |
| 15-8 | RW | RATIO | Master mode only: Clock divider for the transmit frequency. The Wishbone bus clock is divided by a factor of (1+RATIO) to generate the serial transmit clock, SCK. |
| 7-3 | R | - | Unused |
| 2 | RW | RSWAP | 0: Left channel is stored on even addresses 1: Left channel is stored on odd addresses |
| 1 |  | RINTEN | 0: Interrupt output is disabled 1: Interrupt output is enabled |
| 0 |  | RXEN | 0: Receiver is disabled 1: Receiver is enabled |

Reset Value:

RxConfig: 0x0000

#### RxIntMask – Description

Set bit to 1 in the mask register to enable an interrupt source.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-2 | R | - | Unused |
| 1 |  | HSBF | Higher sample buffer full interrupt mask |
| 0 |  | LSBF | Lower sample buffer full interrupt mask |

Reset Value:

RxIntMask: 0x0000

#### RxIntStat – Description

A bit in this register is set to 1 when an event occurs. If the corresponding bit in RxIntMask is set to 1, an interrupt is generated (if enabled). Write a 1 to a bit to clear the event. The interrupt signal goes inactive when all events have been cleared.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-2 | R | - | Unused |
| 1 |  | HSBF | Higher sample buffer full |
| 0 |  | LSBF | Lower sample buffer full |

Reset Value:

RxIntStat: 0x0000

#### Receive sample data – Description

Format of data words in receive sample buffer:

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | DATH | Audio data if resolution is more than 16bits. Unused bits are 0. |
| 15-0 |  | DATL | Audio data. Bit 0 is LSB. |

Reset Value:

Sample buffer: undefined

## I2S Transmitter

#### Transmitter registers - overview

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |
| TxVersion | 0x00 | 16/32 | R | Version register |
| TxConfig | 0x01 | 16/32 | RW | Configuration register |
| TxIntMask | 0x02 | 16/32 | RW | Interrupt mask register |
| TxIntStat | 0x03 | 16/32 | RW | Interrupt status register |

Table 3: Transmitter registers

#### TxVersion – Description

The version register allows the SW to read out all the parameters that were used to generate the receiver.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-13 |  | - | Unused |
| 12-6 |  | ADRW | The value of ADDR_WIDTH |
| 5 |  | MAST | 0: The transmitter is slave 1: The transmitter is master |
| 4 |  | DATW | 0: DATA_WIDTH is 16bit 1: DATA_WIDTH is 32bit |
| 3-0 |  | VER | I2S version number = 1 |

Reset Value:

TxVersion: - depends on generics

#### TxConfig – Description

The configuration register controls the operation of the transmitter. Note that only 16bit data resolution is supported when DATA_WIDTH=16.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-22 | R | - | Unused |
| 21-16 | RW | RES | Sample data resolution. Number of bits that are transmitted from each audio word in the sample buffer. Valid range is 16 to 32. |
| 15-8 | RW | RATIO | Master mode only: Clock divider for the transmit frequency. The Wishbone bus clock is divided by a factor of (1+RATIO) to generate the serial transmit clock, SCK. |
| 7-3 | R | - | Unused |
| 2 | RW | TSWAP | 0: Left channel is stored on even addresses 1: Left channel is stored on odd addresses |
| 1 |  | TINTEN | 0: Interrupt output is disabled 1: Interrupt output is enabled |
| 0 |  | TXEN | 0: Receiver is disabled 1: Receiver is enabled |

Reset Value:

TxConfig: 0x0000

#### TxIntMask – Description

Set bit to 1 in the mask register to enable an interrupt source.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-2 | R | - | Unused |
| 1 | RW | HSBF | Higher sample buffer empty |
| 0 |  | LSBF | Lower sample buffer empty |

Reset Value:

TxIntMask: 0x0000

#### TxIntStat – Description

A bit in this register is set to 1 when an event occurs. If the corresponding bit in TxIntMask is set to 1, an interrupt is generated (if enabled). Write a 1 to a bit to clear the event. The interrupt signal goes inactive when all events have been cleared.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-2 | R | - | Unused |
| 1 | RW | HSBF | Higher sample buffer empty |
| 0 |  | LSBF | Lower sample buffer empty |

Reset Value:

TxIntStat: 0x0000

#### Transmit sample data – Description

Format of data words in transmit sample buffer.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | W | DATH | Audio data if resolution is more than 16bits. If less than 32bit resolution, set MSB’s to zero. |
| 15-0 |  | DATL | Audio data. Bit 0 is LSB. |

Reset Value:

Sample buffer: undefined

Clocks

The I2S interface uses the Wishbone interface clock only.

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |
| wb_clk_i | - | - |  | - | Must be at least 8 times higher than I2S bit rate | System clock. |

Table 4: List of clocks

IO Ports

## Wishbone interface signals (receiver & transmitter)

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| wb_ack_o | 1 | Output | Bus cycle acknowledge |
| wb_adr_i | 8-32 | Input | Address bus |
| wb_bte_i | 2 | Input | Burst type extension |
| wb_clk_i | 1 | Input | Clock |
| wb_cti_i | 3 | Input | Cycle type identifier |
| wb_cyc_i | 1 | Input | Valid bus cycle |
| wb_dat_i | 16/32 | Input | Data to core |
| wb_dat_o | 16/32 | Output | Data from core |
| wb_rst_i | 1 | Input | Asynchronous reset |
| wb_sel_i | 1 | Input | Select Input |
| wb_stb_i | 1 | Input | Strobe |
| wb_we_i | 1 | Input | Write enable |

Table 5: Wishbone signals

| Description | Specification |
| --- | --- |
| General description | 16/32bit slave |
| Supported cycles | SLAVE, READ/WRITE SLAVE, BLOCK READ/WRITE SLAVE, Incrementing burst cycle (linear) |
| Data port, size Data port, granularity Data port, maximum operand size Data transfer ordering Data transfer sequencing | 16 or 32bit 16 or 32bit 16 or 32bit Big/little endian Undefined |

Wishbone properties

## I2S Receiver signals, slave mode

Top level file is rx_i2s_tops.vhd.

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| rx_sd_i | 1 | Input | I2S serial data in |
| rx_ws_i | 1 | Input | Word select signal |
| rx_sck_i | 1 | Input | Serial clock |

Table: Receiver signals, slave

## I2S Receiver signals, master mode

Top level file is rx_i2s_topm.vhd.

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| rx_sd_i | 1 | Input | I2S serial data in |
| rx_ws_o | 1 | Output | Word select signal |
| rx_sck_o | 1 | Output | Serial clock |

Table: Receiver signals, master

## I2S Transmitter signals, slave mode

Top level file is tx_i2s_tops.vhd.

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| tx_sd_o | 1 | Output | I2S serial data out |
| tx_ws_i | 1 | Input | Word select signal |
| tx_sck_i | 1 | Input | Serial clock |

Table: Transmitter signals, slave

## I2S Transmitter signals, master mode

Top level file is tx_i2s_topm.vhd.

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| tx_sd_o | 1 | Output | I2S serial data out |
| tx_ws_o | 1 | Output | Word select signal |
| tx_sck_o | 1 | Output | Serial clock |

Table: Transmitter signals, master