8-bit Pipelined Processor- Instruction Set

| Instruction |  | Instruction OpCode |  |  |  |  |  | Instruction Field |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 | 13 | 12 | 11 | 10 |  | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| END |  | 0 | 0 | 0 | 0 | 0 |  | CODE MEMORY ADDRESS |  |  |  |  |  |  |  |  |  |
| JMP |  | 0 | 0 | 0 | 0 | 1 |  | CODE MEMORY ADDRESS |  |  |  |  |  |  |  |  |  |
| Ld |  | 0 | 0 | 0 | 1 | 0 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| Ldi |  | 0 | 0 | 0 | 1 | 1 |  | X | X | IMMIDIEATE DATA |  |  |  |  |  |  |  |
| ST |  | 0 | 0 | 1 | 0 | 0 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| ADD |  | 0 | 0 | 1 | 0 | 1 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| SUB |  | 0 | 0 | 1 | 1 | 0 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| MUL |  | 0 | 0 | 1 | 1 | 1 |  | * not implemented in current revision |  |  |  |  |  |  |  |  |  |
| DIV |  | 0 | 1 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |
| AND |  | 0 | 1 | 0 | 0 | 1 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| OR |  | 0 | 1 | 0 | 1 | 0 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| XOR |  | 0 | 1 | 0 | 1 | 1 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| GrT |  | 0 | 1 | 1 | 0 | 0 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| GE |  | 0 | 1 | 1 | 0 | 1 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| EQ |  | 0 | 1 | 1 | 1 | 0 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| LE |  | 0 | 1 | 1 | 1 | 1 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| LT |  | 1 | 0 | 0 | 0 | 0 |  | N | I/O/b/B |  | I/O/b/B ADDRESS |  |  |  |  |  |  |
| PRE |  | 1 | 0 | 0 | 0 | 1 |  | X | X | X | X | X | X | Timer/Counter ADDRESS |  |  |  |
| ETY |  | 1 | 0 | 0 | 1 | 0 |  | X | X | X | X | TYPE |  | Timer/Counter ADDRESS |  |  |  |
| RST |  | 1 | 0 | 0 | 1 | 1 |  | X | X | X | X | X | X | Timer/Counter ADDRESS |  |  |  |
| LdTC |  | 1 | 0 | 1 | 0 | 0 |  | X | X | X | X | X | X | Timer/Counter ADDRESS |  |  |  |
| LdACC |  | 1 | 0 | 1 | 0 | 1 |  | X | X | X | X | X | X | Timer/Counter ADDRESS |  |  |  |
| UARTrd |  | 1 | 0 | 1 | 1 | 0 |  | X | X | X | X | X | X | X | X | X | X |
| UARTwr |  | 1 | 0 | 1 | 1 | 1 |  | X | X | X | X | X | X | X | X | X | X |
| UARTstat |  | 1 | 1 | 0 | 0 | 0 |  | X | X | X | X | X | X | X | X | X | X |

__________________________________________________________________________________________________________________________________

X 		don’t care

__________________________________________________________________________________________________________________________________

I/O/b/B		00 – input, 	01 – output, 	10 – bit RAM, 	11 – Byte RAM

__________________________________________________________________________________________________________________________________

TYPE		(related to timer/counter)	00 – on-delay timer	,	01 – off-delay timer,	10 – retentive-on-delay-timer, 	11 – not used for timer

01 – up counter,		10 – down counter,		00 & 11 not used for counter

__________________________________________________________________________________________________________________________________

Timer addresses range from 0000 to 0111

Counter addresses range from 1000 to 1111

Inputs, outputs and both RAMs have addresses from 0000000 to 1111111 (0 to 127 decimal)

Code memory (ROM) address range 0 to 1023

__________________________________________________________________________________________________________________________________