KLC32

# Table of Contents

Table of Contents	1

Goals:	4

Hardware Interface	5

Signal Description	5

Programming Model	7

General Registers	8

Condition Code Register	9

Status Register	9

TICK Register	9

RESET	10

Vector Table	11

Instructions	12

Instruction Formats	13

ADD[.]	16

ADDI.	16

AND[.]	17

ANDI.	17

Bcc	18

CMPI	19

JSR (Ra+Rb)	21

JSR a32	21

LB	22

LBU	22

LBUX	22

LH	24

LHU	24

LHUX	25

LHX	25

LINK	26

LW	27

LWX	27

MULU[.]	28

MULS[.]	28

ORI.	29

POP	30

RTS	31

SETcc	32

SB	33

SBX	33

SH	34

SHX	34

SUBI.	35

SW	36

SWX	36

TRAPcc	37

UNLK	38

KLC32 - Description

Design Approach

# Goals:

Overall processor performance is not as critical as high clock rate.

The author desires a processor with a reasonably high clocking rate. Several designs have been done by the author for which an additional low frequency clock domain was required in order to support a processor. Ideally, a video timing clock may be used as the processor clock.

It's okay for the processor to consume clock cycles while performing internal operations as long as it's not using the bus. In the small system-on-a-chip several devices share the system bus. There will be periods of time when the system bus is not available to the processor, or periods of time where it's desirable for the processor to not be using the system bus.

The multi-clock cycles per instruction approach leaves open windows during which other devices in the system may access the bus.

A typical pipelined RISC approach can achieve a CPI (clock per instruction) of less than 2. The KLC32 requires 3 or more clock cycles to execute an instruction; however it will typically be able to use a higher clock frequency.

# Hardware Interface

- WISHBONE bus master.

- Uses a 68000 style interrupt model.

- The processor uses the same vector table layout as the 68000 and supports a 68000 style

interrupt acknowledge mechanism.

### Signal Description

| Signal | Width |  | Description |
| --- | --- | --- | --- |
| rst_i | 1 | Reset | This input signal resets the processor. |
| clk_i | 1 | Clock |  |
| ipl_i | 3 | Interrupt inputs | Placing a non-zero value on this bus causes the processor to begin interrupt processing. |
| vpa_i | 1 | Vector pull input | Asserting this signal instead of ack_i will cause the processor to use the automatic interrupt vector numbers. |
| halt_i | 1 | Halt input | Asserting this signal causes the processor to halt processing at the end of the current instruction. The processor will remain halted as long as this signal is asserted. |
| inta_o | 1 | Interrupt acknowledge | This signal indicates that an interrupt acknowledge cycle is taking place. |
| fc_o | 1 | Function code | The function codes indicate the type of bus cycle taking place. |
| rst_o | 1 | Reset output | This signal is asserted for 16 clock cycles when the ‘RESET’ instruction is executed. |
| cyc_o | 1 | Cycle is valid | Indicates a valid bus cycle is taking place |
| stb_o | 1 | Strobe | Indicates valid data is on the bus |
| ack_i | 1 | acknowledge | Asserting this signal terminates the bus cycle. |
| err_i | 1 | Error | Asserting this signal terminates the bus cycle and causes the processor to execute the bus error trap. |
| sel_o | 4 | Byte lane select | This signal indicates which byte lanes are in use during the data transfer |
| we_o | 1 | Write Enable | When asserted, this signal indicates that a write cycle is taking place. |
| adr_o | 32 | Address bus | This bus contains the memory / device address during a bus transfer. |
| dat_i | 32 | Data input | This bus contains data being transferred into the processor. |
| dat_o | 32 | Data output | This bus contains data being transferred from the processor to memory / device. |

# Programming Model

- 32 x 32 bit registers, register zero reads as zero, register 31 is the stack pointer.

- Multiple condition code registers akin to the PowerPC.

- Two processing modes, user mode and system mode. Two stack pointers, user and system.

- Register indirect with displacement, and indexed addressing modes.

Register File

KLC32 uses a single ported register file. Multiple clock cycles are used to access the register file in order to read as many as three operands, and write-back a result.

### General Registers

| Reg No. | Name | Comment |
| --- | --- | --- |
| 0 | R0 / 0 | this register always reads as zero |
| 1 | R1 | subroutine return parameter |
| 2 | R2 | subroutine return parameter |
| 3 | R3 |  |
| 4 | R4 |  |
| 5 | R5 |  |
| 6 | R6 |  |
| 7 | R7 |  |
| 8 | R8 |  |
| 9 | R9 |  |
| 10 | R10 |  |
| 11 | R11 |  |
| 12 | R12 |  |
| 13 | R13 |  |
| 14 | R14 |  |
| 15 | R15 |  |
| 16 | R16 |  |
| 17 | R17 |  |
| 18 | R18 |  |
| 19 | R19 |  |
| 20 | R20 |  |
| 21 | R21 |  |
| 22 | R22 |  |
| 23 | R23 |  |
| 24 | R24 |  |
| 25 | R25 |  |
| 26 | R26 |  |
| 27 | R27 |  |
| 28 | R28 |  |
| 29 | R29 |  |
| 30 | FP | frame pointer |
| 31 | USP | user stack pointer |
| 31 | SSP | system stack pointer |

### Condition Code Register

There are eight four-bit condition registers

| cr7 | cr6 | cr5 | cr4 | cr3 | cr2 | cr1 | cr0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NZVC | NZVC | NZVC | NZVC | NZVC | NZVC | NZVC | NZVC |

Any condition code register may be set as a result of a compare instruction. Most instructions can set cr0. Any condition code register may be tested with a branch instruction.

Effective use of multiple condition code registers can result in higher performance in a pipelined processor, because it can eliminate branches which cause pipeline bubbles. Multiple condition codes allow a higher degree of code linearization which is good for performance. Although the KLC is non-pipelined, it supports the multiple condition code register paradigm.

### Status Register

The status register contains the interrupt mask, system mode, and trace flags.

### TICK Register

The processor tick register increments every clock cycle. The tick register is reset to zero by an external reset.

Memory Operation Switch

A reg (mopcode) is used for the memory operation in order to allow both indexed and register displacement modes to use the same code.

### RESET

On reset the initial stack pointer is loaded from address zero, and the initial program counter is loaded from address four. The processor is set to system mode. Trace is disabled, and the interrupt mask is set to level seven. The tick register is set to zero. All other registers are undefined.

### Vector Table

| Address | Vector |  |
| --- | --- | --- |
| 00 | Initial stack pointer |  |
| 04 | Initial program counter |  |
| 08 | Bus Error Vector |  |
| 10 | Illegal Instruction |  |
| 1C | Trap overflow |  |
| 20 | Privilege violation vector |  |
| 24 | Trace vector |  |
| 64 | IRQ vector |  |
| 7C | NMI vector |  |
| 80 | Trap vector |  |

# Instructions

### Instruction Formats

| Opcode |  |  |  |  |  |  |  |  |  |  |  |  |  | Format |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Opcode6 | Ra5 |  |  | Rt5 |  |  | Immediate16 |  |  |  |  |  |  | RI |  |
| Opcode6 | Ra5 |  |  | Rt5 |  |  | 8000h16 |  |  |  |  |  |  | RIX |  |
| Immediate32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Opcode6 | Ra5 |  |  | Rt5 |  |  |  |  |  |  | Rc | Func6 |  | R |  |
| Opcode6 | Ra5 |  |  | Rb5 |  |  |  | Rt5 |  |  | Rc | Func6 |  | RR |  |
| Opcode6 | Ra5 |  |  | Rb5 |  |  |  | Crt3 |  |  |  | Func6 |  | CMP |  |
| Opcode6 | Ra5 |  |  | Crt3 |  |  |  | Immediate16 |  |  |  |  |  | CMPI |  |
| Opcode6 | Cra3 |  |  |  | Cond4 |  |  | Rt5 |  |  |  |  |  | SETcc |  |
| Opcode6 | Cra3 |  |  |  | Cond4 |  |  |  |  |  |  |  |  | TRAPcc |  |
| Opcode6 | Cra3 |  |  |  | Cond4 |  |  | Displacement16 |  |  |  |  |  | BRcc |  |
| Opcode6 | Ra5 |  |  | Rb5 |  |  |  | Rc5 |  | Rd5 |  | Re5 |  |  | PSH / POP |
| Opcode6 | Address26 |  |  |  |  |  |  |  |  |  |  |  |  | JMP / JSR |  |
| Opcode6 | Offs4 |  | Immediate16 |  |  |  |  |  |  |  |  | Func6 |  | RTS |  |

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | Func6 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Function |  |  |
| --- | --- | --- |
| 4 | ADD |  |
| 5 | SUB |  |
| 6 | CMP |  |
| 8 | AND |  |
| 9 | OR |  |
| 10 | EOR |  |
| 11 | ANDC | bitwise and with compliment |
| 12 | NAND | bitwise and then not |
| 13 | NOR | Bitwise or then not |
| 14 | ENOR | bitwise exclusive nor |
| 15 | ORC | bitwise or with compliment |
| 16 | SHL | shift left |
| 17 | SHR | shift right |
| 18 | ROL | rotate left |
| 19 | ROR | rotate right |
| 20 |  |  |
| 21 |  |  |
| 22 | MAX | maximum of two values |
| 23 | MIN | minimum of two values |
| 24 | MULU | unsigned multiply |
| 25 | MULUH | unsigned multiply – return high order bits |
| 26 | MULS | signed multiply |
| 27 | MULSH | signed multiply – return high order bits |
| 28 | DIVU | unsigned divide |
| 29 | DIVS | signed divide |
| 30 | MODU | unsigned remainder |
| 31 | MODS | signed remainder |

| Function |  |  |
| --- | --- | --- |
| 48 | LWX | load word using indexed addressing |
| 49 | LHX | load half- word, indexed addressing |
| 50 | LBX | load byte |
| 51 | LHUX | load half-word unsigned |
| 52 | LBUX | load byte unsigned |
| 56 | SWX | store word using indexed addressing |
| 57 | SHX | store half-word |
| 58 | SBX | store byte |
| 60 | BCDADD | binary coded decimal add |
| 61 | BCDSUB | binary coded decimal subtract |

ADD[.] – add register

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 46 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The sum of Ra and the immediate value supplied in the instruction are loaded into Rt.

Rt <= Ra + immediate

If Rc is true then Cr0 is updated.

ADDI. – add immediate

| 46 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The sum of Ra and the immediate value supplied in the instruction are loaded into Rt.

Rt <= Ra + immediate

Cr0 is updated.

AND[.] – and register

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 86 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The bitwise AND of Ra and Rb are loaded into Rt.

Rt <= Ra and Rb

If Rc is true then Cr0 is updated.

ANDI. – and immediate

| 86 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The bitwise AND of Ra and the immediate value supplied in the instruction are loaded into Rt.

Rt <= Ra and immediate

Cr0 is updated.

Bcc – branch on condition

| 166 | Cra3 |  |  | Cond4 | Displacement16 | BRcc |
| --- | --- | --- | --- | --- | --- | --- |

Operation:

If the specified condition in the condition register is true, then branch, otherwise NOP. Any condition register may be tested. The sign extended sixteen bit displacement is added to the program counter, if the branch is taken.

If (condition true) then

pc <= pc + displacement

| Cond | Mne. |  | Condition |
| --- | --- | --- | --- |
| 0 | BRA | Branch always | 1 |
| 1 | BRN | Branch never | 0 |
| 2 | BHI | Unsigned branch higher | !cf & !zf |
| 3 | BLS | Unsigned branch lower or same | cf ¦ zf |
| 4 | BHS | Unsigned branch higher or same | !cf |
| 5 | BLO | Unsigned branch lower | cf |
| 6 | BNE | Branch if not equal | !zf |
| 7 | BEQ | Branch if equal | zf |
| 8 | BVC | Branch if overflow clear | !vf |
| 9 | BVS | Branch if overflow set | vf |
| 10 | BPL | Branch if plus | !nf |
| 11 | BMI | Branch if minus | nf |
| 12 | BGE | Signed branch greater or equal |  |
| 13 | BLT | Signed branch less than |  |
| 14 | BGT | Signed branch greater than |  |
| 15 | BLE | Signed branch less or equal |  |

CMPI – compare immediate

| Opcode6 | Ra5 | Crt3 |  | Immediate16 | CMPI |
| --- | --- | --- | --- | --- | --- |

Operation:

The target condition register Crt is loaded with result flags of the operation. The operation is a subtract operation of Ra minus the immediate value supplied in the instruction.

Crt <= flags ( Ra – immediate)

Comment:

Any one of the eight condition registers may be the target.

Example:

The following branches if R3 is equal to 1, 5 or 7.

CMPI	CR0,R3,#1

CMPI  CR1,R3,#5

CMPI  CR2,R3,#7

OR CR0.z,CR0.z,CR1.z

OR CR0.z,CR0.z,CR2.z

BEQ CR0,sometarget

EORI. – exclusive or immediate

| 106 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The bitwise exclusive OR of Ra and the immediate value supplied in the instruction are loaded into Rt.

Rt <= Ra eor immediate

Cr0 is updated.

JMP a26 – jump absolute

| 206 | Address26 | JSR |
| --- | --- | --- |

Operation:

The low order 26 bits of the program counter are loaded with the constant supplied by the instruction. The high order six bits of the program counter are not affected.

Address26 => PC[25:0]

JMP (Ra+Rb) – jump register indirect

| 26 | Ra5 | Rb5 |  |  | 0 | 206 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The program counter is loaded with the sum of registers Ra and Rb.

Ra + Rb => PC

JMP a32 – jump to subroutine absolute

| 06 |  |  |  |  |  | 326 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Address32 |  |  |  |  |  |  |  |

Operation:

The program counter is loaded with the constant supplied by the instruction.

Address32 => PC

JSR a26 – jump to subroutine absolute

| 216 | Address26 | JSR |
| --- | --- | --- |

Operation:

The return address is saved on the stack, then the low order 26 bits of the program counter are loaded with the constant supplied by the instruction. The high order six bits of the program counter are not affected.

pc => -(SP)

Address26 => PC[25:0]

JSR (Ra+Rb) – jump to subroutine, register indirect

| 26 | Ra5 | Rb5 |  |  | 0 | 216 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The return address is saved on the stack, then the program counter are loaded with the sum of registers Ra and Rb.

pc => -(SP)

Ra + Rb => PC

JSR a32 – jump to subroutine absolute

| 06 |  |  |  |  |  | 336 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Address32 |  |  |  |  |  |  |  |

Operation:

The return address is saved on the stack, then the program counter is loaded with the constant supplied by the instruction.

pc => -(SP)

Address32 => PC

LB – load byte

| 506 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and the immediate value supplied in the instruction. Register Rb is loaded with the byte addressed at the memory location. The byte value is sign extended to the width of the register.

ea <= Ra + immediate

Rb <= Mem[ea][7:0]

LBU – load unsigned byte

| 526 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and the immediate value supplied in the instruction. Register Rb is loaded with the byte addressed at the memory location. The byte value is zero extended to the width of the register.

ea <= Ra + immediate

Rb <= Mem[ea][7:0]

LBUX – load unsigned byte, indexed addressing

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 526 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and Rb. Register Rt is loaded with memory at the memory address. The byte value is zero extended to the width of the register. If the Rc bit is set in the instruction, then condition register zero is updated, otherwise no condition codes are affected.

ea <= Ra + Rb

Rt <= Mem[ea][7:0]

IF Rc then update cr0

LBX – load byte, indexed addressing

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 506 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and Rb. Register Rt is loaded with memory at the memory address. The byte value is sign extended to the width of the register. If the Rc bit is set in the instruction, then condition register zero is updated, otherwise no condition codes are affected.

ea <= Ra + Rb

Rt <= Mem[ea][7:0]

IF Rc then update cr0

LH – load half-word

| 496 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and the immediate value supplied in the instruction. Register Rb is loaded with the half-word addressed at the memory location. The half-word value is sign extended to the width of the register.

ea <= Ra + immediate

Rb <= Mem[ea][15:0]

LHU – load unsigned half-word

| 516 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and the immediate value supplied in the instruction. Register Rb is loaded with the half-word addressed at the memory location. The half-word value is zero extended to the width of the register.

ea <= Ra + immediate

Rb <= Mem[ea][15:0]

LHUX – load unsigned half-word, indexed addressing

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 516 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and Rb. Register Rt is loaded with memory at the memory address. The half-word value is zero extended to the width of the register. If the Rc bit is set in the instruction, then condition register zero is updated, otherwise no condition codes are affected.

ea <= Ra + Rb

Rt <= Mem[ea][15:0]

IF Rc then update cr0

LHX – load half-word, indexed addressing

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 496 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and Rb. Register Rt is loaded with memory at the memory address. The half-word value is sign extended to the width of the register. If the Rc bit is set in the instruction, then condition register zero is updated, otherwise no condition codes are affected.

ea <= Ra + Rb

Rt <= Mem[ea][7:0]

IF Rc then update cr0

### LINK

Link is used at the beginning of a subroutine in order to establish a stack frame.

| 546 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation

Ra => (SP)+			; Ra is pushed onto the stack

Rt <= SP			; Rt is set equal to the stack pointer

SP <= SP – Immediate	; a stack frame is allocated

Comment:

Rt and Ra should be the same register

Example Code:

A_Subroutine:

LINK R30,#24	; setup stack frame, reserved 24 bytes

…….

UNLK R30		; deallocate stack frame, restore stack pointer

RTS

LW – load word

| 486 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and the immediate value supplied in the instruction. Register Rb is loaded with the word addressed at the memory location.

ea <= Ra + immediate

Rb <= Mem[ea]

LWX – load word, indexed addressing

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 486 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and Rb. Register Rt is loaded with memory at the memory address. If the Rc bit is set in the instruction, then condition register zero is updated, otherwise no condition codes are affected.

ea <= Ra + Rb

Rt <= Mem[ea]

IF Rc then update cr0

MULU[.] – unsigned multiply

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 246 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The target register Rt is loaded with the product of Ra and Rb. Condition code register zero may be updated with result flags. IF the high order bits of the product are non-zero, then the overflow flag may be set.

Rt <= Ra * Rb

If Rc then Cr0 updated

Example:

MULU.	R1,R2,R3

TVS		CR0

MULS[.] – signed multiply

| 26 | Ra5 | Rb5 | Rt5 |  | Rc | 266 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The target register Rt is loaded with the product of Ra and Rb. Condition code register zero may be updated with result flags. IF the high order bits of the product are not equal to the sign bit, then the overflow flag may be set.

Rt <= Ra * Rb

If Rc then Cr0 updated

ORI. – or immediate

| 96 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The bitwise OR of Ra and the immediate value supplied in the instruction are loaded into Rt.

Rt <= Ra or immediate

Cr0 is updated.

PUSH – push registers onto stack

PUSH is used to push registers onto the stack. Up to five registers may be pushed by this instruction.

| 596 | Ra5 | Ra5 | Rc5 | Rd5 | Re5 |  | PUSH / POP |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operations

For reg = a to e

if regno <> 0

R[regno] => -(SP)

POP – pop registers from stack

POP is used to pop registers back off the stack. Up to five registers may be popped from the stack by this instruction

| 536 | Ra5 | Ra5 | Rc5 | Rd5 | Re5 |  | PUSH / POP |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operations

For reg = a to e

if regno <> 0

(SP)+ => R[regno]

RTS – return from subroutine

| 06 | Offs4 | Immediate16 | 346 | RTS |
| --- | --- | --- | --- | --- |

Operation:

First, the program counter is popped from the stack. The four bit offset field is shifted left twice and added to the program counter. Next the stack pointer is incremented by a 16 bit constant.

PC <= (SP)+ + Offs4 << 2

SP <= SP + Immediate16

Comment:

The offset field can cause the program counter to be reset to a point several words after calling instruction. This can be used to skip over static parameters which are placed in-line.

Example Code:

PUSH	R1/R2/R3		; push three words

JSR	someroutine

DC.L	0x1234		; first static parameter

DC.L	0x5678		; second static parameter

……

…….

someroutine:

LINK R30,#24

…….

UNLK R30

RTS #2,#12			; bypass 2 static parameters, add 12 to SP

SETcc - set register based on condition

| 186 | CRa3 |  |  | Cond4 | Rt5 |  |  | SETcc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

If the condition in the specified condition code register is true, then the target register is set to one, else it is set to zero.

if condition(CRa) is true

Rt <= 1

else

Rt <= 0

SB – store byte

| 586 | Ra5 | Rb5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and the immediate value supplied in the instruction. Memory at the memory address is set equal to the low order byte of Rb.

ea <= Ra + immediate

Mem[ea] <= Rb[7:0]

SBX – store byte, indexed addressing

| 26 | Ra5 | Rb5 | Rc5 |  | 0 | 586 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and Rb. Memory at the memory address is set equal to the low order byte of Rc.

ea <= Ra + Rb

Mem[ea] <= Rc[7:0]

SH – store half-word

| 576 | Ra5 | Rb5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and the immediate value supplied in the instruction. Memory at the memory address is set equal to the low order 16 bits of Rb.

ea <= Ra + immediate

Mem[ea] <= Rb[15:0]

SHX – store half-word, indexed addressing

| 26 | Ra5 | Rb5 | Rc5 |  | 0 | 576 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and Rb. Memory at the memory address is set equal to the low order 16 bits of Rc.

ea <= Ra + Rb

Mem[ea] <= Rc[15:0]

STOP – stop clock

| 06 |  |  |  | sf | tf | Im3 | 536 | R |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The STOP instruction causes the processor’s internal clock to be disabled, resulting in all activity in the processor ceasing. In addition, the interrupt mask, trace and system mode flags may be set by the instruction. The clock may be restarted by a reset, or non-maskable interrupt.

Im <= Ir[8:6]

Tf <= ir[9]

Sf <= ir[10]

SUBI. – subtract immediate

| 56 | Ra5 | Rt5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The difference of Ra and the immediate value supplied in the instruction is loaded into Rt.

Rt <= Ra - immediate

Cr0 is updated.

SW – store word

| 566 | Ra5 | Rb5 | Immediate16 | RI |
| --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and the immediate value supplied in the instruction. Memory at the memory address is set equal to Rb.

ea <= Ra + immediate

Mem[ea] <= Rb

SWX – store word, indexed addressing

| 26 | Ra5 | Rb5 | Rc5 |  | 0 | 586 | RR |
| --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

The memory address is calculated as the sum of Ra and Rb. Memory at the memory address is set equal to Rc.

ea <= Ra + Rb

Mem[ea] <= Rc

TRAPcc - trap based on condition

| 176 | CRa3 |  |  | Cond4 |  |  |  | Tcc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Operation:

If the condition in the specified condition code register is true, then trap, else NOP.

if condition(CRa) is true

trap

Comment:

TVS, TVC use the overflow trap vector.

Example – trap on add overflow:

ADDI	R1,R2,#1234

TVS	CR0

UNLK – unlink stack frame

UNLK is used at the end of a subroutine in order to de-allocate the stack frame.

| 16 | Ra5 | Rt5 |  |  | 246 | R |
| --- | --- | --- | --- | --- | --- | --- |

Operations:

SP <= Ra	; stack pointer is set equal to Ra

Rt <= (SP)+	; Rt is popped from the stack

Comment:

Ra and Rt should be the same register.