MDCT IP Core
Specification

Author: Michal Krepa

Rev. 1.1

July 30, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 15/04/2006 | Michal Krepa | Initial Draft |
| 1.1 | 17/04/2006 | Michal Krepa | Updated all sections |
| 1.2 | 17/04/2006 | Michal Krepa | Updated Testbench section |
| 1.3 | 21/04/2006 | Michal Krepa | Added memory information |

Contents

Introduction	3

Architecture	3

Operation	3

Registers	3

List of Registers	3

Register 1 – Description	3

Clocks	3

IO Ports	3

Appendix A	3

Appendix B	3

Index	3

Introduction

The MDCT core is two dimensional discrete cosine transform implementation designed for use in compression systems like JPEG. Architecture is based on parallel distributed arithmetic with butterfly computation. Done as row/column decomposition where two 1D DCT units are connected through transposition matrix memory. Core has simple input FIFO interface. Design is synchronous, with single positive clock edge and no internal tri-state buffers. Latency between first latched input data and first DCT transformed output is 106 clock cycles. Core uses double buffered RAM for storing intermediate product results after first DCT stage for maximized performance. This way both 1D DCT units can work in parallel effectively creating dual stage pipeline. MDCT core takes 8 bit input data and produces 12 bit output using 12 bit DCT matrix coefficients. This may be enhanced to be configurable in the future.

Self checking testbench is included for testing operation of the MDCT core. Testbench takes matlab-converted bitmap image, MDCT core DCT-transforms it and testbench compares input image to reconstructed one by performing behavioral IDCT. Peak signal to noise ratio is computed to see how big error is introduced by fixed point arithmetic used in core. Testbench can also perform quantization of DCT output, but when this option is enabled DCT core operation is not verified. Testbench created DCT images (reconstructed image and error image) can be converted to JPEG format by matlab scripts (included).

Core was not yet tested on real HW FPGA, so it is considered beta release.

Architecture

This section describes the architecture of the MDCT. Below is I/O schematic of MDCT core.

ready

odv

dcto

odv1

dcto1

clk

rst

dctiidv

MDCT

core

Figure 1. Top level schematic

Block diagram is presented below describing the top level of the design.

output DCT

input image

DCT 1D

1st stage

DCT 1D

2nd stage

Transpose RAM

DBUFCTL

ROM memories

ROM memories

Transpose RAMs

Figure 2. Block diagram of MDCT core

MDCT core architecture is based two 1D DCT units connected through transpose matrix RAM. Transposition RAM is double buffered, that is, when 2nd stage of DCT reads out data from transposition memory 1, 1st DCT stage can populate 2nd transposition memory with new data. This enables creation of dual stage pipeline where every stage consist of 1D DCT and transposition memory. 1D DCT units are not internally pipelined, they use parallel distributed arithmetic with butterfly computation to compute DCT values. Because of parallel DA they need considerable amount of ROM memories to compute one DCT value in  single clock cycle. Design based on distributed arithmetic does not use any multipliers for computing MAC (multiply and accumulate), instead it stores precomputed MAC results in ROM memory and grab them as needed. DBUFCTL block is essentially a memory arbiter between 1D DCT stages. Each stage can request memory buffer for read/write and basing on availability one of two RAM buffers is granted. Every RAM has 10 bit width data and 64 memory cells.

On chip RAMs used are dual port synchronous memory with one clock cycle delay. These RAMs are currently 64 words x 14 bits size and there are two of them to create ping-pong buffers for improved throughput.

Design uses also ROM memories for storing precomputed MAC coefficients. ROMs can be either asynchronous or synchronous. ./SOURCE/ROME.VHD and ./SOURCE/ROMO.VHD memory models included in core sources are synchronous synthesizable ROM memory models which can be used for simulation and implementation. But, beware they seems to only synthesize to block rams with mentor tools even if block rom inference is disabled (did not try with other tools). If you want have asynchronous distributed ROMs remove clocking and input address register from these ROMs or use ./SOURCE/XILINX/ROM(E/O).VHD – CoreGen created synchronous distributed ROM memories.

Operation

Interface for host communication if based on FIFO access, ie. ready pin is used for flow control. When ready pin is high (and idv pin is high) data from dcti bus is sampled. Pin ready shall be connected to read enable input of FIFO core. Regarding output DCT handling, there is odv pin - when high means that dcto output is valid (dcto output is 2D DCT transformed data).

Below is example of host data transfer to MDCT core. When idv is sampled high, core requests memory and when granted latches input data to shift register acknowledging it by raising ready pin high – this reads next data from host FIFO. When MDCT core finishes reading 8 input data values it checks if 1D DCT latched them in its own register. If yes, then it can start latching new data.

Figure 3. MDCT read input values

Registers

There are no internal registers in the MDCT core.

Clocks

This section specifies all the clocks.

There is only one clock used by core.

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |
| clk | Input Pad | N/A | N/A | N/A | Duty cycle N/A | Core clock |

Table 1: List of clocks

IO Ports

This section specifies the core IO ports.

MDCT core has following ports:

ready

odv

dcto

odv1

dcto1

clk

rst

dctiidv

MDCT

core

Figure 4. Core interface

| PIN | width | direction | description |
| --- | --- | --- | --- |
| clk | 1 | input | clock |
| rst | 1 | input | async reset |
| dcti | 8 | input | input data |
| idv | 1 | input | input data valid (image sample) |
| ready | 1 | output | ready for new data (read enable for FIFO) |
| odv | 1 | output | output data valid |
| dcto | 12 | output | output data (DCT transformed) |
| odv1 | 1 | output | output data valid after 1st DCT stage (for debug) |
| dcto1 | 12 | output | output data after 1st DCT stage (for debug) |

Table 2. Pins description

Performance

Core Performance/area with current implementation.

Performance: 512 x 512 x 8 bit image is 2D DCT transformed in about 33 ms with 10 MHz input clock. This gives 7.9 mega samples per second throughput with this frequency.

Area: 1545 slices and 2 RAMB16s Blockrams on Xilinx Spartan3 xc3s1000 device (45 MHz max freq) with asynchronous on chip ROM. Using synchronous distributed ROM (with one clock delay) area is larger (1620 slices) but performance is better (55 MHz).

Testbench

Self-veryfing testbench included which takes matlab-converted image as input. Core transforms it to DCT coefficients and behavioral IDCT testbench code reconstructs from it original image. PSNR is computed between original and reconstructed image to find out error introduced by fixed point arithmetic, for sample Lena images PSNR is 48 dB.

Matlab scripts are included for computing floating point DCT/IDCT as reference. Scripts for converting 8 bit bitmap to txt format readable by testbench and vice versa are also available.

You can see here MDCT core-transformed image and reconstructed by behavioral IDCT.

There are few Matlab scripts to aid core verification stored in ./Matlab directory:

- dct_func.m

Matlab dct_func() function which computes floating point 1D DCT using butterfly method. Warning: output from this function is transposed, so applying this function twice will produce 2D DCT without need to transpose after 1st stage.

- dctf.m
m-file for testing dct_func() operation. Before input is taken by dct_func() it is level shifted (128 number is subtracted from every input sample). M-file dctf.m also performs quantization after DCT for testing purposes using quantization matrix mentioned in JPEG STD.

dctm.m
Computes 2D DCT using equation:

2N

1)v

(2j

cos

2N

1)u

(2i

cos

y)

x(x,

C(v)

C(u)

2

v)

X(u,

1

j=0

1

N





























i=0

where C(u),C(v) 	= 2-1/2	for u=0,v=0

N

= 1	otherwise

This equation was used as reference for checking validity of row/column method. Before input is taken to equation above it is level shifted (128 number is subtracted from every input sample).

- img2txt.m
Converts bitmap (*.bmp) image to txt format readable by MDCT testbench core. If you want to use your own image for testing do the following:

Set your bitmap filename in first line of img2txt.m and copy your bitmap to ./Matlab folder. Run img2txt from Matlab – this will create txt file with extension *.txt and basename as your bmp filename. Then copy ‘filename’.txt to ./SOURCE/TESTBENCH  and update constant FILEIN_NAME_C to match your image basename. Then you are good to go with running testbench. Note that image sizes (width, height) must be multiply of DCT block size which is set to 8.

- txt2img.m
Converts testbench output images (error image and reconstructed image) to JPEG format. After VHDL testbench finished testing, copy ./SOURCE/TESTBENCH/imageo.txt and ./SOURCE/TESTBENCH/imagee.txt to ./MATLAB folder and run txt2img.m to obtain imageo.jpg and imagee.jpg. Then you can visually estimate if testing went good.

This section contains an alphabetical list of helpful document entries with their corresponding page numbers.