MDCT IP Core
Specification

Author: Michal Krepa

Rev. 1.1

July 31, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 15/04/2006 | Michal Krepa | Initial Draft |
| 1.1 | 17/04/2006 | Michal Krepa | Updated all sections |

Contents

Introduction	3

Architecture	3

Operation	3

Registers	3

List of Registers	3

Register 1 – Description	3

Clocks	3

IO Ports	3

Appendix A	3

Appendix B	3

Index	3

Introduction

The MDCT core is two dimensional discrete cosine transform implementation designed for use in compression systems like JPEG. Architecture is based on parallel distributed arithmetic with butterfly computation. Done as row/column decomposition where two 1D DCT units are connected through transposition matrix memory. Core has simple input FIFO interface. Design is synchronous, with single positive clock edge and no internal tri-state buffers. Latency between first latched input data and first DCT transformed output is 106 clock cycles. Core uses double buffered RAM for storing intermediate product results after first DCT stage for maximized performance. This way both 1D DCT units can work in parallel effectively creating dual stage pipeline. MDCT core takes 8 bit input data and produces 12 bit output using 12 bit DCT matrix coefficients. This may be enhanced to be configurable in the future.

Self checking testbench is included for testing operation of the MDCT core. Testbench takes matlab-converted bitmap image, MDCT core DCT-transforms it and testbench compares input image to reconstructed one by performing behavioral IDCT. Peak signal to noise ratio is computed to see how big error is introduced by fixed point arithmetic used in core. Testbench can also perform quantization of DCT output, but when this option is enabled DCT core operation is not verified. Testbench created DCT images (reconstructed image and error image) can be converted to JPEG format by matlab scripts (included).

Core was not yet tested on real HW FPGA, so it is considered beta release.

Architecture

This section describes the architecture of the MDCT. Below is I/O schematic of MDCT core.

ready

odv

dcto

odv1

dcto1

clk

rst

dctiidv

MDCT

core

Figure 1. Top level schematic

Block diagram is presented below describing the top level of the design.

output DCT

input image

DCT 1D

1st stage

DCT 1D

2nd stage

Transpose RAM

DBUFCTL

ROM memories

ROM memories

Transpose RAMs

Figure 2. Block diagram of MDCT core

MDCT core architecture is based two 1D DCT units connected through transpose matrix RAM. Transposition RAM is double buffered, that is, when 2nd stage of DCT reads out data from transposition memory 1, 1st DCT stage can populate 2nd transposition memory with new data. This enables creation of dual stage pipeline where every stage consist of 1D DCT and transposition memory. 1D DCT units are not internally pipelined, they use parallel distributed arithmetic with butterfly computation to compute DCT values. Because of parallel DA they need considerable amount of ROM memories to compute one DCT value in  single clock cycle. Design based on distributed arithmetic does not use any multipliers for computing MAC (multiply and accumulate), instead it stores precomputed MAC results in ROM memory and grab them as needed. DBUFCTL block is essentially a memory arbiter between 1D DCT stages. Each stage can request memory buffer for read/write and basing on availability one of two RAM buffers is granted. Every RAM has 10 bit width data and 64 memory cells.

Operation

Interface for host communication if based on FIFO access, ie. ready pin is used for flow control. When ready pin is high (and idv pin is high) data from dcti bus is sampled. Pin ready shall be connected to read enable input of FIFO core. Regarding output DCT handling, there is odv pin which when high means that dcto output is valid (dcto output is 2D DCT transformed data).

Below is example of host data transfer to MDCT core. When idv is sampled high, core requests memory and when granted latches input data to shift register acknowledging it by raising ready pin high – this reads next data from host FIFO. When MDCT core finishes reading 8 input data values it checks if 1D DCT latched them in its own register. If yes, then it can start latching new data.

Figure 3. MDCT read input values

Registers

There are no internal registers in the MDCT core.

Clocks

This section specifies all the clocks.

There is only one clock used by core.

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |
| clk | Input Pad | N/A | N/A | N/A | Duty cycle N/A | Core clock |

Table 1: List of clocks

IO Ports

This section specifies the core IO ports.

MDCT core has following ports:

| PIN | width | direction | description |
| --- | --- | --- | --- |
| clk | 1 | input | clock |
| rst | 1 | input | async reset |
| dcti | 8 | input | input data |
| idv | 1 | input | input data valid (image sample) |
| ready | 1 | output | ready for new data (read enable for FIFO) |
| odv | 1 | output | output data valid |
| dcto | 12 | output | output data (DCT transformed) |
| odv1 | 1 | output | output data valid after 1st DCT stage (for debug) |
| dcto1 | 12 | output | output data after 1st DCT stage (for debug) |

Table 1. Pins description

Performance

Core Performance/area with current implementation.

Performance: 512 x 512 x 8 bit image is 2D DCT transformed in about 33 ms with 10 MHz input clock. This gives 7.9 mega samples per second throughput with this frequency.

Area: 1545 slices and 2 RAMB16s Blockrams on Xilinx Spartan3 xc3s1000 device (45 MHz max freq).

Testbench

Self-veryfing testbench included which takes matlab-converted image as input. Core transforms it to DCT coefficients and behavioral IDCT testbench code reconstructs from it original image. PSNR is computed between original and reconstructed image to find out error introduced by fixed point arithmetic, for sample Lena images PSNR is 48 dB.

Matlab scripts are included for computing floating point DCT/IDCT as reference. Scripts for converting 8 bit bitmap to txt format readable by testbench and vice versa are also available.

You can see here MDCT core-transformed image and reconstructed by behavioral IDCT.

This section contains an alphabetical list of helpful document entries with their corresponding page numbers.