Pattern Generator

Author: Luis Munoz

Lfmunoz4@gmail.com

Rev. 1.0

July 30, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 09/20/11 | Luis Munoz | Specification, first version |

Contents

1 Introduction	1

2 Architecture	2

3 Operation	3

4 Generics	5

5 Clocks	6

6 IO Ports	7

Introduction

This VHDL module generates video patterns which can be used for testing video displays. It currently supports four patterns; horizontal lines, vertical lines, moving horizontal lines and moving vertical lines. It sends out one pixel out at a time forming a video pattern on the fly. This module has been successfully used for testing a Camera Link serializer and SDXC Card video streamer.

Architecture

The Pattern Generator is currently implemented using counters and the pixels are generated dynamically based on the current counter values. (In a future version, if there is enough interest, there will be a version using block rams).

Operation

Figure 1: Timing diagram of typical use

| Name | SEL_i |
| --- | --- |
| Vertical Lines | 001 |
| Horizontal Lines | 010 |
| Moving Vertical Lines | 010 |
| Moving Horizontal Lines | 100 |

Table 1: Generics for transmitter and receiver

Figure 2: Pattern result of vertical lines

Figure 2: Pattern result of horizontal lines

Generics

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| FrameWidth | Integer | 4-32bits | The number of pixels per line |
| FrameHeight | Integer | 4-32bits | The number of lines |
| PIXEL_SIZE | Integer | 4-32bits | Size of a pixel |
| REG_SIZE | Integer | 4-32bits |  |

Table 2: Generics for transmitter and receiver

Clocks

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |
| CLK_i | - | - |  | - |  | This clock is used by all the logic, limited by the device. |

Table 3: List of clocks

IO Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| RST_i | 1 | Input | Puts the module in a known state |
| SEL_i | 1 | Input | Selects the pattern to output, either horizontal lines, vertical lines, moving horizontal lines, or moving vertical lines |
| CLKen_i | 1 | Input | Enables the pixels to change, it effectively just prevents the clock from having an effect |
| PIXEL_o | PIXEL_SIZE | Input | The actual pixels |
| VALID_o | 1 | Output | High when a pixel is valid |
| ENDline_o | 1 | Output | High for 1 cycle on the last pixel of each line |
| ENDframe_o | 1 | Output | High for 1 cycle on the last pixel of each frame |

Table 4: Module Signals