Programmable Interval Timer (PIT)
Specification

Author: Robert Hayes

rehayes@opencores.org

Rev. 0.1

July 30, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 14/04/09 | Robert Hayes | First draft release |
| 0.2 | 22/05/09 | Robert Hayes | Changed the recommended connection of ext_sync_i for a PIT module that is only to be used in Master mode from “tie low” to “tie to cnt_sync_o”. Added wb_sel_i to the signals lists. Corrected minor typo’s. |
| 0.3 | 03/06/09 | Robert Hayes | Added documentation on the SINGLE_CYCLE parameter. |
| 0.4 | 10/02/10 | Robert Hayes | Updated timing constraints for WISHBONE bus when wait states are enabled. |

Contents

Introduction	1

FEATURES	1

Architecture	2

2.1 WISHBONE Interface	3

2.2 Control Registers	3

2.3 Prescale Counter	3

2.4 Main Counter	3

Operation	4

Registers	6

List of Registers	6

4.1 CNTRL Register	7

4.1 MOD Register	10

4.2 CNT Register	11

Clocks	12

IO Ports	13

6.1 WISHBONE Interface	14

6.2 PIT signals	15

6.3 PIT Core Parameters	16

Appendix A	19

Index	20

Introduction

The Programmable Interval Timer Module, PIT, is a simple timer to generate a periodic signal for a microcontroller system. This signal may be used for a variety of purposes such as triggering the start of an Analog to Digital or Digital to Analog conversion, as a periodic system interrupt, or to synchronize the start of various other hardware processes.

The PIT RTL code is parameterized so that every module instantiation can be customized to meet system requirements without wasting unneeded resources.

### FEATURES

- 16 bit Main Counter with programmable modulo

- 15 bit Prescale Counter with programmable modulo selections

- Slave mode for synchronizing multiple PIT modules

- Interrupt or bit-polling

- Static synchronous design

- Fully synthesizable

- Parameterized so each instance can be optimized for size

Architecture

The PIT core is built around four primary blocks; the WISHBONE Interface, the Control Registers, the Prescale Counter and the Main Counter.

cnt_sync_o

Prescale

Counter

WISHBONE Interface

Command Register

Modulo Register

Count Register

Main

Counter

ext_sync_i

pit_o

cnt_flag_o

pit_int_o

Fig. 2.1 Internal structure PIT Core

### 2.1 WISHBONE Interface

The WISHBONE Interface isolates the PIT functionality from the WISHBONE bus. This interface takes the bus specific signals and generates a generic set of control signals to drive the PIT control registers. Isolating the WISHBONE bus should help promote PIT module reusability by localizing the scope of changes needed to retarget the PIT module to another bus environment.

### 2.2 Control Registers

The Command Register, Modulo Register and Count Register are combined into a single module that controls the programmable functions of the PIT. Various bits in the Command Register define the basic operating mode and function enables of the PIT.

### 2.3 Prescale Counter

The Prescale counter extends the range of the main counter. Using the Prescale counter reduces the timing granularity in terms of master clocks over which the period of the output interrupts can be controlled. As an example if the Prescale Counter modulo is set for a value of 10 then counts of 90 or 100 of the master clock could be decoded but not a master clock count of 95.

### 2.4 Main Counter

The Main Counter increments at a rate determined by the setting of the Prescale modulo value and the master clock frequency. Each time the Main Counter reaches the modulo value the PIT output signals are activated and the PIT FLAG status bit is set.

Operation

The PIT module is a very simple counter that can be controlled by commands from the WISHBONE bus. The output from the module is a pulse that is one wb_clk_i period wide at a frequency of:

pit_o = wb_clk_i / (MOD_VAL * PRE_SCALR)

Where MOD_VAL is a 16-bit register accessed from the WISHBONE bus and PRE_SCLAR is a four-bit value accessed from the WISHBONE bus that is decoded into 16 possible multiplier values.

Figure 3.1 is an illustration for the case of a PIT module operating with MOD_VAL=5 and PRE_SCLR=0. The start of the counters incrementing is determined by the setting of the CNT_EN bit in the control register. If the PIT module is operating in slave mode then the CNT_EN signal shown in the figure is actually the [ext_sync_i] signal that is controlled by the CNT_EN bit of the master PIT module. Note that the fall time of the signal [cnt_flag_o] is not explicitly defined because it is dependent on receiving a command from the WISHBONE bus to reset the FLAG register.

MSTR_CLK

0002

0003

0001

0004

0005

0001

0002

0003

0004

0005

0001

0002

0003

0004

0005

0001

0002

0003

PIT_O

CNT_FLAG_O

MSTR_ROLL

MSTR_CNT

CNT_EN

Fig. 3.1 PIT Signals, MOD_VAL=5

MSTR_CLK

0001

0001

0002

0001

0002

0001

0002

0001

0002

0001

0002

0001

0002

0001

0002

PIT_O

CNT_FLAG_O

0001

0002

0002

0001

0002

0003

0004

0004

0001

0001

0003

0002

PRE_SCALE

MSTR_CNT

PRE_OUT

MSTR_ROLL

CNT_EN

Figure 3.2 is an illustration of the timing relationships of a PIT module operating with MOD_VAL=4 and PRE_SCLR=1(Divide by 2).

Fig. 3.2 PIT Signals, MOD_VAL=4, PRE_SCLR=1

The recommended software procedure for using the PIT Module:

- Initialize PIT Slave modules

- Set MOD_VAL

- Set SLAVE, PRE_SCALR, and ENA_INT

- Initialize PIT Master module

- Set MOD_VAL

- Set SLAVE, PRE_SCALR, and ENA_INT

- Set CNT_EN

Registers

The PIT module can be configured through the use of the DWIDTH parameter to have a WISHBONE bus interface with either an 8-bit bus with 8-bit granularity or to use a 16-bit bus with 16-bit granularity. This document shows the resultant address and bit field for both configurations although in an actual instance of the PIT module only one of the pairs of tables will be valid. For an end user or programmer reference it may be best to simplify this document by removing the tables that reference the unused bus configuration.

### List of Registers

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |
| CNTRL | 0x00 | 16 | RW | PIT Control Register |
| MOD | 0x01 | 16 | RW | PIT Modulo Register |
| CNT | 0x02 | 16 | R | PIT Counter Value |

Table 1: List of registers, 16 bit data (default DWIDTH=16)

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |
| CNTRL_0 | 0x00 | 8 | RW | PIT Control Register Low |
| CNTRL_1 | 0x01 | 8 | RW | PIT Control Register High |
| MOD_0 | 0x02 | 8 | RW | PIT Modulo Register Low Byte |
| MOD_1 | 0x03 | 8 | RW | PIT Modulo Register High Byte |
| CNT_0 | 0x04 | 8 | R | PIT Counter Value Low Byte |
| CNT_1 | 0x05 | 8 | R | PIT Counter Value High Byte |

Table 1a: List of registers, 8 bit data (DWIDTH=8)

### 4.1 CNTRL Register

| Bit # | Access | Description |
| --- | --- | --- |
| 15 | RW | SLAVE, Slave mode enable bit When set to ‘1’ the Prescaler Counter and the Main Counter are enabled by an external master mode PIT. This allows the slave mode PIT counters to be synchronized to the CNT_EN bit of the master mode PIT module. When set to ‘0’ the PIT counters are enabled by the CNT_EN bit. |
| 14 | R | ###### DECADE_CNTR, Returns the value of the DECADE_CNTR parameter |
| 13 | R | ###### NO_PRESCALE, Returns the value of the NO_PRESCALE parameter. |
| 12 | R | ###### Reserved |
| 11:8 | RW | PRE_SCALR, Prescale Counter modulo value. Sets the period of the Prescale Counter in bus clocks. See Table 3 If the NO_PRESCALE parameter is set then this field will always return all zeros. |
| 7:3 | R | ###### Reserved |
| 2 | W | ###### FLAG, Main counter rollover status flag. Read - Returns the current status of the rollover status flag and the interrupt output state. ###### Write – When set to ‘1’ clears the FLAG status bit and the interrupt output. When set to ‘0’ there is no effect. |
| 1 | W | ###### ENA_INT, Interrupt enable. When set to ‘1’ the interrupt output is enabled. ###### When set to ‘0’ the interrupt output is disabled. Clearing this bit only disables the interrupt output, it does not clear the source of the interrupt which is the FLAG status bit. |
| 0 | W | ###### CNT_EN, Prescale and Main Counter enable. Clearing this bit resets the Prescale and Main counter to their default state. This bit has no effect when the SLAVE bit is set. |
| Reset Value: |  |  |
|  |  | CNTRL: 0000h |

Table 2: CNTRL Register Bits

| Bit # | Access | Description |
| --- | --- | --- |
| 7 | RW | SLAVE, Slave mode enable bit When set to ‘1’ the Prescaler Counter and the Main Counter are enabled by an external master mode PIT. This allows the slave mode PIT counters to be synchronized to the CNT_EN bit of the master mode PIT module. When set to ‘0’ the PIT counters are enabled by the CNT_EN bit. |
| 6 | R | ###### DECADE_CNTR, Returns the value of the DECADE_CNTR parameter |
| 5 | R | ###### NO_PRESCALE, Returns the value of the NO_PRESCALE parameter. |
| 4 | R | ###### Reserved |
| 3:0 | RW | PRE_SCALR, Prescale Counter modulo value. Sets the period of the Prescale Counter in bus clocks. See Table 3 If the NO_PRESCALE parameter is set then this field will always return all zeros. |

Reset Value:

CNTRL_1: 40h

Table 2a: CNTRL Register Bits

| Bit # | Access | Description |
| --- | --- | --- |
| 7:3 | R | ###### Reserved |
| 2 | W | ###### FLAG, Main counter rollover status flag. Read - Returns the current status of the rollover status flag and the interrupt output state. ###### Write – When set to ‘1’ clears the FLAG status bit and the interrupt output. When set to ‘0’ there is no effect. |
| 1 | W | ###### ENA_INT, Interrupt enable. When set to ‘1’ the interrupt output is enabled. ###### When set to ‘0’ the interrupt output is disabled. Clearing this bit only disables the interrupt output, it does not clear the source of the interrupt which is the FLAG status bit. |
| 0 | W | ###### CNT_EN, Prescale and Main Counter enable. Clearing this bit resets the Prescale and Main counter to their default state. This bit has no effect when the SLAVE bit is set. |

Reset Value:

CNTRL_0: 00h

Table 2b: CNTRL Register Bits

| PRE_SCALR | DECADE_ENA = 0 | DECADE_ENA = 1 |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 2 | 2 |
| 2 | 4 | 4 |
| 3 | 8 | 8 |
| 4 | 16 | 10 |
| 5 | 32 | 100 |
| 6 | 64 | 1000 |
| 7 | 128 | 10000 |
| 8 | 256 | 20000 |
| 9 | 512 | 20000 |
| 10 | 1024 | 20000 |
| 11 | 2048 | 20000 |
| 12 | 4096 | 20000 |
| 13 | 8192 | 20000 |
| 14 | 16384 | 20000 |
| 15 | 32768 | 20000 |

Table 3: PRE_SCLAR Decode Values

### 4.1 MOD Register

16 Bit Data Bus

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | RW | MOD_VAL, Main Counter modulo value Sets the rollover value of the Main Counter. |

Reset Value:

MOD_VAL: 0000h

Table 4: MOD Register Bits

8 Bit Data Bus

| Bit # | Access | Description |
| --- | --- | --- |
| 7:0 | RW | MOD_VAL_0, MOD_VAL[7:0] Main Counter modulo value Sets the rollover value of the Main Counter. |

Reset Value:

MOD_VAL_0: 00h

Table 4a: MOD_VAL_0 Register Bits

| Bit # | Access | Description |
| --- | --- | --- |
| 7:0 | RW | MOD_VAL_1, MOD_VAL[15:8] Main Counter modulo value Sets the rollover value of the Main Counter. |

Reset Value:

MOD_VAL_1: 00h

Table 4b: MOD_VAL_1 Register Bits

### 4.2 CNT Register

16 Bit Data Bus

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | RW | COUNT_VAL, Current state of the Main Counter. |

Reset Value:

CNT: 0001h

Table 5: CNT Register Bits

8 Bit Data Bus

| Bit # | Access | Description |
| --- | --- | --- |
| 7:0 | RW | COUNT_VAL_0, COUNT_VAL[7:0] Current state of the Main Counter. Note: To minimize the gates there is no register to capture the full value of COUNT_VAL when only a byte is read. This means that the value that the processor sees is only the approximate value of COUNT_VAL because one of the bytes will have changed between the read of the first byte of COUNT_VAL and the second byte of COUNT_VAL. |

Reset Value:

CNT_0: 01h

Table 5a: CNT_0 Register Bits

| Bit # | Access | Description |
| --- | --- | --- |
| 7:0 | RW | COUNT_VAL_1, COUNT_VAL[15:8] Current state of the Main Counter. |

Reset Value:

CNT_1: 00h

Table 5b: CNT_1 Register Bits

Clocks

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |
| wb_clk_i | System | 200 | - | - | Master clock for all PIT registers. Positive edge active. | System clock. |

Table 3: List of clocks

The wb_clk_i has no timing constraints based on the RTL implementation although there may be constrains applied for synthesis results to be compatible with the target physical implementation. If the PIT is targeted for an ASIC implementation then [wb_clk_i] should be used as the scan clock, any clock multiplexing required to make [wb_clk_i] the scan clock should be done at the system level external to the PIT module.

IO Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| wb_clk_i | 1 | Input | WISHBONE Bus Clock Input, Master Clock |
| wb_rst_i | 1 | Input | WISHBONE Synchronous Reset |
| wb_adr_i | 3 | Input | WISHBONE Lower address bits |
| wb_dat_i | 8/16 | Input | WISHBONE Bus Data |
| wb_dat_o | 8/16 | Output | WISHBONE Bus Data |
| wb_we_i | 1 | Input | WISHBONE Write enable |
| wb_stb_i | 1 | Input | WISHBONE Strobe signal/Core select |
| wb_cyc_i | 1 | Input | WISHBONE Valid bus cycle |
| wb_sel_i | 2 | Input | WISHBONE Data Bus Byte Select |
| wb_ack_o | 1 | Output | WISHBONE Bus cycle acknowledge |
| pit_int_o | 1 | Output | PIT Interrupt signal |
| pit_o | 1 | Output | PIT output pulse |
| cnt_flag_o | 1 | Output | Counter rollover flag |
| cnt_sync_o | 1 | Output | Sync signal to slave mode PIT |
| ext_sync_i | 1 | Input | Sync signal from master mode PIT |
| arst_i | 1 | Input | Asynchronous Reset |

Table 4: List of IO ports

### 6.1 WISHBONE Interface

The core features a WISHBONE RevB.3 compliant WISHBONE Classic interface that operates in SLAVE mode. All output signals are registered. Each access takes 2 clock cycles. To limit a WISHBONE access to just two clock cycles the following synthesis rules should be used:

- Single cycle timing for wb_cyc_i, wb_stb_i, wb_adr_i and wb_data_o. The current bus design latches the wb_cyc_i, wb_stb_i, and wb_adr_i signals in the first period and decodes the address in the second period.

- Two cycle timing for wb_data_i. (Single cycle timing could be used but it would be a waste of  resources to meet an over constrained timing path.)

- Note: Use the “SINGLE_CYCLE” parameter to do a WISHBONE bus access in one clock cycle.

| WISHBONE DATASHEET |  |  |
| --- | --- | --- |
| Description | Specification |  |
| General description: | 8-bit SLAVE |  |
| Supported Cycles: | SLAVE, READ/WRITE |  |
| Data port, size: Data port, granularity: Data port, maximum operand size: Data transfer ordering: Data transfer sequencing: | Default: 16, option 8 bit Default: 16, option to match 8 bit port size |  |
| Supported signal list and cross reference to equivalent WISHBONE signals: | Signal Name | WISHBONE Equiv. |
|  | wb_clk_i | CLK_I |
|  | wb_rst_i | RST_I |
|  | wb_adr_i | ADR_I() |
|  | wb_dat_i | DAT_I() |
|  | wb_dat_o | DAT_O() |
|  | wb_we_i | WE_I |
|  | wb_stb_i | STB_I |
|  | wb_cyc_i | CYC_I |
|  | wb_sel_i | SEL_I |
|  | wb_ack_o | ACK_O |

#### 6.1.1 wb_rst_i

The synchronous reset signal has a minimum pulse width requirement of one [wb_clk_i] clock period. It will take two [wb_clk_i] clock cycles for all registers in the PIT module to initialize. Also see information on pin [arst_i].

#### 6.1.2 wb_adr_i

Connections to the WISHBONE address pin will depend on the size of the WISHBONE data bus that is set by the DWIDTH parameter. If DWIDTH=8 the all address pins should be connected, if DWIDTH=16 then [wb_adr_i(2)] should be tied low.

#### 6.1.3 wb_sel_i

The [wb_sel_i] is the WISHBONE byte lane select signal. It is currently unimplemented in the PIT module and should be tied hi.

### 6.2 PIT signals

#### 6.2.1 pit_o

The PIT output signal. This is a one master clock, [wb_clk_i], wide pulse that is output when the Master Counter rolls over after reaching the value stored in the Modulo Register.

#### 6.2.2 cnt_flag_o

This signal is an optional output that tracks the value in the FLAG register. The positive edge becomes active at the same time as the [pit_o] signal and the negative edge is dependent on the PIT interrupt service routine, or polling loop, to clear the FLAG register. This signal may be occasionally useful when an output signal with a longer pulse width is needed to be resynchronized to a slower clock domain than the PIT master clock.

#### 6.2.3 pit_int_o

This signal is the interrupt output from the PIT. The timing and function of this signal is the same as [cnt_flag_o] with an additional output inhibit provided by the ENA_INT control bit.

#### 6.2.4 cnt_sync_o

A PIT that is to be used in master mode will use this signal to synchronize the start of counting in PIT instances that are operating in slave mode. All slave mode PIT’s that will synchronize to a common master will connect their [ext_sync_i] inputs to this output. The CNT_EN control register drives this signal.

#### 6.2.5 ext_sync_i

A PIT module operating in slave mode will use this signal to synchronize the start of its counters to the setting of the CNT_EN register in the associated master mode PIT. If the PIT module is never to be used in SLAVE mode then this input signal should be tied to the cnt_sync_o signal.

#### 6.2.6 arst_i

The signal [arst_i] is an asynchronous reset signal that goes to all flops in the PIT. It is provided for FPGA implementations and test methodologies that require this function for initialization. Using [arst_i] instead of [wb_rst_i] can result in lower cell-usage and higher performance for a FPGAs implementation because the standard FPGA cell already provides a dedicated asynchronous reset path. Using [wb_rst_i] for an ASIC implementation might synthesize to a smaller module because smaller non_reset flops can be used. Use either [arst_i] or [wb_rst_i], tie the other to a negated state. The active level of [arst_i] is determined by the parameter ARST_LVL which defaults to active low.

### 6.3 PIT Core Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| ARST_LVL | Bit | 1’b0 | Asynchronous reset level |
| COUNT_SIZE | Int | 16 | Number of bits in Main Counter and modulo |
| DECADE_CNTR | Bit | 1’b1 | Prescaler decode, 0 = Binary counter, 1 = Decade counter |
| NO_PRESCALE | Bit | 1’b0 | Prescale counter disabled, 0 = Prescale counter enabled, 1 = Prescale counter disabled |
| SINGLE_CYCLE | Bit | 1’b0 | WISHBONE wait state |
| DWIDTH | Int | 16 | Data Bus size |

#### 6.3.1 ARST_LVL

The asynchronous reset level can be set to either active high (1’b1) or active low (1’b0).

Allowed values: 1’b0, 1’b1

#### 6.3.2 COUNT_SIZE

The number of bits in the Main Counter. This feature is not fully implemented in the current PIT RTL.

Allowed values: 16-1

#### 6.3.3 DECADE_CNTR

The Prescale Counter can be set to count as a binary decoded output or a decade decoded output. Setting this value may offer a more intuitive programmer interface. Clearing this bit will allow the maximum prescale value and may also slightly reduce the amount of combinational logic required to decode the Prescale Counter output.

Allowed values: 1’b0, 1’b1

#### 6.3.4 NO_PRESCALE

Setting this bit should remove the Prescale Counter from the synthesis results to save size and power if this function is not needed to meet system requirements.  Synthesis scripts should be coded to allow for the removal of flops with unconnected outputs. Synthesis logfiles should be reviewed to ensure the Prescale counter and it’s control registers are removed.

Allowed values: 1’b0, 1’b1

#### 6.3.5 SINGLE_CYCLE

The default operation of the PIT WISHBONE bus interface is to insert one wait state by delaying the assertion of the wb_ack_o by one wb_clk_i period. Setting the SINGLE_CYCLE parameter generates the wb_ack_o combinationaly  so a WISHBONE bus cycle can be completed in one wb_clk_i period. Also the input address register is removed, saving three flops, which tightens the timing constraints on a bus read transaction.

Allowed values: 1’b0, 1’b1

#### 6.3.6 DWIDTH

The width of the microcontroller data buses connected to PIT module. The PIT module can support either an 8-bit data bus with 8-bit resolution or a 16-bit data bus with 16-bit resolution.

Allowed values: 8, 16

Name

[This section may be added to outline different specifications.]

[This section contains an alphabetical list of helpful document entries with their corresponding page numbers.]