Raptor CPU Core Reference

Robert Finch

© 2007,2012

This page intentionally blank

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2005-08-18 | Robert Finch | First Draft |
| 1.1 | 2007-06-28 | Robert Finch | Switched to SETHI, SETLO, added data type field |
| 1.2 | 2012-04-26 | Robert Finch | 32 regs |

Raptor CPU Core

© 2007 Robert Finch

Home

Table of Contents

1	Overview	7

1.1	Design Objectives	7

1.2	Unique Features	8

1.3	Definitions	8

2	Programming Model	9

2.1	Execution Contexts	9

2.2	General Registers	9

2.2.1	Register Format	10

2.2.2	Floating Point Format	10

2.3	Program Counter (R45)	11

2.3.1	Instruction Packets	11

2.3.2	Program Counter Format	11

2.3.3	Program Counter Relative Addressing	11

2.4	Special Purpose Registers	12

2.4.1	Status Register (SR)	13

2.4.2	Core Number  (COR)	14

2.4.3	Version Register	15

2.4.4	Random Number Register (RND)	16

2.4.5	#32  Vector Table Base Address Register (VBA)	17

2.4.6	#5  Tick Count Register (TIC)	18

2.4.7	Active Executive Context (AXC)	19

2.4.8	Execution Pattern Registers (EP0-3)	19

2.4.9	#30  Floating Point Status and Control Register (FPC)	20

3	Hyper-Threading	22

4	Interrupts	22

4.1	Interrupt Vector Table	23

5	Program Flow Control	24

5.1	Compare Operation	24

5.2	Branches	25

5.2.1	Overview	25

5.2.2	Targets	25

5.2.3	Branch Conditions	26

Branch Conditions	26

5.3	Method Invocation (Jumps and Calls)	27

5.4	System Calls and Interrupts	27

6	Datapath Operations	28

6.1	Immediate Constants	28

6.2	Integer Operations	28

6.2.1	Addition / Subtraction / Multiply / Divide	28

6.2.2	Comparisons	28

6.2.3	Logical Operations	28

6.2.4	Shifts	28

6.2.5	BitField Operations	28

6.3	Floating Point Operations	29

6.3.1	Addition / Subtraction / Multiply / Divide	29

6.3.2	Absolute Value / Negation / Sign Determination / Fractional Extraction	29

6.3.3	Numeric Conversions	29

6.3.4	Comparisons	29

6.3.5	Other Instructions	29

7	Memory Operations	30

7.1	Endian	30

7.2	Operations	30

7.3	Alignment	30

7.4	Memory Map	30

8	Opcode Formats	31

8.1	Instruction Layout	31

8.2	Opcode Formats	32

8.3	Detailed Opcode Formats	33

8.4	Data Type	34

8.5	Opcode Maps	35

8.5.1	Major Instruction Group	35

8.5.2	Register Instruction Group	36

8.5.3	Miscellaneous Control	37

8.5.4	Floating Point Operations	38

9	Instruction Set Description	40

Documentation Notes

This document refers to an eight bit quantity as a byte, a sixteen bit quantity as a character, a thirty-two bit quantity as a half-word, and a sixty-four bit quantity as a word.

# Overview

The Raptor is a powerful multi-context 64 bit processor that represents a generational refinement of processor architecture. The processor contains 32, 32 bit general purpose registers which may be used for either integer or floating point operations. The Raptor uses a fixed 42 bit instruction and handles 8, 16, 32, and 64 bit data within a 64 bit address space.

## Design Objectives

This processor is targeted towards high performance operation as a general purpose processor. Following are some of the criteria that were used on which to base the design.

- Designed for Superscalar operation - the ability to execute more than one instruction at a time. To achieve high performance it is generally accepted that a processor must be able to execute more than a single instruction in any given clock cycle.

- Simplicity - architectural simplicity leads to a design that is easy to implement resulting in reliability and assured correctness along with easy implementation of supporting tools such as compilers. Simplicity also makes it easier to obtain high performance and results in lower overall cost.

- Extensibility - the design must be extensible so that features not present in the first release can easily be added at a later date.

- Low Cost

This design meets the above objectives in the following ways. The instruction set has been designed to minimize the interactions between instructions, allowing instructions to be executed as independent units for superscalar operation. There are a sufficient number of registers to allow the compiler to schedule parallel processing of code. A reasonably large general purpose register set is available making the design reasonably compatible with many existing compilers and assemblers. Where needed, additional specialized instructions have been added to the processor to support a sophisticated operating system and interrupt management.

## Unique Features

Combined compare and branch operations. This is a code compression technique that both reduces code size and increases performance.

Evaluation of logical operations during branch condition determination in addition to the usual relational operations. This is another code compression and thus performance enhancing feature.

The processor supports mystery logical operations where the exact operation to be performed is unknown until runtime, at which point the operation to be performed is determined by the value of a register. This is useful in applications such as raster graphics operations and ciphering.

## Definitions

Instruction Packet:

An instruction packet is a sixteen byte aligned block of memory that contains four instructions.

# Programming Model

## Execution Contexts

Internally the Raptor uses multiple register contexts in order to support hyper-threading. However, only a single set of registers is visible for programming at one time. A register context consists of the general registers and a program counter.

## General Registers

There are 32 sixty-four bit registers - r0 to r31. Twenty-eight of the registers are available for any use. The other registers have dedicated uses.

- R0 always reads as zero and can’t be updated.

- R25 is automatically updated by hardware to store the current program counter when an interrupt occurs.

- R26 is the conventional constant building register. This register is used by the assembler to build constants that won’t fit into the immediate field of an instruction.

- R29 reflects the address of the instruction and can’t be updated. It may be used to generate program counter relative addresses.

- R30 is the conventional stack pointer

- R31 is automatically updated by hardware to store the current program counter when a subroutine call occurs.

| Code (5 bits) | Register Name | Description / Suggested Usage Convention |
| --- | --- | --- |
| 0 | R0 or 0 | This register always reads as zero |
| 1 | R1 | First return value / first argument |
| 2 | R2 | Second return value / second argument |
| 3 to 23 | R3-R23 | Application usage. |
| 24 | C0 | constant hold |
| 25 | ILR | Interrupt link register (interrupt return address) |
| 26 | C2 | Constant building reghister |
| 27 | BP | Base Pointer |
| 28 | XRA | Exception return address |
| 29 | PC | Program counter. This is an alias for the instruction address. This register is read only. |
| 30 | SP | Stack Pointer. Any hardware based stack operations will use this register. |
| 31 | RA | Return Address register. This register is used to store the return address from a subroutine or method call. |

- Programming Model

| Special Registers |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 63 23 22 0 |  |  | 63 0 |  |
| Program Counter |  | PB | PC |  | R0/0 |  |
| Link Register |  | PB | PC |  | R1 |  |
| Except. Address |  | PB | PC |  | R2 |  |
|  |  |  |  |  | R3 |  |
|  |  |  |  |  | R4 |  |
| System Vec. |  | SV |  |  | R5 |  |
|  |  |  |  |  | R6 |  |
|  |  |  |  |  | R7 |  |
|  |  |  |  |  | R8 |  |
|  |  |  |  |  | R9 |  |
|  |  |  |  |  | R10 |  |
|  |  |  |  |  | R11 |  |
|  |  | 63 0 |  |  | R12 |  |
|  |  |  |  |  | R13 |  |
| Tick |  | TICK |  |  | R14 |  |
|  |  |  |  |  | R15 |  |
|  |  |  |  |  | R16 |  |
|  |  |  |  |  | R17 |  |
|  |  |  |  |  | R18 |  |
|  |  |  |  |  | R19 |  |
|  |  |  |  |  | R20 |  |
|  |  |  |  |  | R21 |  |
|  |  |  |  |  | R22 |  |
|  |  |  |  |  | R23 |  |
|  |  |  |  |  | R24 |  |
|  |  |  |  |  | R25/ILR |  |
|  |  |  |  |  | R26 |  |
|  |  |  |  |  | R27/BP |  |
|  |  |  |  |  | R28 |  |
|  |  |  |  |  | R29/PC |  |
|  |  |  |  |  | R30/SP |  |
|  |  |  |  |  | R31/LR |  |

General purpose register R0 always contains the value zero.

The Tick register increments with every clock cycle. It may be set using the ‘mtspr’ instruction.

### Register Format

Integer values in registers are represented in twos complement format.

| 63 | 62 0 |
| --- | --- |
| Sign | value |

### Floating Point Format

Floating point values are represented as IEEE-754 standard single precision values. This is a sign-magnitude format.

| Sign | Exponent | Mantissa |
| --- | --- | --- |
| 31 | 30 23 | 22 0 |

## Program Counter

### Instruction Packets

An instruction packet is just a fancy name for an aligned sixteen byte block of memory that contains three instructions.

The processor uses thirty-two bit instructions with opcodes packed four per 128 bits as shown in the format below.

|  | 127 96 | 95 64 | 63 32 | 31 0 |
| --- | --- | --- | --- | --- |
| Opcode Slot 3 |  | Opcode Slot 2 | Opcode Slot 1 | Opcode Slot 0 |

Instruction addresses are composed of a packet number which indicates which sixteen byte memory area contains the instruction, and a slot number. The instruction slot number is one of zero, one,  two or three. The slot number indicates which opcode within the packet is active.

Any one of the opcodes within an instruction packet may be used as a target address.  The program counter contains a packet address and slot number. As a program is executed the slot number increments from zero to one to three, then back to zero, while the packet number increments when the slot number changes from three to zero. The packet number matches with a sixteen-byte aligned data address. The slot number just indicates which opcode within the packet is active.

### Program Counter Format

| 63 2 | 1 0 |
| --- | --- |
| Instruction Packet Number | Slot Number |

### Program Counter Relative Addressing

R29 contains the value of the program counter for the instruction currently being executed. R29 may be used to form program counter relative addresses.

### Execution Contexts

There is a separate program counter register for each execution context. All program counter registers are initialized to the reset vector ($FFFF_FFFF_FFFF_FFF0) on reset. A context jump table may be setup in software to vector to different code by testing the contents of the active execution context (AXC) register.

## Special Purpose Registers

There are several special purpose registers within the processor. The processor model may be extended to support additional functionality at a later date.

| Code | Register Name | Description |
| --- | --- | --- |
| 0 | SR | Status Register |
| 1 | COR | CORe number |
| 2 | THR | {this register is reserved – thread number} |
| 3 | VER | Processor Version / Features - major, minor, revision |
| 4 | RND | random number register |
| 5 | TIC | tick count register |
| Execution Context Control |  |  |
| 8 | EP0 | Execution Pattern |
| 9 | EP1 |  |
| 10 | EP2 |  |
| 11 | EP3 |  |
| 12 | AXC | active execution context |
| 15 | PTA | Page Table Address |
| 14…31 |  | {these register are reserved} |
| 30 | FPC | floating point control |
| 32 | VBA | vector table base address |
| 33-47 |  | {reserved library registers} |
|  | TLBPageMask | Controls the memory page size |
|  | TLBVirtAddr |  |
|  | TLBPhysAddr |  |
|  | TLBASID |  |
|  | ASID | Address space identifier (process id) |

These registers may be accessed using the TRS and TSR commands.

### Status Register (SR)

This register contains bits that control the overall operation of the processor or reflect the processor’s state. Bits are included for interrupt masking, and system / application mode indicator. This register is split into two halves with both halves having the same format. The lower half of the register is what determines how the processor works. The upper half of the register maintains a backup copy of the lower half for interrupt processing. There are instructions provided for manipulating the interrupt mask. (EI, DI, and RIF).

| 31..16 | 15 | 14 | 13 | 12 | 11..10 | 9..0 |
| --- | --- | --- | --- | --- | --- | --- |
| same format as 15..0 | Interrupt Mask | Reserved | Kernel / Application Mode Indicator | Float Except. Enable |  | reserved |
|  | IM | ~ | S | FXE |  | ~ |

IM = interrupt mask

Maskable interrupts are disabled when this bit is set.

### Core Number  (COR)

This register is used to identify which core is active in a multi-core system.

| 31 28 | 27 24 | 23 16 | 15 0 |
| --- | --- | --- | --- |
| Board4 | Core4 | Process8 | Thread16 |

### Version Register

This register contains the major, minor, and revision numbers of the processor. This register is read only.

| 31 24 | 23 16 | 15 8 | 7 0 |
| --- | --- | --- | --- |
| Major | Minor | Revision | {Reserved} |
| 1 | 0 | 0 | 0 |

### Random Number Register (RND)

This register contains a pseudo-random value based on a LCG (linear congruential generator). The value may be seeded by writing to the register. The register may contain either a random 32 bit integer or rand floating point value between 0.0 and 1.0. The GRAN instruction is used to generate a 32 bit integer value. The GRAF instruction is used to generate a floating point value.

LCG Parameters – 32 bit

next = (cur * a + c) mod M

a = 69069 decimal

c = 1

M = 2^32

31 0

random number

63 0

random number

### Vector Table Base Address Register (VBA)

This register establishes the base address of the interrupt vector table in memory. At reset it is set to zero, the first 4KB page of memory or $0000. The low order thirteen bits of this register are always zero. The vector number from the system call instruction is combined with this register to form the address from which the vector should be taken. This register is read only unless the processor is kernel mode.

| 63 13 | 12 0 |
| --- | --- |
| FFFF_FFFF_FFFF_E | 000 |

### Tick Count Register (TIC)

This register contains a count of the number of clock cycles that have passed since the last time the register was written. It may be useful for performance monitoring. This register is read only unless the processor is supervisor mode.

|  | 63 0 |  |
| --- | --- | --- |
|  | clock cycle count |  |

### Active Executive Context (AXC)

This register contains a number identifying the currently active executive context. This register is read-only.

| 31 4 | 3 0 |
| --- | --- |
| 0 | Number |

### Execution Pattern Table (EP)

The execution pattern table is a table with 256 available buckets. Each bucket can contain a four bit execution context number.

This table records the frequency of execution for each context.

Contexts are executed based on their frequency of appearance in the execution pattern table.

The execution pattern registers must be set in order to allow other contexts to run.

The CTX0 bucket is fixed at context zero it canot be altered. This guarentees that context zero (the kernel context) always gets a chance to run.

### Floating Point Status and Control Register (FPC)

This register contains control values for floating point and floating point status bits.

| Bit |  | Symbol | Description |
| --- | --- | --- | --- |
| 31:30 | RM | rm | rounding mode |
| 29 |  |  | {reserved} |
| 28 | E5 | inexe | - inexact exception enable |
| 27 | E4 | dbzxe | - divide by zero exception enable |
| 26 | E3 | underxe | - underflow exception enable |
| 25 | E2 | overxe | - overflow exception enable |
| 24 | E1 | invopxe | - invalid operation exception enable |
| 23 | NS | ns | - non standard floating point indicator |
| Result Status |  |  |  |
| 22 |  | fractie | - the last instruction (arithmetic or conversion) rounded intermediate result (or caused a disabled overflow exception) |
| 21 | RA | rawayz | rounded away from zero (fraction incremented) |
| 20 | SC | C | denormalized, negative zero, or quiet NaN |
| 19 | SL | neg < | the result is negative (and not zero) |
| 18 | SG | pos > | the result is positive (and not zero) |
| 17 | SE | zero = | the result is zero (negative or positive) |
| 16 | SI | inf ? | the result is infinite or quiet NaN |
| Exception Occurrence |  |  |  |
| 15 | X6 | swt | {reserved} - set this bit using software to trigger an invalid operation |
| 14 | X5 | inerx | - inexact result exception occurred (sticky) |
| 13 | X4 | dbzx | - divide by zero exception occurred |
| 12 | X3 | underx | - underflow exception occurred |
| 11 | X2 | overx | - overflow exception occurred |
| 10 | X1 | giopx | - global invalid operation exception – set if any invalid operation exception has occurred |
| 9 | GX | gx | - global exception indicator – set if any enabled exception has happened |
| 8 | SX | sumx | - summary exception – set if any exception could occur if it was enabled - can only be cleared by software |
| Exception Type Resolution |  |  |  |
| 7 | X1T | cvt | - attempt to convert NaN or too large to integer |
| 6 | X1T | sqrtx | - square root of non-zero negative |
| 5 | X1T | NaNCmp | - comparison of NaN not using unordered comparison instructions |
| 4 | X1T | infzero | - multiply infinity by zero |
| 3 | X1T | zerozero | - division of zero by zero |
| 2 | X1T | infdiv | - division of infinities |
| 1 | X1T | subinfx | - subtraction of infinities |
| 0 | X1T | snanx | - signaling NaN |

| RM | Rounding Mode |
| --- | --- |
| 0 | round towards nearest even |
| 1 | round towards zero (truncate) |
| 2 | round towards positive infinity |
| 3 | round towards negative infinity |

* add floating point invalid operation exception instruction

### TLBPageMask

The TLBPageMask register contains a mask that controls which address bits are significant during a TLB lookup. If a mask bit is ‘1’ then that address bit is not significant for comparison purposes. This allows the size of memory management pages to be varied.

| 63 32 | 31 13 | 12 0 |
| --- | --- | --- |
| ---- | Page Mask | ---- |

|  | Mask | Page Size |
| --- | --- | --- |
|  | 0000000000000000000 | 8kB |
|  | 0000000000000000011 | 16KB |
|  | 000000000000001111 | 64kB |
|  | 000000000000111111 | 256kB |
|  | 000000000011111111 | 1MB |
|  | 0000000001111111111 | 4MB |
|  | 000000011111111111 | 16MB |
|  | 000000111111111111 | 64MB |
|  | 000011111111111111 | 256MB |
|  | 001111111111111111 | 1GB |
|  | 111111111111111111 | 4GB |

### TLBPhysPage

The TLBPhysPage register is a holding register that contains the physical address for an associated virtual address. This register is transferred to or from the TLB by TLB instructions.

| 63 13 | 12 0 |
| --- | --- |
| Physical Page Address | ---- |

### TLBVirtPage

The TLBVirtPage register is a holding register that contains the virtual address for an associated physical address. This register is transferred to or from the TLB by TLB instructions.

| 63 13 | 12 0 |
| --- | --- |
| Virtual Page Address | ---- |

### TLBASID

The TLBASID register is a holding register that contains the address space identifier (ASID) , valid, dirty, global, and cachability bits associated with a TLB entry. This register is transferred to or from the TLB by TLB instructions.

| 63 16 | 15 8 | 6 4 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
| ----- | ASID | C | G | D | V |

Hyper-Threading

The Raptor supports hyper-threading via the use of multiple contexts. The processor can switch in a controlled fashion between contexts on a cycle-by-cycle basis. Which context is active during any clock cycle depends on the list configured in the execution pattern table. The core cycles through the entries in the execution registers using the context number (0-15) stored in the list register to determine the next context to use during execution. Execution context zero is reserved for processing hardware exceptions. When a hardware exception occurs, the processor automatically forces execution context zero to become active, overriding the contents of the execution pattern list.

# Interrupts

Interrupts cause an automatic transfer of processor execution to context zero.

This processor supports interrupts and exceptions in the form of a system call instruction. When a hardware interrupt or program exception occurs the appropriate system call instruction is forced into the processor's internal instruction pipeline (this is accomplished by hardware internal to the processor). There are several pre-defined exception numbers for specific vectors in order to support hardware interrupts as listed in the table below. An exception vectors to the handler whose address is located by combining the exception number (a field in the system call instruction)  with the base address stored in the vector base address register (VBA). The REX1 (return from exception) instruction is used to return from an exception routine.

Occurrence of an exception results in the status register being copied to the back up version, the maskable interrupt mask being set to disable further interrupts, and the exception return address being copied to XRA, so that these may be restored by the exception return operation. The processor is also switched to system mode, which enables registers and instructions available only to system mode.

## Interrupt Vector Table

The interrupt vector table is a table of 512 entries containing the addresses of routines that may be called using the system call instruction (SYS). They may also be called during hardware interrupt processing. The location of the table in memory is controlled by the vector base address register (VBA) which on reset is set to $0000_0000_0000_0000, effectively placing the vector table in the first 4KiB of memory.

The reset vector is not governed by the VBA and has fixed addresses in memory of $FFFF_FFFF_FFFF_FFF0.

The last 64 entries in the table are reserved for interrupts that the processor might generate. The remaining entries are free for use as needed by the operating system.

| Entry # | Description |
| --- | --- |
| Operating System ( reserved) |  |
| 0 |  |
| 32 | TRAP |
| BIOS (32 reserved) |  |
| 416 | RTC |
| 417 | Keyboard |
|  | Network |
|  | Mass Storage |
| 447 |  |
| Processor Hardware Exceptions (64 reserved) |  |
| 448 | spurious interrupt |
| 449 | hardware interrupt level 1 (IRQ) |
| 450 | hardware interrupt level 2 |
| ... | ... |
| 479 | hardware interrupt level 31 |
| 480-487 | {reserved} |
| 488 | division by zero |
| 489 | overflow |
| 490-493 | {reserved} |
| 494 | illegal instruction address |
| 495 | unimplemented instruction |
| 496 | Privilege violation |
| 497-505 | {reserved} |
| 506 | TLB exception – data |
| 507 | TLB exception - instruction |
| 508 | Bus Error – load / store or I/O |
| 509 | Bus Error – instruction fetch |
| 510 | Non-maskable interrupt vector |
| 511 | Reset Vector |

# Program Flow Control

There are four basic flow control instructions (JAL, Bcc, JMP, CALL, RET, and SYS).

JMP is an unconditional program flow transfer instruction.

JAL is an unconditional program flow transfer instruction.

The Bcc instructions are conditional transfers.

The RET instruction is a return instruction that also performs an add operation,

and the SYS instruction is used to invoke exception handlers.

## Compare Operation

Conditional program flow instructions are guided by comparisons. The comparisons may be performed directly by the branch instruction, or via one of the datapath comparison instructions such as a floating point comparison. The compare instruction compares either two registers or a register and an immediate value and writes the result of the comparison to the target register. The target register will be written with a one to represent a true value or a zero for false. There are six basic relational comparison operations than can be performed (equal, not equal, less than, less than or equal, greater than or greater than or equal).

There are ten integer comparison instructions which cover the standard relational comparisons for both signed and unsigned values. (SLT, SGE, SLE, SGT, SLTU, SGEU, SLEU, SGTU, SEQ, SNE).

There are two additional comparison operations (CMP, CMPU) which perform a relational comparison between a register and either another register or immediate value; and place the result of the comparison to the target register. For these two instructions the target register is written with -1 to indicate the first operand is less than the second, 0 to indicate that both operands are the same, and +1 to indicate that the first operand is greater than the second. CMP performs a signed comparison, CMPU performs an unsigned comparison.

## Branches

### Overview

Branches are the most frequent form of program flow control operation and are usually performed in a conditional manner. Branches are integrated with compare operations allowing a comparison and a branch to be processed as one unit for improved code density and performance.

The processor includes a (2,2) 64 entry correlating branch outcome predictor. This allows many branch instructions to execute in only a single clock cycle.

### Targets

Branch target addresses are determined in one of two fashions; the branch target may be an absolute address stored in a register, or it may be a relative address specified by a field in the branch instruction itself. The processor allows a twelve bit target address field which covers virtually 100% of branch cases.

Packet-Relative Addresses

When the branch target is not coming from a register, the branch target field contains a packet relative address and an absolute instruction slot number. This mechanism allows for easy relocation of code on packet boundaries. The twelve bit target field is split into two subfields – a relative packet displacement and the instruction slot number. The relative packet displacement is used to determine which instruction packet is the target relative to the packet of the branch instruction. The slot number indicates which slot within the target packet is the target for the branch. The slot number must have a value of 0, 1, 2 or 3.

Packet displacements may vary from -512 to +511 packets.

Target Field Specifier (non-register targets)

| 11 2 | 1 0 |
| --- | --- |
| Packet Displacement | Slot |

### Branch Conditions

A standard set of branch conditions is provided, outlined in the table below. The branch conditional is determined by comparing the two registers or a register and a small immediate value specified by the branch instruction. The branch architecture is such that it is possible to branch based directly on a value in a register. The ISA features logical tests in addition to the regular relational tests.

### Branch Conditions

| Mnemonic | Description | Conditional Test |
| --- | --- | --- |
| BLT | Branch if Less Than | a < b |
| BGE | Branch if Greater than or Equal | a >= b |
| BLE | Branch if Less than or Equal | a <= b |
| BGT | Branch if Greater Than | a > b |
| BEQ / BXNOR | Branch if Equal | a == b |
| BLTU | Branch if Less Than | a < b |
| BGEU | Branch if Greater than or Equal | a >= b |
| BLEU | Branch if Less than or Equal | a <= b |
| BGTU | Branch if Greater Than | a > b |
| BNE / BXOR | Branch if Not Equal | a <> b |
| BAND | Branch if both true | a and b |
| BOR | Branch if either true | a or b |

## Method Invocation (Jumps and Calls)

Methods may be invoked by the JAL. instruction. JAL automatically stores the method return address in the return address link register (R31). A method return instruction (RET) is used to return from a method by restoring the program counter from the return address register.

The JAL instruction is a simple powerful instruction common in many newer architectures that provides for most common program flow transfer operations that are not covered by branches. The JAL instruction is a generic unconditional program flow control instruction. This single instruction can perform regular jump, method call and return operations, interrupt call and return operations, using absolute or register indirect addresses. The default form of the JAL instruction where the return address is stored in the return address register (R31) may referred to as a CALL in assembly language. When a jump (JMP) is required, the register to store the return address in may be specified as R0 (R0 will not be updated).

The JAL instruction may be used to perform program counter relative calls with the use of the program counter alias register, or program offset based calls by loading a register with the address of the program. JAL may target absolute addresses in memory by setting the target register field to R0.

JAL forms a target address by adding a register and immediate value, or two registers. The address generated is treated as an instruction address (It’s not a byte address).

## System Calls and Interrupts

Calls to the operating system or interrupt routines may be made using the system call (SYS) instruction. Returns from system routines may be made using the ERET instruction. System, interrupt calls and returns cause transfers between processor control bits and their alternates. The system call instruction automatically places the processor in system mode.

# Datapath Operations

## Immediate Constants

Large immediate values used in integer arithmetic operations may be loaded into a register using the ‘set’ instructions (SETLO, SETMID, and SETHI).

## Integer Operations

Integers are represented using two’s complement notation.

### Addition / Subtraction / Multiply / Divide

Basic addition (ADD) and subtraction (SUB) operations are supported. Addition and subtraction work with both signed and unsigned values. The ADD and SUB instructions will cause an exception to occur if there is an overflow condition. The unsigned ADDU and SUBU instructions can be used as an alternate and do not cause an overflow exception.

Multiplication and division of signed numbers are supported by the ISA with the MUL, MULH and DIV instructions A remainder function (MOD) is also available. The MULH instruction returns the high order thirty-two bits of a signed multiply operation.

### Comparisons

There are ten comparison instructions performing signed and unsigned comparisons between two registers or a register and an immediate value (SLT, SLE, SGT SGE, SLTU, SLEU, SGTU SGEU, SEQ, and SNE).

### Logical Operations

The processor supports a standard set of logical operations including and (AND), or (inclusive)(OR), and exclusive or (XOR). An additional derived operation is NOT which is an alternate form of the XOR instruction. The primary logical operations support both register and immediate forms. There are three more additional secondary logical operations (NAND, NOR, and XNOR) which have only register formats.

### Shifts

The ISA supports a full complement of shift instructions. Left shifts are supported via the SHL instruction. Right shifts are supported with arithmetic (ASR) and logical (SHR) shifts. Rotates are supported via the rotate left ROL and rotate right ROR instructions.

### BitField Operations

The processor has support for bitfield operations via the BFINS (bitfield insert) and BFEXT (bitfield extract) instructions. An additional bitfield operation is the BFCLR (bitfield clear) instruction which is an extended mnemonic for the BFINS instruction.

## Floating Point Operations

### Addition / Subtraction / Multiply / Divide

Basic addition (FADD), subtraction (FSUB), multiplication (FMUL) and division (FDIV) operations are supported.

### Absolute Value / Negation / Sign Determination / Fractional Extraction

Absolute value (FABS), negation (FNEG), sign determination (FSGN), fractional extraction (FMAN) are supported.

### Numeric Conversions

Conversions between integer and floating point values are handled with the float-to-int (F2INT) and int-to-float instructions (INT2F).

### Comparisons

There are five floating point comparison instructions; four are relational comparisons, the fifth tests for ordering of operands. (FCLT, FCLE, FCEQ, FCNE, FCOR). The floating point comparisons compare the values in two registers and set the target register to true if the comparison is true. Other comparisons may be synthesized from existing instructions; for example, by switching the operands around.

### Other Instructions

A random floating point value may be obtained using the GRAF (Get RAndom Float) instruction.

# Memory Operations

## Endian

The Raptor is a little endian processor. The least significant byte is at the lowest address.

## Operations

The Raptor is a load / store architecture; the only operations accessing memory are load and store operations. The processor supports byte (8 bit), character (16 bit), half-word (32 bit), and word (64 bit) loads and stores of data to memory.

The ISA supports address update forms of the load / store instructions, where the addressing register is updated with the calculated effective address after the load / store operation is complete. Indexed address mode is also supported. There are instructions which automatically unpack and pack floating point values for transfer to or from memory.

There are numerous load and store instructions (not all are shown) which support the following operations:

| operation | suffix character | Sample Mnemonics |
| --- | --- | --- |
| default / sign extension (loads) | None | LB LH LW SB SH SW |
| zero extension (loads) | Z | LBZ LHZ |
| indexed operation |  | LB LH LW SB SH SW FLS FSS LBZ LHZ |
| floating point conversion |  | LF SF |

## Data Alignment 32-bit

Memory is byte addressable; character, half-word and word accesses must be aligned on their respective address boundaries.

Bytes may be loaded from any memory address.

| 3 | 2 | 1 | 0 |
| --- | --- | --- | --- |

Characters may be loaded from any memory address, except an address where the least significant two bits are 11.

| 2 |  | 0 |  |
| --- | --- | --- | --- |
|  | 1 |  |  |

CLB

If the address ends in 11, then the byte is loaded into register bits 9 to 15; otherwise the instruction is treated as NOP.

| 0 |  |  |  |
| --- | --- | --- | --- |

| 6 |  | 4 |  | 2 |  | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 |  | 3 |  | 1 |  |  |

|  |  |  |  | 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |

|  | 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  |

## Data Alignment

Memory is byte addressable; character, half-word and word accesses must be aligned on their respective address boundaries.

Bytes may be loaded from any memory address.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Characters may be loaded from any memory address, except an address where the least significant three bits are 111.

When a character is loaded from an address ending in 111, the load is treated as if it were a byte load instruction. The lowest byte of the target register is loaded.

CLC

If the address ends in 11, then a byte is loaded into register bits 8 to 15; otherwise the instruction is treated as NOP.

| 6 |  | 4 |  | 2 |  | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 |  | 3 |  | 1 |  |  |

CLH

If the address ends in 101, then a byte is loaded into register bits 24 to 31;

If the address ends in 110, then a character is loaded into register bits 16 to 31;

If the address ends in 111, then a character is loaded into register bits 8 to 31;

otherwise the instruction is treated as NOP.

|  |  |  |  | 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  | 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |

|  | 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  |

LCA

LCZA

If the address ends in 111, then 8 bits are loaded into register bits 8 to 15; otherwise the instruction is treated as NOP.

LHA

LHZA

If the address ends in 101, then a byte is loaded into register bits 24 to 31;

If the address ends in 110, then a character is loaded into register bits 16 to 31;

If the address ends in 111, then a character is loaded into register bits 8 to 31;

otherwise the instruction is treated as NOP.

LWA

If the address ends in 001, then 56 bits are loaded into register bits 8 to 63;

If the address ends in 010, then 48 bits are loaded into register bits 16 to 63;

If the address ends in 011, then 40 bits are loaded into register bits 24 to 63;

If the address ends in 100, then 32 bits are loaded into register bits 32 to 63;

If the address ends in 101, then 24 bits are loaded into register bits 40 to 63;

If the address ends in 110, then 16 bits are loaded into register bits 48 to 63;

If the address ends in 111, then 8 bits is loaded into register bits 56 to 63;

otherwise the instruction is treated as NOP.

## Memory Map

The processor supports a 64 bit (or 2^64 Bytes) address space.

| FFFx_xxxx_xxxx_xxxx | Unmapped |  |
| --- | --- | --- |
| FFEx_xxxx_xxxx_xxxx | Unmapped |  |
| FFDx_xxxx_xxxx_xxxx | Unmapped |  |
| FFCF_FFFF_FFFF_FFFF | mapped |  |
| 0000_0000_0000_0000 |  |  |

## TLB

The processor uses a 16 entry TLB (translation lookaside buffer) in order to support virtual memory. The TLB supports variable page sizes from 8kB to 4GB.

TLB Entries:

Virtual Address63...13

Physical Address63...13

ASID7...0

G

V

D

C2..0

Physical Address63...13

G = Global

ASID = address space identifier

C = cachability bits

D = dirty bit

V = valid bit

If the G bit is set in the TLB entry, then the ASID field is ignored during the address comparison.

Opcode Formats

Ideally from a compiler's perspective the ISA should be able to support three operand instructions (one destination, two sources) including cases where all three operands are registers in order to allow a compiler to allocate registers more efficiently. This is commensurate with current compiler technology. This ISA supports the ideal condition.

## Instruction Layout

Opcodes are thirty-two bits in size. Instructions are placed four per instruction packet which must be aligned on 16-byte (128 bit) addresses.

|  | 127 96 | 95 64 | 63 32 | 31 0 |
| --- | --- | --- | --- | --- |
| 0 | Opcode Slot 3 | Opcode Slot 2 | Opcode Slot 1 | Opcode Slot 0 |
| 1 | Opcode Slot 3 | Opcode Slot 2 | Opcode Slot 1 | Opcode Slot 0 |
| 2 | Opcode Slot 3 | Opcode Slot 2 | Opcode Slot 1 | Opcode Slot 0 |
| … | -- | … | … | … |
| 30 | Opcode Slot 3 | Opcode Slot 2 | Opcode Slot 1 | Opcode Slot 0 |
| 31 | Opcode Slot 3 | Opcode Slot 2 | Opcode Slot 1 | Opcode Slot 0 |

## Opcode Formats

There are several different opcode formats, see the subsequent tables for specific formats. The major opcode formats are shown below. All instructions begin with a seven bit field in the most significant bits which determines the major opcode grouping or instruction.

I-Type

Usage: Register-immediate instructions,

loads and stores with displacements,

jump and link

| 31 25 | 24 20 | 19 15 | 14 0 |
| --- | --- | --- | --- |
| Opcode7 | RegA5 | RegT5 | Immediate15 |

R-Type

Usage: Register-Register instructions and loads and stores using indexed addressing

| 31 25 | 24 20 | 19 15 | 14 10 | 9 7 | 6 0 |
| --- | --- | --- | --- | --- | --- |
| Opcode7 | RegA5 | RegB5 | RegT5 | ----3 | Func7 |

B-Type

Usage: Branches

| 31 25 | 24 20 | 19 8 | 7 0 |
| --- | --- | --- | --- |
| Opcode7 | RegA5 | target12 | Immediate8 |

BF – Type

To be updated

Usage: Bitfield operations

| 41 35 | 34 29 | 28 23 | 22 17 | 16 12 | 11 6 | 5 0 |
| --- | --- | --- | --- | --- | --- | --- |
| Opcode7 | RegA5 | RegT5 | RegC5 | Opcode8 | Width6 | Start6 |

## Detailed Opcode Formats

| Op4 | Imm10 |  |  | Rt6 | Imm23 |  |  |  | SET |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Op6 |  | Typ2 | Ra6 | Rt6 | Imm22 |  |  |  | Imm |
| Op6 |  | Typ2 | Ra6 | Rt6 | Rc6 | Op10 |  | Rb6 | Reg |
| Op6 |  | Typ2 | Ra6 | Rt6 | Rc6 | Op4 | Wd6 | St6 | BFC |
| Op6 |  | Typ2 | Ra6 | Rt6 | Rc6 | Op4 | Wd6 | Rb6 | BFR |
| Op6 |  | Typ2 | Ra6 | target12 |  | Imm16 |  |  | Branch |
| Op6 |  | Typ2 | Ra6 | 326 | Rc6 | Imm16 |  |  | Branch |
| Op6 |  | Typ2 | Ra6 | target12 |  | Op8 |  | Rb6 | Branch |
| Op6 |  | Typ2 | Ra6 | 326 | Rc6 | Op8 |  | Rb6 | Branch |

BFC – Type

Bitfield, constant field index

| 41 36 | 35 32 | 31 26 | 25 20 | 19 14 | 13 12 | 11 6 | 5 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Opcode6 | Opcode 4 | RegA6 | RegT6 | RegC6 | Opcode2 | Width6 | Start6 |

BFR – Type

Bitfield, register field index

| 41 36 | 35 32 | 31 26 | 25 20 | 19 14 | 13 12 | 11 6 | 5 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Opcode6 | Opcode 4 | RegA6 | RegT6 | RegC6 | Opcode2 | Width6 | RegB6 |

| Op7 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Misc |  | Miscellaneous |
| 0 |  |  | ---28 |  |  |  |  |  |  |  | 0 | BRK |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  | 2 | IRET |  | Return from interrupt |
| 0 |  |  |  |  |  |  |  |  |  |  | 3 | ERET |  | Return from exception |
| 0 |  |  |  |  |  |  |  |  |  |  | 40 | WAIT |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  | 49 | TLBP |  | Probe TLB entries |
| 0 |  |  |  |  |  |  |  |  |  |  | 50 | TLBR |  | Read TLB entry |
| 0 |  |  |  |  |  |  |  |  |  |  | 51 | TLBWI |  | Write TLB entry using index |
| 0 |  |  |  |  |  |  |  |  |  |  | 52 | TLBWR |  | Write random TLB entry |
| 0 |  |  |  |  |  |  |  |  |  |  | 64 | CLI |  | Clear interrupt mask |
| 0 |  |  |  |  |  |  |  |  |  |  | 65 | SEI |  | Set interrupt mask |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  | Ra5 |  | Rt5 |  | Rc5 |  | Op13 |  | Opcd7 | {reg} |  | register operate |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 4 | COM |  | compliment |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 5 | NOT |  | Logical not |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 6 | NEG |  | Negative -a |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 7 | ABS |  | Absolute value (a) |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 16 | CTLZ |  | Count leading zeros |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 17 | CTLO |  | Count leading ones |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 18 | CTPOP |  | Count population |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 19 | SEXT8 |  | Sign extend 8 to 64 bits |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 20 | SEXT16 |  | Sign extend 16 to 64 bits |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 21 | SEXT32 |  | Sign extend 32 to 64 bits |
| 1 |  |  | Ra5 |  | Rt5 |  |  |  |  |  | 24 | SQRT |  | Square root |
| 1 |  |  |  |  | Rt5 |  |  |  |  | SPR6 | 40 | MFSPR |  | Move from special register |
| 1 |  |  | Ra5 |  |  |  |  |  |  | SPR6 | 41 | MTSPR |  | Move to special register |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 27 | ADD |  | a + b, error on overflow |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 37 | ADDU |  | a + b, no errors |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 47 | SUB |  | a - b, error on overflow |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 57 | SUBU |  | a - b, no errors |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 67 | CMP |  | compare |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 77 | CMPU |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 87 | AND |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 97 | OR |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 107 | XOR |  | Exclusive or |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 117 | ANDC |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 127 | NAND |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 137 | NOR |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 147 | XNOR |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 15 | ORC |  | Or with complement |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 20 | MIN |  | Minimum of a or b |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 21 | MAX |  | Maximum of a or b |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 24 | MULU |  | Unsigned multiply |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 25 | MULS |  | Signed multiply |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 26 | DIVU |  | Unsigned divide |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 27 | DIVS |  | Signed divide |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 30 | MOVZ |  | Conditional move if zero |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 31 | MOVNZ |  | Conditional move if not zero |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 40 | SHL |  | Shift left |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 41 | SHRU |  | Unsigned Shift right |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 42 | ROL |  | Rotate left |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 43 | ROR |  | Rotate right |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 44 | SHR |  | Signed shift right |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 45 | ROLAM |  | Rotate left and mask |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 96 | SLT |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 96 | SGE |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 98 | SLE |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 99 | SGT |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 100 | SLTU |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 101 | SGEU |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 102 | SLEU |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 102 | SGTU |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 104 | SEQ |  |  |
| 2 |  |  | Ra5 |  | Rb5 |  | Rt5 |  | Op13 |  | 105 | SNE |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Basic Arithmetic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | ADDI | Add immediate |
| 5 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | ADDUI | Add immediate |
| 6 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | SUBI | Subtract immediate |
| 7 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | CMPI | Compare immediate |
| 8 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | CMPUI | Unsigned compare immediate |
| 9 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | ANDI | And |
| 10 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | ORI | Or |
| 11 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | XORI | Exclusive or |
| 12 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | MULUI | Unsigned multiply |
| 13 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | MULSI | Signed multiply |
| 14 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | DIVUI | Unsigned divide |
| 15 |  |  | Ra5 |  | Rt5 |  | Imm25 |  |  |  |  |  | DIVSI | Signed divide |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  | Ra5 |  | Rb5 |  |  |  |  |  | Cond7 |  | TRAPcc | Trap on condition |
| 18 |  |  | Ra5 |  | Cond5 |  | Imm25 |  |  |  |  |  | TRAPcci | Trap on condition |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Rt5 |  |  | Imm32 |  |  |  |  |  |  |  |  | SETLO | Set low order bits |
| 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  | Address35 |  |  |  |  |  |  |  |  |  |  | CALL | call subroutine |
| 25 |  | Address35 |  |  |  |  |  |  |  |  |  |  | JMP | jump |
| 26 |  | Ra5 |  |  | Rt5 | Address25 |  |  |  |  |  |  | JAL | jump and link |
| 27 |  | Ra5 |  |  | Rb5 | Rt5 |  | Imm20 |  |  |  |  | RET | return and add |
| 28 | Rt5 |  |  | Imm32 |  |  |  |  |  |  |  |  | SETHI | Set high order bits |
| 29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Memory Operations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | Ra5 | Rt5 | disp25 |  |  | LB | load byte, sign extend |
| 33 | Ra5 | Rt5 | disp25 |  |  | LC | load character, sign extend |
| 34 | Ra5 | Rt5 | disp25 |  |  | LH | load half-word, sign extend |
| 35 | Ra5 | Rt5 | disp25 |  |  | LW | load word |
| 36 | Ra5 | Rt5 | disp25 |  |  | LP | load pair |
| 37 | Ra5 | Rt5 | disp25 |  |  | LBU | load byte, zero extend |
| 38 | Ra5 | Rt5 | disp25 |  |  | LCU | load character, zero extend |
| 39 | Ra5 | Rt5 | disp25 |  |  | LHU | load half-word, zero extend |
| 40 | Ra5 | Sprt5 | disp25 |  |  | LSH | load special half-word |
| 41 | Ra5 | Sprt5 | disp25 |  |  | LSW | load special word |
| 42 | Ra5 | Rt5 | disp25 |  |  | LF | load float |
| 43 | Ra5 | Rt5 | disp25 |  |  | LFD | load float double |
| 44 | Ra5 | Rt5 | disp25 |  |  | LFP | load float pair |
| 45 | Ra5 | Rt5 | disp25 |  |  | LFDP | load float double pair |
| 46 | Ra5 | Rt5 | disp25 |  |  | LWR | load word, reserve address |
| 47 |  |  |  |  |  |  |  |
| 48 | Ra5 | Rt5 | disp25 |  |  | SB | store byte (zero) |
| 49 | Ra5 | Rt5 | disp25 |  |  | SC | store character |
| 50 | Ra5 | Rt5 | disp25 |  |  | SH | store half-word |
| 51 | Ra5 | Rt5 | disp25 |  |  | SW | store word |
| 52 | Ra5 | Rt5 | disp25 |  |  | SP | store pair |
| 53 | Ra5 | Rb5 | Rt5 | disp13 | Op7 |  | Indexed memory operations |
| 54 |  |  |  |  |  |  | reserved |
| 55 |  |  |  |  |  |  | reserved |
| 56 | Ra5 | Sprs5 | disp25 |  |  | SSH | store special half-word |
| 57 | Ra5 | Sprs5 | disp25 |  |  | SSW | store special word |
| 58 | Ra5 | Rb5 | disp25 |  |  | SF | store float |
| 59 | Ra5 | Rbt5 | disp25 |  |  | SFD | store float double |
| 60 | Ra5 | Rb5 | disp25 |  |  | SFP | store float pair |
| 61 | Ra5 | Rb5 | disp25 |  |  | SFDP | store float double pair |
| 62 | Ra5 | Rb5 | disp25 |  |  | SWCR | store word, clear address reservation |
| 63 |  |  |  |  |  |  |  |
| IO Operations |  |  |  |  |  |  |  |
| 64 | Ra5 | Rt5 | I/O Address25 |  |  | INB | input byte, sign extend |
| 65 | Ra5 | Rt5 | I/O Address25 |  |  | INCH | input character, sign extend |
| 66 | Ra5 | Rt5 | I/O Address25 |  |  | INH | input half-word, sign extend |
| 67 | Ra5 | Rt5 | I/O Address25 |  |  | INW | input word |
| 68 | Ra5 | Rt5 | I/O Address25 |  |  | INBU | input byte, zero extend |
| 69 | Ra5 | Rt5 | I/O Address25 |  |  | INCU | input character, zero extend |
| 70 | Ra5 | Rt5 | I/O Address25 |  |  | INHU | input half-word, zero extend |
| 71 |  |  |  |  |  |  |  |
| 72 | Ra5 | Rt5 | I/O Address25 |  |  | OUTB | output byte |
| 73 | Ra5 | Rt5 | I/O Address25 |  |  | OUTC | output character |
| 74 | Ra5 | Rt5 | I/O Address25 |  |  | OUTH | output half-word |
| 75 | Ra5 | Rt5 | I/O Address25 |  |  | OUTW | output word |
| 76 |  |  |  |  |  |  |  |
| 77 |  |  |  |  |  |  |  |
| 78 |  |  |  |  |  |  |  |
| 79 |  |  |  |  |  |  |  |

| Flow Control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | Ra5 | Target12 |  |  |  | Imm18 |  | BLTI | branch if less than |
| 81 | Ra5 | Target12 |  |  |  | Imm18 |  | BGEI | branch if greater or equal |
| 82 | Ra5 | Target12 |  |  |  | Imm18 |  | BLEI | branch if less or equal |
| 83 | Ra5 | Target12 |  |  |  | Imm18 |  | BGTI | branch if greater than |
| 84 | Ra5 | Target12 |  |  |  | Imm18 |  | BLTUI | branch if lower |
| 85 | Ra5 | Target12 |  |  |  | Imm18 |  | BGEUI | branch if higher or same |
| 86 | Ra5 | Target12 |  |  |  | Imm18 |  | BLEUI | branch if lower or same |
| 87 | Ra5 | Target12 |  |  |  | Imm18 |  | BGTUI | branch if higher |
| 88 | Ra5 | Target12 |  |  |  | Imm18 |  | BEQI | branch if equal |
| 89 | Ra5 | Target12 |  |  |  | Imm18 |  | BNEI | branch if not equal |
| 90 | Ra5 | Target12 |  |  |  | Imm18 |  | LEQI | loop if equal |
| 91 | Ra5 | Target12 |  |  |  | Imm18 |  | LNEI | loop if not equal |
| 92 |  |  |  |  |  |  |  |  |  |
| 93 |  |  |  |  |  |  |  |  |  |
| 94 | Ra5 | Rb5 | Op5 | Imm20 |  |  |  | Branch to register |  |
| 94 | Ra5 | Rb5 | 0 | Imm20 |  |  |  | BLTRI | branch if less than |
| 94 | Ra5 | Rb5 | 1 | Imm20 |  |  |  | BGERI | branch if greater or equal |
| 94 | Ra5 | Rb5 | 2 | Imm20 |  |  |  | BLERI | branch if less or equal |
| 94 | Ra5 | Rb5 | 3 | Imm20 |  |  |  | BGTRI | branch if greater than |
| 94 | Ra5 | Rb5 | 4 | Imm20 |  |  |  | BLTURI | branch if lower |
| 94 | Ra5 | Rb5 | 5 | Imm20 |  |  |  | BGEURI | branch if higher or same |
| 94 | Ra5 | Rb5 | 6 | Imm20 |  |  |  | BLEURI | branch if lower or same |
| 94 | Ra5 | Rb5 | 7 | Imm20 |  |  |  | BGTURI | branch if higher |
| 94 | Ra5 | Rb5 | 8 | Imm20 |  |  |  | BEQRI | branch if equal |
| 94 | Ra5 | Rb5 | 9 | Imm20 |  |  |  | BNERI | branch if not equal |
| 95 |  |  |  |  |  |  |  | Branch to register |  |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 0 | BLT | a < b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 1 | BGE | a >= b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 2 | BLE | a<=b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 3 | BGT | a >b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 4 | BLTU | a < b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 5 | BGEU | a >=b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 6 | BLEU | a <=b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 7 | BGTU | a > b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 8 | BEQ | a == b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 9 | BEQ | a == b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 10 | BRA | 1 |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 11 | BRN | 0 |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 12 | BAND | a && b |
| 95 | Ra5 | Rb5 | Target20 |  |  |  | 13 | BOR | a ¦¦ b |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 16 | BLTR | branch if less than |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 17 | BGER | branch if greater or equal |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 18 | BLER | branch if less or equal |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 19 | BGTR | branch if greater than |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 20 | BLTUR | branch if lower |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 21 | BGEUR | branch if higher or same |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 22 | BLEUR | branch if lower or same |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 23 | BGTUR | branch if higher |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 24 | BEQR | branch if equal |
| 95 | Ra5 | Rb5 | Rc5 |  |  |  | 25 | BNER | branch if not equal |

| Relational Compares |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 96 | Ra5 | Rt5 | Imm25 | SLTI | a < b |
| 97 | Ra5 | Rt5 | Imm25 | SGEI | a >= b |
| 98 | Ra5 | Rt5 | Imm25 | SLEI | a <= b |
| 99 | Ra5 | Rt5 | Imm25 | SGTI | a > b |
| 100 | Ra5 | Rt5 | Imm25 | SLTUI | a < b |
| 101 | Ra5 | Rt5 | Imm25 | SGEUI | a >= b |
| 102 | Ra5 | Rt5 | Imm25 | SLEUI | a <= b |
| 103 | Ra5 | Rt5 | Imm25 | SGTUI | a > b |
| 104 | Ra5 | Rt5 | Imm25 | SEQI | a == b |
| 105 | Ra5 | Rt5 | Imm25 | SNEI | a != b |
| 106 |  |  |  |  |  |
| 107 |  |  |  |  |  |

| 108 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 109 | Ra5 | Rt5 |  |  | Funct7 | FPLOO | Floating point |
| 109 | Ra5 | Rt5 |  |  | 13 | I2F | Integer to float |
| 109 | Ra5 | Rt5 |  |  | 14 | F2I | Float to integer |
| 110 | Ra5 | Rt5 |  |  | Funct7 | FPZL | Floating point |
| 110 | Ra5 | Rt5 |  |  | 0 | FABS | Floating point absolute value |
| 110 | Ra5 | Rt5 |  |  | 1 | FNABS | Floating point neg. absolute |
| 110 | Ra5 | Rt5 |  |  | 2 | FNEG | Floating point negate |
| 110 | Ra5 | Rt5 |  |  | 3 | FMOV | Floating point move |
| 110 | Ra5 | Rt5 |  |  | 4 | FSIGN | Floating point get sign |
| 110 | Ra5 | Rt5 |  |  | 5 | FMAN | Floating point get mantissa |
| 110 | Ra5 | Rt5 |  |  | 6 |  |  |
| 110 | Ra5 | Rt5 |  |  | 7 |  |  |
| 110 | Ra5 | Rb5 | Rt5 |  | 8 | FCLT | a < b |
| 110 | Ra5 | Rb5 | Rt5 |  | 9 | FCGE | a >= b |
| 110 | Ra5 | Rb5 | Rt5 |  | 10 | FCLE | a <= b |
| 110 | Ra5 | Rb5 | Rt5 |  | 11 | FCGT | a > b |
| 110 | Ra5 | Rb5 | Rt5 |  | 12 | FCEQ | a == b |
| 110 | Ra5 | Rb5 | Rt5 |  | 13 | FCNE | a <> b |
| 110 | Ra5 | Rb5 | Rt5 |  | 14 | FCUN | Floating point unordered |
| 110 | Ra5 | Rb5 | Rt5 |  | 15 | FCOR | Floating point ordered |

| 111 |  |  |  |  | NOP |  |
| --- | --- | --- | --- | --- | --- | --- |
| 73 | Imm39 |  |  |  | IMM | Immediate prefix |

## Opcode Maps

### Major Instruction Group

Bits 41-35

| Opcode7 | Opcode35 |
| --- | --- |

MSB is down, LSB is across

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | {misc} | {reg} | {float} |  | ADDM # | SUBM # | ADDH # | SUBH # |
| 1 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
|  | ADD # | SUB # | MUL # | DIV # | MULH # | MOD # |  |  |
| 2 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
|  | XOR # | AND # | OR # | MLO | XORM # | ANDM # | ORM # |  |
| 3 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
|  | XORH # | ANDH # | ORH # |  | {bitfield} |  |  |  |
| 4 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
|  | LB | LC | LH | LW | LP | LBZ | LCZ | LHZ |
| 5 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
|  | LSH | LSW | LF | LFD | LFP | LFDP | {cache} | {cacheu} |
| 6 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
|  | SB | SC | SH | SW | SP | SBZ | SCZ | SHZ |
| 7 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
|  | SSH | SSW | SF | SFD | SFP | SFDP |  |  |
| 8 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
|  | INB | INC | INH | INW | INBZ | INCZ | INHZ |  |
| 9 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
|  | OUTB | OUTC | OUTH | OUTW |  |  |  |  |
| 10 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |
|  | BLT # | BLE # | BGT # | BGE # | BLTU # | BLEU # | BGTU # | BGEU # |
| 11 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
|  | BEQ # | BNE # |  |  | BAND # | BNAND # | BOR # | BNOR # |
| 12 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 |
|  | JAL | RET | JMP | CALL |  | NOP |  |  |
| 13 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
|  | CLT # | CLE # | CGT # | CGE # | CLTU # | CLEU # | CGTU # | CGEU # |
| 14 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
|  | CEQ # | CNE # | CMP # | CMPU # |  |  |  |  |
| 15 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
|  |  |  | SETLO | SETMID | SETHI |  |  |  |

### Register Instruction Group

Bits 12-6, Bits 41-35 = 4

| 17 | Ra6 | Rt6 | Rc6 | Opcode11 | Rb6 |
| --- | --- | --- | --- | --- | --- |

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | {misc} |  |  | {bitfield} |  |  |  |  |
| 1 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
|  | ADD | SUB | MUL | DIV | MULH | MOD |  |  |
| 2 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
|  | XOR | AND | OR |  |  |  |  |  |
| 3 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 4 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
|  | LB | LC | LH | LW | LP | LBZ | LCZ | LHZ |
| 5 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
|  | LSH | LSW | LF | LFD | LFP | LFDP | {cache} | {cacheu} |
| 6 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
|  | SB | SC | SH | SW | SP |  |  |  |
| 7 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
|  | SSH | SSW | SF | SFD | SFP | SFDP |  |  |
| 8 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
|  | INB | INC | INH | INW | INBZ | INCZ | INHZ |  |
| 9 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
|  | OUTB | OUTC | OUTH | OUTW |  |  |  |  |
| 10 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |
|  | BLT | BLE | BGT | BGE | BLTU | BLEU | BGTU | BGEU |
| 11 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
|  | BEQ | BNE |  |  | BAND | BNAND | BOR | BNOR |
| 12 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 |
|  | JAL | RET |  |  |  | NOP |  |  |
| 13 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
|  | CLT | CLE | CGT | CGE | CLTU | CLEU | CGTU | CGEU |
| 14 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
|  | CEQ | CNE | CMP | CMPU |  |  |  |  |
| 15 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |
|  |  |  | SETLO | SETMID | SETHI |  |  |  |

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 |
| 17 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 |
| 18 | 144 | 145 | 146 | 147 | 148 | 149 | 150 | 151 |
|  | XNOR | NAND | NOR |  |  |  |  |  |
| 19 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 |
| 20 | 160 | 161 | 162 | 163 | 164 | 165 | 166 | 167 |
| 21 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 |
| 22 | 176 | 177 | 178 | 179 | 180 | 181 | 182 | 183 |
| 23 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 |
| 24 | 192 | 193 | 194 | 195 | 196 | 197 | 198 | 199 |
|  | SHL # | ROL # | SHR # | ASR # |  |  | ROR # |  |
| 25 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 |
|  | SHL | ROL | SHR | ASR |  |  | ROR |  |
| 26 | 208 | 209 | 210 | 211 | 212 | 213 | 214 | 215 |
|  | SRAND | GRS | GRINT | GRAF |  |  |  |  |
| 27 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 |
|  | CPOP | CLZ | CLO |  |  |  |  |  |
| 28 | 224 | 225 | 226 | 227 | 228 | 229 | 230 | 231 |
| 29 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| 30 | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 |
| 31 | 248 | 249 | 250 | 251 | 252 | 253 | 254 | 255 |

Miscellaneous Control

Bits 12-6, Bits 31-25 = 1

Bits 41-35

| Opcode7 | Opcode35 |
| --- | --- |

Bits 12-6, Bits 41-35 = 4

| 07 | Ra6 | Rt6 | Rc6 | Opcode11 | Rb6 |
| --- | --- | --- | --- | --- | --- |

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  |  |  | DI | EI | RI |  | FLUSH |  |
| 1 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
|  | STOP |  | END |  | WAI |  |  |  |
| 2 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
|  | END | sim1 | sim2 | sim3 | sim4 | sim5 | sim6 | sim7 |
| 3 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
|  | swe0 | swe1 | swe2 | swe3 | swe4 | swe5 | swe6 | swe7 |
| 4 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| 5 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| 6 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
| 7 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
|  |  |  |  |  |  |  | TRS | TSR |
| 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |

### Floating Point Operations

Bits 12-6, Bits 41-35 = 4

| 27 | Ra6 | Rt6 | Rc6 | Opcode11 | Rb6 |
| --- | --- | --- | --- | --- | --- |

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
|  | FADD | FSUB | FMUL | FDIV |  |  |  |  |
| 1 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
|  | FABS | FNABS | FNEG | FSIGN | FMOV | FMAN | FNOP |  |
| 2 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
|  | F2INT | INT2F |  |  |  |  |  |  |
| 3 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
|  | FCLT | FCGE | FCLE | FCGT | FCEQ | FCNE | FCOR | FCNO |
| 4 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| 5 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| 6 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 |
| 7 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
|  | FSTAT |  |  | FRM | FTX | FCX | FEX | FDX |
| 8 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
|  | FADDD | FSUBD | FMULD | FDIVD |  |  |  |  |
| 9 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
|  | FABSD | FNABSD | FNEGD | FSIGND | FMOVD | FMAND | FNOPD |  |
| 10 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 |
|  | F2INTD | INT2FD |  |  |  |  |  |  |
| 11 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 |
|  | FCLTD | FCGED | FCLED | FCGTD | FCEQD | FCNED | FCORD | FCNOD |
| 12 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 |
| 13 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 |
| 14 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| 15 | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 |

# Instruction Set Description

ADD

AND

ASR

BAND

BEQ

BFCHG

BFCLR

BFEXT

BFINS

BGE

BGT

BLE

BLT

BNAND

BNE

BNOR

BOR

BRK

BXNOR

BXOR

CALL

CEQ

CGE

CGT

CLE

CLT

CNE

DI

DIV

EI

END

F2INT

FABS

FADD

FCEQ

FCGE

LB

LBU

LC

LCU

LH

LHU

LW

LWC

LWU

MLO

ROL

ROR

SB

SBU

SC

SCU

SGN

SH

SHL

SHR

SHU

SUB

SW

SWU

TRS

TSR

XNOR

XOR

ADD Rt,Ra,Rb

ADDI Rt,Ra,#n

Immediate Format:

| 41 35 | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- |
| 47 | Ra5 | Rt5 | immediate25 |

Register Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 7 | 6 0 |
| --- | --- | --- | --- | --- | --- |
| 27 | Ra5 | Rb5 | Rt5 | ~13 | 27 |

Synopsis

'Add' register with register or immediate. If the result overflows an overflow exception is triggered.

Detail

Rt = Ra + Rb

Rt = Ra + #imm

ADDU Rt,Ra,Rb

ADDUI Rt,Ra,#n

Immediate Format:

| 41 35 | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- |
| 57 | Ra5 | Rt5 | immediate25 |

Register Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 7 | 6 0 |
| --- | --- | --- | --- | --- | --- |
| 27 | Ra5 | Rb5 | Rb5 | ~13 | 37 |

Synopsis

'Add' register with register or immediate. No exceptions will occur.

Detail

Rt = Ra + Rb

Rt = Ra + #imm

AND Rt,Ra,Rb

ANDI Rt,Ra,#n

Synopsis

Logically 'and' register with register or immediate.

Immediate Format:

| 41 35 | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- |
| 107 | Ra5 | Rt5 | immediate25 |

Register Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 7 | 6 0 |
| --- | --- | --- | --- | --- | --- |
| 27 | Ra5 | Rb5 | Rb5 | ~13 | 87 |

Detail

Rt = Ra & Rb

Rt = Ra & #imm

ASR Rt,Ra,#cnt

ASR Rt,Ra,Rb

Synopsis

Arithmetically shift register right.

Detail

Rd = Rd ^>> Rb

The sign bit of the register is preserved during the shift.

Rt = Ra ^>> #cnt

BccI Ra,#,label

Bcc Ra,Rb,label

BccI Ra,#,Rb

Bcc Ra,Rb,Rc

Synopsis

Branch to label or register if condition is true.

Detail

if register – register compare

if Ra cond Rb is true then

if register form

pc = Rc

else

pc = pc + displacement

else

if Ra cond #n is true then

if register form

pc = Rb

else

pc = pc + displacement

Branch Conditions

| Mnemonic | Description | Conditional Test |
| --- | --- | --- |
| BLT | Branch if Less Than | a < b |
| BGE | Branch if Greater than or Equal | a >= b |
| BLE | Branch if Less than or Equal | a <= b |
| BGT | Branch if Greater Than | a > b |
| BLTU | Branch if Less Than | a < b |
| BGEU | Branch if Greater than or Equal | a >= b |
| BLEU | Branch if Less than or Equal | a <= b |
| BGTU | Branch if Greater Than | a > b |
| BNE / BXOR | Branch if Not Equal | a <> b |
| BEQ / BXNOR | Branch if Equal | a == b |
| BAND | Branch if both true | a and b |
| BOR | Branch if either true | a or b |

BFCHG Rt,Ra,#bfb,#bfe

Synopsis

Insert into bitfield..

Detail

Rt[bfb:fbe] = Ra

Flip the bits between BFB (bit field begin) and BFE (bit field end) of Ra and transfer the result to Rt. (Ra is not altered).

BFCLR Rt,#bfb,#bfe

Synopsis

Clear bitfield..

Detail

Rt[bfb:fbe] = 0

Clear the bits between BFB (bit field begin) and BFE (bit field end) inclusive of Rt.

This is an extended mnemonic for the BFINS instruction.

BFEXT Rt,Ra,#bfb,#bfe

Synopsis

Extract bitfield..

Detail

Rt = Ra[bfb:fbe]

Extract the bits between BFB (bit field begin) and BFE (bit field end) inclusive and transfer to the low order bits of Rt. The result in Rt is zero extended.

BFINS Rt,Ra,#bfb,#bfe

Synopsis

Insert into bitfield..

Detail

Rt[bfb:fbe] = Ra

Insert the low order bits from Ra into the bits between BFB (bit field begin) and BFE (bit field end) inclusive of Rt.

BRK

Synopsis

Call break exception handler.

Detail

XRA = pc; pc = mem[FFFF_F000]; status.backup = status; status.im = 1

This instruction causes the processor to perform a software initiated 'break' interrupt routine. It is purposely defined as a zero word so the processor will execute the break routine in the event that code flows into a region of memory that has been nulled out. This helps promote reliable system operation.

BSR label

Synopsis

Branch to subroutine.

Detail

R31 = pc;

pc[31:8] = pc[31:8] + {target[25:6]};

pc[7:0] = {target[5:0],00}

Call subroutine using program counter page relative offset address form. The twenty-six bit displacement is shifted left twice and sign extended to 32 bits before being used. Note the branch is made in a similar fashion to conditional branches. The low order six bits of the target are copied directly to the pc, while the upper twenty bits represent a page relative address.

CALL d[Ra]

CALL label

Synopsis

Call subroutine

Detail

R47 = pc; pc = disp + Ra;

Call subroutine using register indirect with displacement mode. This is an alternate form of the JAL instruction.

R47 = pc; pc = pc + displacement;

Call subroutine quick form. The twenty-five bit address field is shifted left twice and added to the program counter to form the target address. This allows an relative address call to a method within the same 64MB region of memory.

S<cond> Rt,Ra,Rb

S<cond> Rt,Ra,#n

Synopsis

Compare register with register or immediate.

Detail

Rt = Ra cond Rb

Rt = Ra cond #imm

This instruction performs a comparison of two values and stores the result (1 if condition is true, 0 otherwise) in the target register.

Compare Conditions

CMP Rt,Ra,Rb

CMPI Rt,Ra,#n

CMPU Rt,Ra,Rb

CMPUI Rt,Ra,#n

Synopsis

CMP: 'Compare' register with register or immediate.

CMPU: Unsigned compare of register with register or immediate

Detail

Rt = Ra cond Rb

Rt = Ra cond #imm

This instruction performs a comparison of two values and stores the result in the target register. If the first value is less than the second one, -1 is written to the target register. If the two values are equal, zero is written to the target register. If the first value is greater than the second one +1 is written to the target register. This instruction may also be used to obtain the sign of a value by comparing the value to zero.

CP1 #imm

CP2 #imm

Synopsis

Constant prefix

The upper bits of the immediate constant are set for the next instruction, overriding sign extension of the immediate.

The constant prefix instruction (CP1) indicates the presence of an additional twenty-eight constant bits in the instruction stream intended to represent significant bits 9 to 36 for the immediate constant of next instruction. These bits will take precedence over sign extension of the constant for the next instruction. CP1 sign extends from bit 37 to 64 bits.

The constant prefix instruction (CP2) adds twenty-seven more bits to the immediate constant of the following instruction, overriding any sign extension and substituting for bits 57 to 63.

Constant prefix instructions do not require the presence of other constant prefix instructions. For example CP2 may be used without CP1. CP1 may be used without CP2.

When multiple prefix are present the constant prefix instructions should be issued in the order CP1 then CP2 to ensure a properly built constant.

Interrupts are prevented from occurring between this instruction and the next instruction. Normally this instruction is automatically inserted by the assembler wherever an extended constant value is required.

DI

Synopsis

Disable interrupts.

Detail

status.im = 1

This instruction disables maskable hardware interrupts by setting the interrupt mask in the processor control register. It also disables the IRQ instruction.

DIV Rt,Ra,Rb

DIVI Rt,Ra,#n

Synopsis

Divide register by register or immediate.

Detail

Rt = Ra / Rb

Rt = Ra / #imm

This is a signed division operation. Division by zero will cause an exception.

EI

Synopsis

Enable interrupts.

Detail

status.im = 0

This instruction enables maskable hardware interrupts by clearing the interrupt mask in the status register. Enabling interrupts occurs after a minimum of one clock cycle delay.

END

Synopsis

No operation.

Detail

<none>

This instruction is provided for use in a software emulator of the processor. It indicates the end of a sequence of instructions to emulate. The processor will treat this instruction as a NOP instruction. This instruction is not meant to be used by actual hardware.

F2INT Rt,Ra

Synopsis

Floating point convert single precision to integer. Register to register.

Detail

Rt = int(Ra)

FABS Rt,Ra

Synopsis

Take floating point absolute value of register.

Detail

Rt = Ra;

Rt.sign = 0;

This instruction simply clears the sign bit of register.

FADD Rt,Ra,Rb

Synopsis

Floating point addition. Add register with register.

Detail

Rt = Ra + Rb

FCLT Rt,Ra,Rb

FCLE Rt,Ra,Rb

FCGT Rt,Ra,Rb

FCGE Rt,Ra,Rb

FCEQ Rt,Ra,Rb

FCNE Rt,Ra,Rb

FCOR Rt,Ra,Rb

Synopsis

Floating point single precision comparison, register with register.

Detail

Rt = Ra cond Rb

This instruction performs a comparison of two values and stores a one (true) or zero (false) to the target register depending on the whether the tested condition is true or false.

Compare Conditions

FDIV Rt,Ra,Rb

Synopsis

Floating point division. Divide register by register.

Detail

Rt = Ra / Rb

This instruction may cause a division by zero exception.

FIP

Synopsis

Flush instruction pipeline.

Detail

flags.im = 0

This instruction flushes the processor’s instruction pipeline of all instructions following the FIP instruction.

Flags Affected

| I |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | - | - | - | - | - | - | - |

FMUL Rt,Ra,Rb

Synopsis

Floating point multiplication. Multiply register by register.

Detail

Rt = Ra * Rb

FNABS Rt,Ra

Synopsis

Take floating point negative absolute value of register.

Detail

Rt = Ra

Rt[31] = 1

This instruction simply sets the sign bit of register.

FNEG Rt,Ra

Synopsis

Negate floating point value in register.

Detail

Rt = Ra

Rt[31] = ~Ra[31]

This instruction simply complements the sign bit of register.

FSUB Rt,Ra,Rb

Synopsis

Floating point subtraction. Subtract register from register.

Detail

Rt = Ra - Rb

JAL Rt,d[Ra]

Synopsis

Jump and link to subroutine

Detail

Rt = inc(pc); pc = disp + Ra;

Jump to subroutine using register indirect with displacement mode. The current value of the program counter is stored in the destination register. Normally this instruction will be used with a constant prefix.

JMP address

Synopsis

Jump to address

Detail

pc = {pc[63:37],address,00};

Jump to address quick form. The thirty-five bit address field is shifted left twice and concatenated to the upper 27 bits of the program counter to form the target address. This instruction may be extended via the constant prefix instruction so that a 64-bit address may be formed.

Format:

| 41 35 | 34 0 |
| --- | --- |
| 257 | Address37..2 |

64-bit target address format:

| 4139 | 38 27 |  | 26 0 |
| --- | --- | --- | --- |
| 73 |  |  | Address63..38 |
| 257 |  | Address37..2 |  |

LB Rt,d[Ra]

LB Rt,[Ra + Rb]

Synopsis

Load register byte from memory

Detail

Rt = Memory8 [Ra + disp]

Rt = Memory8 [Ra + Rb]

The byte loaded from memory is sign extended to the register width.

LBU Rt,d[Ra]

LBU Rt,[Ra+Rb]

Synopsis

Load register byte from memory

Detail

Rt = Memory8 [Ra + disp]

Rt = Memory8 [Ra + Rb]

The byte loaded from memory is zero extended to the register width.

LEA Rt,d[Ra]

LEA Rt,d[Ra+Rb]

Synopsis

Load effective address.

Detail

Rt = Ra + Rb

Rt = Ra + #imm

The effective address is loaded into the target register. This instruction is an alternate form of the ADD instruction.

LF Rt,d[Ra]

LF Rt,d[Ra+Rb]

Synopsis

Load floating point register word from memory

Detail

Rt = funpack(memory32[Ra + disp])

Rt = funpack(memory32 [Ra + Rb])

Load floating point value from memory and unpack the value into register Rt.

LH Rt,d[Ra]

LH Rt,d[Ra+Rb]

Synopsis

Load register half-word from memory

Detail

Rt = Memory16 [Ra + disp]

Note the format of the displacement field. D0 is forced to zero because half-word accesses must be half-word aligned.

Rt = Memory16 [Ra + Rb]

The half-word loaded from memory is sign extended to the register width.

LHU Rt,d25[Ra]

LHU Rt,d14[Ra+Rb]

Synopsis

Load register half-word from memory

Detail

Rt = Memory32 [Ra + disp]

Indirect Format:

| 41 35 | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- |
| 397 | Ra5 | Rt5 | Offset25 |

Prefixed Indirect Format:

| 4139 35 |  | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- | --- |
| 73 | Offset63..25 |  |  |  |
| 397 |  | Ra5 | Rt5 | Offset24..0 |

Note the format of the offset field. D1..0 is forced to zero because half-word accesses must be half-word aligned.

The half-word loaded from memory is zero extended to the register width.

Rt = Memory32 [Ra + Rb + disp]

Indexed Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 6 | 5 0 |
| --- | --- | --- | --- | --- | --- |
| 537 | Ra5 | Rb5 | Rt5 | Offset14 | 76 |

If register zero is specified as the target register, then the instruction is treated as a prefetch load instruction. No errors will occur under any circumstances.

LW Rt,d25[Ra]

LW Rt,d14[Ra+Rb]

Synopsis

Load register word from memory

Detail

Rt = Memory64 [Ra + disp]

Indirect Format:

| 41 35 | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- |
| 357 | Ra5 | Rt5 | Offset25 |

Prefixed Indirect Format:

| 4139 35 |  | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- | --- |
| 73 | Offset63..25 |  |  |  |
| 357 |  | Ra5 | Rt5 | Offset24..0 |

Note the format of the displacement field. D2..0 is forced to zero because word accesses must be word aligned.

Rt = Memory64 [Ra + Rb]

Indexed Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 6 | 5 0 |
| --- | --- | --- | --- | --- | --- |
| 537 | Ra5 | Rb5 | Rt5 | Offset14 | 36 |

If register zero is specified as the target register, then the instruction is treated as a prefetch load instruction. No errors will occur under any circumstances.

MLO Rt,Ra,Rb,Rc

Synopsis

Mystery logical operation register with register.

Detail

Rt = Ra <Rc> Rb

Perform a logical operation between Ra and Rb and place result in target register Rt. The operation to be performed is determined by the low order three bits of register Rc. This instruction is used to avoid switch case statements (branches) where the logical operation to be performed may be determined at run time. There is no immediate form for this instruction.

Opcode:

| register value | operation |
| --- | --- |
| 0 | exclusive or |
| 1 | and |
| 2 | inclusive or |
| 3 | {reserved} |
| 4 | exclusive nor |
| 5 | nand |
| 6 | nor |
| 7 | {reserved} |

MOD Rt,Ra,Rb

Synopsis

Get remainder from divide of register by register or immediate.

Detail

Rt = Ra % Rb

Rt = Ra % #imm

This is a signed division operation. The remainder of the division is placed in the target register Rt.

MUL Rt,Ra,Rb

MUL Rt,Ra,#n

Synopsis

Multiply register with register or immediate.

Detail

Rt = Ra * Rb

Rt = Ra * #imm

This is a signed multiply operation. The low order 64 bits of the result are placed in the target register.

MULH Rt,Ra,Rb

MULH Rt,Ra,#n

Synopsis

Multiply register with register or immediate; return high order bits.

Detail

Rt = Ra * Rb

Rt = Ra * #imm

This is a signed multiply operation. The high order 32 bits of the result are placed in the target register.

NAND Rt,Ra,Rb

Synopsis

Logically ''nand' register with register.

Detail

Rt = ~(Ra & Rb)

There is no immediate form for this instruction.

NMI

Synopsis

Run nmi routine.

Detail

R30 = pc; pc = FFFF_FFF8; flags.backup = flags; flags.im = 1

This instruction causes the processor to perform a software initiated non-maskable interrupt routine. It has the same effect as an external hardware non-maskable interrupt.

Flags Affected

| I |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - | - | - | - | - | - | - |

NOP

Synopsis

No operation

Detail

This instruction acts merely as a placeholder. It performs no operation and has no effect on the processor. Many processors lack an explicit NOP operation resulting in different instructions being used for this purpose within the same processor. By providing an explicit NOP instruction some consistency in programs can be achieved. Without a NOP instruction many processors typically use instructions which have side effects in the form of affecting the processor status flags; this is not the case here.

NOR Rt,Ra,Rb

Synopsis

Logically inclusively 'nor' register with register.

Detail

Rt = ~(Ra | Rb)

There is no immediate form for this instruction.

NOT Rt,Ra

Synopsis

Take ones complement of register.

Detail

Rt = ~Ra

There is no immediate form for this instruction.

OR Rt,Ra,Rb

ORI Rt,Ra,#n

Synopsis

Logically inclusively 'or' register with register or immediate.

Detail

Rt = Ra | Rb

Rt = Ra | #imm

RESET

Synopsis

Run reset routine.

Detail

EPC = pc; pc = FFFF_FFFC; flags.backup = flags; flags.im = 1

This instruction causes the processor to perform a software initiated reset routine.

Flags Affected

| I |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | - | - | - | - | - | - | - |

RET

Synopsis

Return from subroutine.

Detail

pc = Rb

Rt = Ra + imm

| 27 | Ra5 | Rb5 | Rt5 | Imm20 | RET | return and add |
| --- | --- | --- | --- | --- | --- | --- |

This instruction returns to the calling routine by loading the program counter with the contents of the register Rb. Normally Rb will be R31 which is automatically updated by the call instruction.

This instruction also performs and addition, allowing a stack frame to be updated.

RI

Synopsis

Restore interrupt flag from backup copy.

Detail

flags.im = flags.backup im

This instruction restores the previous interrupt flag state from the backup copy of the processor control register. This allows restoring the interrupt state that was present when the backup of the status register was made. This is useful in operating system code where interrupts must be disabled to perform certain system functions (like updating system lists) and then restored to a previous state after performing the operation.

Flags Affected

| I |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X | - | - | - | - | - | - | - |

ROLI Rt,Ra,#cnt

ROL Rt,Ra,Rb

Synopsis

Rotate register left.

Detail

Rd = Rd !<< Rb

The most significant bit is shifted into the least significant bit.

Rt = Ra !<< #cnt

RORI Rt,Ra,#cnt

ROR Rt,Ra,Rb

Synopsis

Rotate register right.

Detail

Rd = Rd !>> Rb

The most significant bit is shifted into the least significant bit.

Rt = Ra !>> #cnt

ERET

Synopsis

Return from exception subroutine.

Detail

pc = R30

This instruction returns from an exception processing routine by restoring the flag register and jumping back to the code that was interrupted (who's address is stored in EPC).

SB Rs,d[Ra]

SB Rs,d[Ra + Rb]

Synopsis

Store byte from register to memory.

Detail

Memory8 [Ra + disp] = Rs

Memory8 [Ra + Rb + disp] = Rs

SETHI

S-Type

Usage: setting register to constant value

| 41 38 | 35 25 | 25 20 | 19 0 |
| --- | --- | --- | --- |
| 154 | immediate12 | RegT6 | immediate20 |

The SETHI instruction sets the high order 32 bits of the target register to the immediate value specified in the opcode. The low order 32 bits of the register are unaffected. This instruction is typically used after the SETLO instruction in order to form a 64 bit constant.

SETLO

S-Type  SETHI, SETLO

Usage: setting register to constant value

| 41 37 | 36 25 | 24 20 | 19 0 |
| --- | --- | --- | --- |
| 145 | immediate12 | RegT5 | immediate20 |

The SETLO instruction sets the target register to the immediate value specified in the opcode. The low order 32 bits of the register are set as specified in the opcode, the high order 32 bits of the register are set to the sign extension of the low order bits.

SF Rt,d[Ra]

SF Rt,[Ra+Rb]

Synopsis

Store floating point register word to memory

Detail

memory32[Ra + disp] = fpack(Rs)

| desc: | op | Ra | Rs | disp. |
| --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 16 |
| bits: | 31 26 | 25 21 | 20 16 | 15 0 |
| bit pattern: | 28 | R4..0 | R4..0 | d15..0 |

memory32[Ra + Rb] = fpack(Rs)

| desc: | op | Ra | Rs | Rb | X | op |
| --- | --- | --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 5 | 5 | 6 |
| bits: | 31 26 | 27 24 | 23 20 | 15 11 | 10 6 | 5 0 |
| bit pattern: | 60 | R4..0 | R4..0 | R4..0 | ~ | 28 |

Store packed floating point single value from register Rs to memory.

SGN Rt,Ra

Synopsis

Obtain sign of number.

Detail

Rt = Ra cond 0

This instruction performs a comparison of a register to zero and stores the result in the target register. If the value is less than zero, -1 is written to the target register. If the value is equal to zero, zero is written to the target register. If the value is greater than zero, +1 is written to the target register. This instruction is an alternate form of the CMP instruction, where the register is compared to the value zero..

SH Rt,d[Ra]

SH Rt,[Ra+Rb]

Synopsis

Store half-word from register to memory

Detail

Memory16 [Ra + disp] = Rs[15:0]

| desc: | op | Ra | Rs | disp |
| --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 16 |
| bits: | 31 26 | 25 21 | 20 16 | 15 0 |
| bit pattern: | 54 | R4..0 | R4..0 | d15..0 |

Memory16 [Ra + Rb] = Rs[15:0]

| desc: | Op | Ra | Rs | ~ | Op | Rb |
| --- | --- | --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 5 | 6 | 5 |
| bits: | 31 26 | 25 21 | 20 16 | 15 11 | 10 5 | 4 0 |
| bit pattern: | 2 | R4..0 | R4..0 | ~ | 54 | R4..0 |

Note the format of the displacement field. D0 is forced to zero because half-word accesses must be half-word aligned.

SHL Rt,Ra,#cnt

SHL Rt,Ra,Rb

Synopsis

(Logically) shift register left.

Detail

Rd = Rd << Rb

Zeroes are shifted into the least significant bits.

| desc: | op | Ra | Rt | X | op | Rb |
| --- | --- | --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 5 | 6 | 5 |
| bits: | 31 26 | 25 21 | 20 16 | 15 11 | 10 5 | 4 0 |
| bit pattern: | 12 | R4..0 | R4..0 | ~ | 8 | R4..0 |

Rt = Ra << #cnt

| desc: | op | Ra | Rt | X | op | cnt |
| --- | --- | --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 5 | 6 | 5 |
| bits: | 31 26 | 25 21 | 20 16 | 15 11 | 10 5 | 4 0 |
| bit pattern: | 12 | R4..0 | R4..0 | ~ | 0 | N4..0 |

SHR Rt,Ra,#cnt

SHR Rt,Ra,Rb

Synopsis

(Logically) shift register right.

Detail

Rd = Rd >> Rb

A zero is shifted into the most significant bit.

| desc: | op | Ra | Rt | X | op | Rb |
| --- | --- | --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 5 | 6 | 5 |
| bits: | 31 26 | 25 21 | 20 16 | 15 11 | 10 5 | 4 0 |
| bit pattern: | 12 | R4..0 | R4..0 | ~ | 11 | R4..0 |

Rt = Ra >> #cnt

| desc: | op | Ra | Rt | X | op | cnt |
| --- | --- | --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 5 | 6 | 5 |
| bits: | 31 26 | 25 21 | 20 16 | 15 11 | 10 5 | 4 0 |
| bit pattern: | 12 | R4..0 | R4..0 | ~ | 3 | N4..0 |

STOP

Synopsis

Stop the processor from executing instructions and wait for the external hardware 'go' signal or a non-maskable interrupt.

Detail

| desc: | op |
| --- | --- |
| size: | 32 |
| bits: | 31 0 |
| bit pattern: | 0000_0008 |

This instruction can be used to synchronize the processor to an external event.

SUB Rt,Ra,Rb

SUBI Rt,Ra,#n

Synopsis

'Subtract' register with register or immediate.

Detail

Rt = Ra - Rb

| desc: | op | Ra | Rt | ~ | op | Rb |
| --- | --- | --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 5 | 6 | 5 |
| bits: | 31 26 | 25 21 | 20 16 | 15 11 | 10 5 | 4 0 |
| bit pattern: | 2 | R4..0 | R4..0 | ~ | 5 | R4..0 |

Rt = Ra - #imm

| desc: | op | Ra | Rt | imm |
| --- | --- | --- | --- | --- |
| size: | 6 | 5 | 5 | 16 |
| bits: | 31 26 | 25 21 | 20 16 | 15 0 |
| bit pattern: | 5 | R4..0 | R4..0 | n15..0 |

SW Rs,d25[Ra]

SW Rs,d14[Ra + Rb]

Synopsis

Store word from register to memory

Indirect Format:

| 41 35 | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- |
| 517 | Ra5 | Rs5 | disp25 |

Indexed Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 6 | 5 0 |
| --- | --- | --- | --- | --- | --- |
| 537 | Ra5 | Rb5 | Rs5 | disp14 | 196 |

Detail

Memory32 [Ra + disp25] = Rs

Note the format of the displacement field. D2..0 is forced to zero because word accesses must be word aligned.

Memory32 [Ra + Rb + disp14] = Rs

SYS

Synopsis

Perform system call.

Detail

R62 = pc; pc = <ETA>+nnn; pcr.backup = pcr; pcr.im = 1

This instruction causes the processor to perform a system call. It is meant for operating system support. It is a specialized form of the JAL instruction.

Effects:

The processor control register is copied to it’s backup version. Then the interrupt mask is set, disabling further interrupts.

The program counter is stored in register 62.

The program counter is loaded with the sum of the exception table address register and the call number shifted left twice.

I - Type

Register-Immediate Datapath,

Jump and Link

| 31 25 | 24 19 | 18 13 | 12 10 | 9 0 |
| --- | --- | --- | --- | --- |
| 27 | Ra6 | 62 | 0 | Call No10 |

TRS Ra,Spr

Synopsis

Transfer register to special purpose register.

Detail

Special Register = Ra

Special Registers

Currently there are only five special registers defined, the remaining codes are reserved for future use.

| Code | Register Name | Description |
| --- | --- | --- |
| 0 | PCR | Processor Control Register |
| 1 |  | {this register is reserved} |
| 2 |  | {this register is reserved} |
| 3 | VER | Processor Version - major, minor, revision |
| 4 | RND | random number register |

TSR Rt,Spr

Synopsis

Transfer special purpose register to register.

Detail

Rt = Special Register

Special Registers

Currently there are only five special registers defined, the remaining codes are reserved for future use.

| Code | Register Name | Description |
| --- | --- | --- |
| 0 | PCR | Processor Control Register |
| 1 |  | {this register is reserved} |
| 2 |  | {this register is reserved} |
| 3 | VER | Processor Version - major, minor, revision |
| 4 | RND | random number register |
| 5 | TC | Tick Count |

WAI

Synopsis

Wait for interrupt.

Register Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 7 | 6 0 |
| --- | --- | --- | --- | --- | --- |
| 07 | 05 | 05 | 05 | ~13 | 407 |

Detail

If the processor is in the interrupt context, this instruction will cause the processor to exit the interrupt context; otherwise this instruction will be treated as a nop. When an interrupt occurs, the processor will begin execution immediately after the wai instruction, if the wai instruction was used to exit the interrupt context.

Normally this instruction would be used in an interrupt service routine at the point at which it is desired to begin interrupt processing.

XNOR Rt,Ra,Rb

Synopsis

Logically exclusively 'nor' register with register.

Register Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 7 | 6 0 |
| --- | --- | --- | --- | --- | --- |
| 27 | Ra5 | Rb5 | Rt5 | ~13 | 147 |

Detail

Rt = ~(Ra ^ Rb)

There is no immediate form for this instruction.

XOR Rt,Ra,Rb

XORI Rt,Ra,#n

Synopsis

Logically exclusively 'or' register with register or immediate.

Immediate Format:

| 41 35 | 34 30 | 29 25 | 24 0 |
| --- | --- | --- | --- |
| 127 | Ra5 | Rt5 | immediate25 |

Register Format:

| 41 35 | 34 30 | 29 25 | 24 20 | 19 7 | 6 0 |
| --- | --- | --- | --- | --- | --- |
| 27 | Ra5 | Rb5 | Rt5 | ~13 | 107 |

Detail

Rt = Ra ^ Rb

Rt = Ra ^ #imm