Table of Contents

Programming Model	7

Configurations	7

Instruction Prefixes	7

FAR	7

OUTER	8

Additional Instructions	9

JMP FAR	9

JSR FAR	9

RTF	9

Differences from the 6809	9

Control Registers	10

Checkpoint Register	10

Hardware:	11

Push / Pull Post-byte	11

6309 Instructions Not supported	11

Size	12

Instruction Set Description	12

The Index Post-byte	13

ABX – Add B Accumulator to X	14

ADCA – Add with Carry to Accumulator A	15

ADCB – Add with Carry to Accumulator B	16

ADCD – Add with Carry to Accumulator D	17

ADDA – Add to Accumulator A	20

ADDB – Add to Accumulator B	21

ADDD – Add to Accumulator D	22

ANDA – Bitwise ‘And’ to Accumulator A	28

ANDB – Bitwise ‘And’ to Accumulator B	29

ANDCC – Bitwise ‘And’ to Condition Code Reg	30

ANDD – Bitwise ‘And’ to Accumulator D	31

ASL – Arithmetic Shift Left Memory	33

ASLA – Arithmetic Shift Left Accumulator A	34

ASLB – Arithmetic Shift Left Accumulator B	35

ASLD – Arithmetic Shift Left Accumulator D	36

ASR – Arithmetic Shift Right Memory	37

ASRA – Arithmetic Shift Right Accumulator A	38

ASRB – Arithmetic Shift Right Accumulator B	39

ASRD – Arithmetic Shift Right Accumulator D	40

BCC – Branch if Carry Clear	40

BCS – Branch if Carry Set	41

BEQ – Branch if Equal	41

BGE – Branch if Greater or Equal	42

BGT – Branch if Greater Than	42

BHI – Branch if Higher	43

BHS – Branch if Higher or Same	43

BITA – Bitwise ‘And’ to Accumulator A	44

BITB – Bitwise ‘And’ to Accumulator B	45

BITD – Bitwise ‘And’ to Accumulator D	46

BLE – Branch if Less or Equal	47

BLO – Branch if Lower	49

BLS – Branch if Lower or the Same	49

BLT – Branch if Less Than	50

BMI – Branch if Minus	51

BNE – Branch if Not Equal	52

BPL – Branch if Plus	52

BRA – Branch Always	53

BRN – Branch Never	53

BSR – Branch To Subroutine	54

BVC – Branch if Overflow Clear	54

BVS – Branch if Overflow Set	55

CLR – Clear Memory	56

CLRA – Clear Accumulator A	57

CLRB – Clear Accumulator B	58

CLRD – Clear Accumulator D	59

CMPA – Compare to Accumulator A	63

CMPB – Compare to Accumulator B	64

CMPD – Compare to Accumulator D	65

COM – Complement Memory	74

COMA – Complement Accumulator A	75

COMB – Complement Accumulator B	75

COMD – Complement Accumulator D	76

CWAI – Wait For Interrupt	78

DAA – Decimal Adjust after Addition	78

DEC – Decrement Memory	79

DECA – Decrement Accumulator A	80

DECB – Decrement Accumulator B	81

DECD – Decrement Accumulator D	81

EORA – Bitwise Exclusive ‘Or’ to Accumulator A	82

EORB – Bitwise Exclusive ‘Or’ to Accumulator B	87

EORD – Bitwise Exclusive ‘Or’ to Accumulator D	88

EXG – Exchange Registers	90

INC – Increment Memory	91

INCA – Increment Accumulator A	92

INCB – Increment Accumulator B	93

INCD – Increment Accumulator D	93

JMP – Unconditional Jump	94

JSR –Jump to Subroutine	96

LBCC – Long Branch if Carry Clear	97

LBCS – Long Branch if Carry Set	98

LBEQ – Long Branch if Equal	98

LBGE – Long Branch if Greater or Equal	99

LBGT – Long Branch if Greater Than	99

LBHI – Branch if Higher	100

LBHS – Long Branch if Higher or Same	100

LBLE – Branch if Less or Equal	101

LBLO – Long Branch if Lower	102

LBLS – Long Branch if Lower or the Same	102

LBLT – Long Branch if Less Than	103

LBMI – Long Branch if Minus	104

LBNE – Long Branch if Not Equal	105

LBPL – Long Branch if Plus	105

LBRA – Long Branch Always	106

LBRN – Long Branch Never	106

LBSR – Long Branch To Subroutine	107

LBVC – Long Branch if Overflow Clear	107

LBVS – Long Branch if Overflow Set	108

LDA – Load Accumulator A	109

LDB – Load Accumulator B	110

LDD – Load Accumulator D	111

LEAS – Load Effective Address Into S	112

LEAU – Load Effective Address Into U	120

LEAX – Load Effective Address Into X	120

LEAY – Load Effective Address Into Y	120

LSL – Logical Shift Left Memory	121

LSLA – Logical Shift Left Accumulator A	122

LSLB – Logical Shift Left Accumulator B	123

LSLD – Logical Shift Left Accumulator D	124

LSR – Logical Shift Right Memory	125

LSRA – Logical Shift Right Accumulator A	126

LSRB – Logical Shift Right Accumulator B	127

LSRD – Logical Shift Right Accumulator D	128

MUL – Multiply	128

NEG – Negate Memory	129

NEGA – Negate Accumulator A	130

NEGB – Negate Accumulator B	130

NEGD – Negate Accumulator D	131

NOP – No Operation	131

ORA – Bitwise ‘Or’ to Accumulator A	132

ORB – Bitwise ‘Or’ to Accumulator B	133

ORCC – Bitwise ‘Or’ to Condition Code Reg	134

ORD – Bitwise ‘Or’ to Accumulator D	135

PSHS – Push onto Stack	136

PSHU – Push onto User Stack	137

PULS – Pull from Stack	138

PULU – Pull from User Stack	139

ROL – Rotate Left Memory	140

ROLA – Rotate Left Accumulator A	141

ROLB – Rotate Left Accumulator B	142

ROLD – Rotate Left Accumulator D	143

ROR – Rotate Right Memory	144

RORA – Rotate Right Accumulator A	145

RORB – Rotate Right Accumulator B	146

RORD – Rotate Right Accumulator D	147

RTF – Return From Far Subroutine	147

RTI – Return From Interrupt	149

RTS – Return From Subroutine	150

SBCA – Subtract with Carry from Accumulator A	152

SBCB – Subtract with Carry from Accumulator B	153

SBCD – Subtract with Carry from Accumulator D	154

SEX – Sign Extend	155

STA – Store Accumulator A	156

STB – Store Accumulator B	157

STD – Store Accumulator D	157

STS – Store Stack Pointer	158

STU – Store User Stack Pointer	158

STX – Store X Register	159

STY – Store Y Register	159

SUBA – Subtract from Accumulator A	160

SUBB – Subtract from Accumulator B	161

SUBD – Subtract from Accumulator D	162

SWI – Software Interrupt	163

SWI2 – Software Interrupt	164

SWI3 – Software Interrupt	165

SYNC – Halt and Wait for Interrupt	166

TFR – Transfer Registers	167

TST – Test Memory	168

TSTA – Test Accumulator A	169

TSTB – Test Accumulator B	169

TSTD – Test Accumulator D	170

## Programming Model

| 35 24 | 23 0 |  |
| --- | --- | --- |
|  | X – Index Register |  |
|  | Y – Index Register |  |
|  | U – User stack pointer |  |
| 0000 | S – Hardware stack pointer |  |
| PC |  |  |
|  | A | B |
|  | DPR | 0 |
|  |  | CCR |

A,B registers concatenate to form D register

## Condition Code Register

|  |  | M | D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

C: carry bit for extended precision arithmetic

V: overflow

Z: result zero

N: result negative

I: IRQ interrupt mask

H: half carry

F: FIRQ interrupt mask

E: entire state saved indicator

D: decimal arithmetic mode

M: IRQ mask bit

The decimal flag is automatically cleared at entry of an interrupt subroutine.

## Configurations

The rf6809 core may be configured to use 12-bit bytes which increases the address range to 36-bits. The rf6809 core may also be configured to support many instructions compatible with the 6309 processor.

## Instruction Prefixes

rf6809 makes use of instruction prefixes to extend the addressing modes available. There are two prefixes FAR, and OUTER, which indicate to use a far address or outer indexing.

### FAR

FAR when applied to extended addressing indicates to use a full 24-bit/triple byte address rather than a 16 bit one.

When the FAR prefix is applied to indirect addressing the prefix indicates that the indirect address is 24-bit. This allows the use of a 24-bit indirect address to reach anywhere in memory.

Opcode: 0x15

### OUTER

The OUTER prefix indicates that the index register is applied after retrieving an indirect address. Normally the index register is used in the calculation of the indirect address.

When configured for 12-bit bytes the OUTER prefix is not used as there are sufficient bits in the index post-byte to encode outer indexing mode.

Opcode: 0x1B

## Additional Instructions

JMP FAR – performs a jump using a 24-bit extended address.

Opcode: 0x8F

JSR FAR – performs a jump to subroutine using a 24-bit extended address. The full 24-bit program counter is stored on the stack.

Opcode: 0xCF

RTF – performs a far return from subroutine by loading a full 24-bit program counter from the stack.

Opcode: 0x38

Indirect addresses must reside within the first 64k bank of memory.

ADDx, ADCx, SUBx, SBCx, NEGx, and MUL all support BCD arithmetic if the decimal mode bit is set in the condition code register.

## Differences from the 6809

The program counter is a full 24-bit register. The JMP and JSR instructions modify only the low order 16 bits of the program counter. To modify the full 24-bits use the JMP FAR and JSR FAR instructions. A return from a far subroutine may be done using the RTF instruction.

During interrupt processing the entire 24-bit program counter is stacked. The RTI instruction also loads the entire 24-bit program counter.

If 6309 instructions are enabled then the E, F registers are pushed onto the stack for interrupts except for the FIRQ. The RTI instruction will also reload the E, F registers.

### Control Registers

There are several control registers mapped into the address space.

| Address | Access | Register Usage |
| --- | --- | --- |
| FF..F00/01 | RW | Debug address register #0 |
| FF..F02/03 | RW | Debug address register #1 |
| FF..F04/05 | RW | Debug address register #2 |
| FF..F06/07 | RW | Debug address register #3 |
| FF..F08 | RW | Debug control register #0 (for address register #0) |
| FF..F09 | RW | Debug control register #1 (for address register #1) |
| FF..F0A | RW | Debug control register #2 (for address register #2) |
| FF..F0B | RW | Debug control register #3 (for address register #3) |
| FF..F10 | RO | Core ID – used to identify core in multi-core application. Reflects the value of the coreid_i input. |
| FF..F11 | WO | Checkpoint register. If checkpointing is enabled this register must be written within one second, or an NMI will occur. |
| FF..FE14/15 | RO | high order bits of millisecond count |
| FF.FF16/17 | RO | low order bits of millisecond count |

The millisecond count register contains a count of the number of milliseconds since the last reset.

### Debug Address Registers

This set of register is used to generate debug breakpoint interrupts when an address matches the value in the address register. Address matching must be enabled in the corresponding control register.

### Debug Control Registers

These registers all function identically so only one is described.

| Bits |  |  |
| --- | --- | --- |
| 0 to 3 | Address match mask | bits that are clear in the mask will be treated as an automatic match in the address compare. Only the low order four address bits have this capability. All other address bits must match for an interrupt to be generated. Setting these bits to all ones means all the address bits must match during a compare operation. |
| 4, 5 | Address match type | Sets the type of memory access that must match for an interrupt to be generated. One of BMT_DS (data store), BMT_LS (data load or data store), or BMT_IA (instruction address) |
| 6 | Trace enable | When this bit is set the address match does not generate an interrupt. Instead instruction tracing is triggered and the trace fifo will fill with addresses of executing instructions. |
| 7 | enable | When this bit is set an interrupt will be generated if the address matches the one in the corresponding breakpoint address register and the access type is of the correct type, |
| 8 to 10 | reserved | These bits are currently not used. |
| 11 |  | This status bit is set by hardware to indicate a match occurred for the corresponding breakpoint address register. This bit must be cleared by software. |

### Checkpoint Register

The core may be configured to include a checkpoint register and timer. When checkpointing is present an NMI will be generated is the checkpoint register is not written to within one second.

### Hardware:

This is a softcore implementation of a 6809 compatible processor. As such no attempt was made to duplicate the 6809’s bus cycle activity. Instructions may not execute in the same number of clock cycles as the 6809. Instructions in some circumstances execute in fewer clock cycles.

## Push / Pull Post-byte

When 6309 instructions are enabled, and the core is configured for 12-bit bytes the push and pull instructions may include pushing and pulling of accumulators E and F. The push / pull order is outline below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC | higher memory address |

Push / pull post-byte (12-bit bytes)

| 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ~ | ~ | F | E | PC | U or S | Y | X | DP | B | A | CCR |

## 6309 Instructions Not supported

PSHSW, PULSW, PSHUW, PULUW

TFM

Memory bit manipulation instructions.

The Q register load and store instructions.

# Size

12-bit bytes, 6309 support: 8000 LUTs 5 block rams, 1 DSP

12-bit bytes, no 6309 support 6500 LUTs, 5 block rams, 1 DSP.

Core Features:

Hardware breakpoints can trap on a match of a load, store, or instruction address. They feature a zone within which a match may occur, some of the least significant bits may be masked off during the compare and made don’t cares. It is now possible to set breakpoints in ROM routines. Setting a breakpoint does not modify the code or data at the breakpoint address.

# Instruction Set Description for 12-bit Bytes

## The Index Post-byte

The indexed addressing mode specification field is twelve bits in size.

Bits rr specifies one of the index registers, XR, YR, SP, or UP

Bits ddddddddd specifies a nine-bit displacement.

The ‘i’ bit indicates one level of indirection is added for the data fetch.

The ‘o’ bit indicates that indexing is applied after indirection.

| Ndx Pattern |  |  |  |
| --- | --- | --- | --- |
| 0rrddddddddd | EA = ,R + 9 bit offset |  |  |
| 1rri00000000 | EA = ,R+ |  |  |
| 1rri00000001 | EA = ,R++ |  |  |
| 1rri00000010 | EA = ,-R |  |  |
| 1rri00000011 | EA = ,--R |  |  |
| 1rrio0000100 | EA = ,R + 0 offset |  |  |
| 1rrio0000101 | EA = ,R + ACCB offset |  |  |
| 1rrio0000110 | EA = ,R + ACCA offset |  |  |
| 1rrio0001000 | EA = ,R + 12 bit offset |  |  |
| 1rrio0001001 | EA = ,R + 24 bit offset |  |  |
| 1rrio0001010 | EA = ,R + 36 bit offset |  |  |
| 1rrio0001011 | EA = ,R + D offset |  |  |
| 1rrio0001100 | EA = ,PC + 12 bit offset |  |  |
| 1rrio0001101 | EA = ,PC + 24 bit offset |  |  |
| 1rrio0001110 | EA = ,PC + 36 bit offset |  |  |
| 1rrio0001111 | EA = [,Address] |  |  |
| 1rrio0010101 | EA = ,R + ACCF offset |  |  |
| 1rrio0010110 | EA = ,R + ACCE offset |  |  |
| 1rrio0011011 | EA = ,R + W offset |  |  |

### ABX – Add B Accumulator to X

Description

The B accumulator is added to the X register.

Instruction Format: INH

11 0

03Ah

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### ADCA – Add with Carry to Accumulator A

Description

The source operand is added to accumulator A including a carry. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 089h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 099h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A9h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B9h |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| M | D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### ADCB – Add with Carry to Accumulator B

Description

If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0C9h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D9h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E9h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F9h |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### ADCD – Add with Carry to Accumulator D

Description

The source operand is added to accumulator D including a carry. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

*This instruction is available only if 6309 instruction supported is configured.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 189h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 199h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1A9h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1B9h |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### ADCR – Add with Carry Register to Register

Description

Add register to register with carry.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

| 23 20 | 19 16 | 15 12 | 11 0 |
| --- | --- | --- | --- |
| ~ | r0 | r1 | 131h |

| r0/r1 |  | r0/r1 |  |
| --- | --- | --- | --- |
| 0 | D | 8 | A |
| 1 | X | 9 | B |
| 2 | Y | 10 | CC |
| 3 | U | 11 | DP |
| 4 | S | 12 | 0 |
| 5 | PC | 13 | 0 |
| 6 | W | 14 | E |
| 7 | resv | 15 | F |

Flags Affected:

N set equal to the most significant bit of the result

Z set if result value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit from the most significant bit, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### ADDA – Add to Accumulator A

Description

The source operand is added to accumulator A. The carry is not included in the addition but is generated as a result flag. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 08Bh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 09Bh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0ABh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0BBh |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### ADDB – Add to Accumulator B

Description

The source operand is added to accumulator B. The carry is not included in the addition but is generated as a result flag. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0CBh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0DBh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0EBh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0FBh |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### ADDD – Add to Accumulator D

Description

The source operand is added to accumulator D. The carry is not included in the addition but is generated as a result flag. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 0C3h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D3h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E3h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F3h |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### ADDE – Add to Accumulator E

Description

The source operand is added to accumulator E. The carry is not included in the addition but is generated as a result flag.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 28Bh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 29Bh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2ABh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2BBh |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### ADDF – Add to Accumulator F

Description

The source operand is added to accumulator F. The carry is not included in the addition but is generated as a result flag.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 2CBh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 2DBh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2EBh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2FBh |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### ADDR – Add Register to Register

Description

Add register to register.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

| 23 20 | 19 16 | 15 12 | 11 0 |
| --- | --- | --- | --- |
| ~ | r0 | r1 | 131h |

| r0/r1 |  | r0/r1 |  |
| --- | --- | --- | --- |
| 0 | D | 8 | A |
| 1 | X | 9 | B |
| 2 | Y | 10 | CC |
| 3 | U | 11 | DP |
| 4 | S | 12 | 0 |
| 5 | PC | 13 | 0 |
| 6 | W | 14 | E |
| 7 | resv | 15 | F |

Flags Affected:

N set equal to the most significant bit of the result

Z set if result value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit from the most significant bit, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### ADDW – Add to Accumulator W

Description

The source operand is added to accumulator W. The carry is not included in the addition but is generated as a result flag.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 18Bh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 19Bh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1ABh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1BBh |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### ANDA – Bitwise ‘And’ to Accumulator A

Description

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 084h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 094h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A4h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B4h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### ANDB – Bitwise ‘And’ to Accumulator B

Description

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0C4h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D4h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E4h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F4h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### ANDCC – Bitwise ‘And’ to Condition Code Reg

Description

This instruction can be used to clear bits in the condition code register. A common use is to clear the interrupt mask bits.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 01Ch |

Flags Affected:

Flags for which the immediate constant has a zero bit will be cleared, other flags will not be affected.

|  |  | M | D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### ANDD – Bitwise ‘And’ to Accumulator D

Description

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 184h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 194h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1A4h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1B4h |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### ANDR – Bitwise ‘And’ Register to Register

Description

And register to register.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

| 23 20 | 19 16 | 15 12 | 11 0 |
| --- | --- | --- | --- |
| ~ | r0 | r1 | 134h |

| r0/r1 |  | r0/r1 |  |
| --- | --- | --- | --- |
| 0 | D | 8 | A |
| 1 | X | 9 | B |
| 2 | Y | 10 | CC |
| 3 | U | 11 | DP |
| 4 | S | 12 | 0 |
| 5 | PC | 13 | 0 |
| 6 | W | 14 | E |
| 7 | resv | 15 | F |

Flags Affected:

N set equal to the most significant bit of the result

Z set if result value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### ASL – Arithmetic Shift Left Memory

Description

Memory is read, bits are shifted to the left by one bit, then the result is written back to memory. A zero is shifted into the least significant bit and the most significant bit is captured in the carry result flag.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 008h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 068h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 078h |

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the result

Z set if result value is zero, otherwise cleared

V set to the exclusive or of bits 10 and 11

C set to the original value of bit 11

| E |  | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### ASLA – Arithmetic Shift Left Accumulator A

Description

Bits in the accumulator A are shifted once to the left. A zero is shifted into the least significant bit and the most significant bit is captured in the carry result flag.

Instruction Format: INH

11 0

048h

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 10 and 11

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### ASLB – Arithmetic Shift Left Accumulator B

Description

Bits in the accumulator B are shifted once to the left. A zero is shifted into the least significant bit and the most significant bit is captured in the carry result flag.

Instruction Format: INH

11 0

058h

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 10 and 11

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### ASLD – Arithmetic Shift Left Accumulator D

Description

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

148h

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 22 and 23

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### ASR – Arithmetic Shift Right Memory

Description

Memory is read, bits are shifted to the left by one bit, then the result is written back to memory. A zero is shifted into the least significant bit and the most significant bit is captured in the carry result flag.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 007h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 067h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 077h |

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the result

Z set if result value is zero, otherwise cleared

C set to the original value of bit 0

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ |  | ↕ |

### ASRA – Arithmetic Shift Right Accumulator A

Description

Bits in the accumulator A are shifted once to the right. The sign bit is shifted into the most significant bit and the least significant bit is captured in the carry result flag.

Instruction Format: INH

11 0

047h

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

C set if there is a carry out of bit 0, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ |  | ↕ |

### ASRB – Arithmetic Shift Right Accumulator B

Description

Bits in the accumulator B are shifted once to the right. The sign bit is shifted into the most significant bit and the least significant bit is captured in the carry result flag.

Instruction Format: INH

11 0

057h

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ |  | ↕ |

### ASRD – Arithmetic Shift Right Accumulator D

Description

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

147h

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

C set if there is a carry out of bit 0, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ |  | ↕ |

### BCC – Branch if Carry Clear

Description

BCC performs a PC relative branch using a 12-bit sign extended displacement if the carry flag bit is clear in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 024h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BCS – Branch if Carry Set

Description

BCC performs a PC relative branch using a 12-bit sign extended displacement if the carry flag bit is set in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 025h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BEQ – Branch if Equal

Description

BEQ performs a PC relative branch using a 12-bit sign extended displacement if the zero-flag bit is set in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 027h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BGE – Branch if Greater or Equal

Description

BGE performs a PC relative branch using a 12-bit sign extended displacement if the negative-flag bit and overflow flag bit are both clear, or are both set in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 02Ch |

Operation:

if ((cc.n = 1 and cc.v = 1) or (cc.n = 0 and cc.v = 0))

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BGT – Branch if Greater Than

Description

BGT performs a PC relative branch using a 12-bit sign extended displacement if the negative-flag bit and overflow flag bit are both clear, or are both set and the zero-flag bit is clear in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 02Eh |

Operation:

if )((cc.n = 1 and cc.v = 1) or (cc.n = 0 and cc.v = 0)) and cc.z = 0)

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BHI – Branch if Higher

Description

BHI performs a PC relative branch using a 12-bit sign extended displacement if the zero-flag bit and carry flag bit are both clear in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 022h |

Operation:

if (cc.z = 0 and cc.c = 0)

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BHS – Branch if Higher or Same

Description

BHS performs a PC relative branch using a 12-bit sign extended displacement if the carry flag bit is clear in the condition codes register.

This is an alternate mnemonic for the BCC instruction.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 024h |

Operation:

if (cc.c = 0)

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BITA – Bitwise ‘And’ to Accumulator A

Description

This instruction works in the same manner as the ANDA instruction except that the result is discard and accumulator A is not updated. Only the result status flags are updated.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 085h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 095h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A5h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B5h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### BITB – Bitwise ‘And’ to Accumulator B

Description

This instruction works in the same manner as the ANDB instruction except that the result is discard and accumulator B is not updated. Only the result status flags are updated.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0C5h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D5h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E5h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F5h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### BITD – Bitwise ‘And’ to Accumulator D

Description

This instruction works in the same manner as the ANDD instruction except that the result is discarded, and accumulator D is not updated. Only the result status flags are updated.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 185h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 195h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1A5h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1B5h |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| M | D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ↕ | ↕ | 0 |  |

### BITMD – Bitwise ‘And’ to Mode Reg

Description

This instruction can be used to test bits in the mode register. Performing a BITMD instruction will clear the divide-by-zero and illegal operation bits in the register if they are tested. The result status of the and operation is returned in the Z flag of the ccr.

Note that operation of this instruction is slightly different than the 6309. This instruction may be used to test all bits of the mode register making it possible to detect native mode.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 23Ch |

Flags Affected:

Z set if result value is zero, otherwise cleared

| D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ↕ |  |  |

Mode Register Effects:

Z and O will be reset if tested.

|  |  |  |  | Z | O |  |  |  | I | F | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 0 |  |  |  |  |  |  |

### BLE – Branch if Less or Equal

Description

BLE performs a PC relative branch using a 12-bit sign extended displacement if the negative-flag bit and overflow flag bit are different or the zero-flag bit is set in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 02Fh |

Operation:

if ((cc.n <> cc.v) or (cc.z))

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BLO – Branch if Lower

Description

BLO performs a PC relative branch using a 12-bit sign extended displacement if the carry-flag bit is set in the condition codes register.

This is an alternate mnemonic for the BCS instruction.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 025h |

Operation:

if (cc.c)

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BLS – Branch if Lower or the Same

Description

BLS performs a PC relative branch using a 12-bit sign extended displacement if the carry-flag bit is set or the zero-flag bit is set in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 023h |

Operation:

if (cc.c or cc.z)

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BLT – Branch if Less Than

Description

BLT performs a PC relative branch using a 12-bit sign extended displacement if the negative-flag bit is not equal to the overflow-flag bit in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 02Dh |

Operation:

if (cc.n <> cc.v)

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BMI – Branch if Minus

Description

BMI performs a PC relative branch using a 12-bit sign extended displacement if the negative-flag bit is set in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 02Bh |

Operation:

if (cc.n)

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BNE – Branch if Not Equal

Description

BEQ performs a PC relative branch using a 12-bit sign extended displacement if the zero-flag bit is clear in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 026h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BPL – Branch if Plus

Description

BPL performs a PC relative branch using a 12-bit sign extended displacement if the negative-flag bit is clear in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 02Ah |

Operation:

if (cc.n = 0)

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BRA – Branch Always

Description

BRA always performs a PC relative branch using a 12-bit sign extended displacement.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 020h |

Operation:

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BRN – Branch Never

Description

BRA never performs a PC relative branch using a 12-bit sign extended displacement. It is effectively a two-byte NOP instruction. The displacement may contain any useful value.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 021h |

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BSR – Branch To Subroutine

Description

BSR performs a PC relative branch using a 12-bit sign extended displacement after pushing the address of the next instruction on the stack.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 08Dh |

Operation:

SP = SP - 2

Memory[SP] = PC

PC = PC + sign extend(disp12)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BVC – Branch if Overflow Clear

Description

BCC performs a PC relative branch using a 12-bit sign extended displacement if the overflow flag bit is clear in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 028h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### BVS – Branch if Overflow Set

Description

BCC performs a PC relative branch using a 12-bit sign extended displacement if the overflow flag bit is set in the condition codes register.

Instruction Format: REL

| 23 12 | 11 0 |
| --- | --- |
| Disp12 | 029h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### CLR – Clear Memory

Description

Zero is written to memory.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 00Fh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 06Fh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 07Fh |

Operation:

Flags Affected:

N clear

Z set

V clear

C clear

| E |  | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 0 | 1 | 0 | 0 |

### CLRA – Clear Accumulator A

Description

A zero is loaded into accumulator A.

Instruction Format: INH

11 0

04Fh

Operation:

Acca = 0

Flags Affected:

N cleared

Z is set

V is cleared

C is cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 0 | 0 |

### CLRB – Clear Accumulator B

Description

A zero is loaded into accumulator B.

Instruction Format: INH

11 0

05Fh

Operation:

Accb = 0

Flags Affected:

N cleared

Z is set

V is cleared

C is cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 0 | 0 |

### CLRD – Clear Accumulator D

Description

A zero is loaded into accumulator D.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

14Fh

Operation:

Accd = 0

Flags Affected:

N cleared

Z is set

V is cleared

C is cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 0 | 0 |

### CLRE – Clear Accumulator E

Description

A zero is loaded into accumulator E.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

24Fh

Operation:

Acca = 0

Flags Affected:

N cleared

Z is set

V is cleared

C is cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 0 | 0 |

### CLRF – Clear Accumulator F

Description

A zero is loaded into accumulator F.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

25Fh

Operation:

Accf = 0

Flags Affected:

N cleared

Z is set

V is cleared

C is cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 0 | 0 |

### CLRW – Clear Accumulator W

Description

A zero is loaded into accumulator W.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

15Fh

Operation:

Accw = 0

Flags Affected:

N cleared

Z is set

V is cleared

C is cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 1 | 0 | 0 |

### CMPA – Compare to Accumulator A

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 081h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 091h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A1h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B1h |

Flags Affected:

H the state of this bit is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### CMPB – Compare to Accumulator B

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0C1h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D1h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E1h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F1h |

Flags Affected:

H the state of this bit is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### CMPD – Compare to Accumulator D

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 183h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 193h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1A3h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1B3h |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### CMPE – Compare to Accumulator E

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 281h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 291h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2A1h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2B1h |

Flags Affected:

H the state of this bit is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### CMPF – Compare to Accumulator F

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 2C1h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 2D1h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2E1h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2F1h |

Flags Affected:

H the state of this bit is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### CMPR – Compare Register to Register

Description

Compare two registers.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

| 23 20 | 19 16 | 15 12 | 11 0 |
| --- | --- | --- | --- |
| ~ | r0 | r1 | 137h |

| r0/r1 |  | r0/r1 |  |
| --- | --- | --- | --- |
| 0 | D | 8 | A |
| 1 | X | 9 | B |
| 2 | Y | 10 | CC |
| 3 | U | 11 | DP |
| 4 | S | 12 | 0 |
| 5 | PC | 13 | 0 |
| 6 | W | 14 | E |
| 7 | resv | 15 | F |

Flags Affected:

N set equal to the most significant bit of the result

Z set if result value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of the most significant bit, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### CMPS – Compare to Stack Pointer

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 18Ch |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 19Ch |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1ACh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1BCh |

Flags Affected:

N set equal to bit 23 of the stack pointer

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### CMPU – Compare to User Stack Pointer

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 183h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 193h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1A3h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1B3h |

Flags Affected:

N set equal to bit 23 of the user stack pointer

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### CMPW – Compare to Accumulator W

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 081h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 091h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A1h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B1h |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### CMPX – Compare to X Index Register

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 08Ch |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 09Ch |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0ACh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0BCh |

Flags Affected:

N set equal to bit 23 of the index register

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### CMPY – Compare to Y Index Register

Description

This instruction performs a subtract operation and discards the result. The result status flags are updated.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 18Ch |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 19Ch |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1ACh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1BCh |

Flags Affected:

N set equal to bit 23 of the index register

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### COM – Complement Memory

Description

Memory is read, complemented then written.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 003h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 063h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 073h |

Operation:

Flags Affected:

N clear

Z set

V clear

C clear

| E |  | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ↕ | ↕ | 0 | 1 |

### COMA – Complement Accumulator A

Description

The ones complement of accumulator A is loaded into accumulator A.

Instruction Format: INH

11 0

043h

Operation:

Acca = ~Acca

Flags Affected:

N is set to bit 11 of the result

Z is set if the result is zero

V is cleared

C is set

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 | 1 |

### COMB – Complement Accumulator B

Description

The ones complement of accumulator B is loaded into accumulator B.

Instruction Format: INH

11 0

053h

Operation:

Accb = ~Accb

Flags Affected:

N is set to bit 11 of the result

Z is set if the result is zero

V is cleared

C is set

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 | 1 |

### COMD – Complement Accumulator D

Description

The ones complement of accumulator D is loaded into accumulator D.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

143h

Operation:

Accd = ~Accd

Flags Affected:

N is set to bit 23 of the result

Z is set if the result is zero

V is cleared

C is set

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 | 1 |

### COME – Complement Accumulator E

Description

The ones complement of accumulator E is loaded into accumulator E.

Instruction Format: INH

11 0

243h

Operation:

Acce = ~Acce

Flags Affected:

N is set to bit 11 of the result

Z is set if the result is zero

V is cleared

C is set

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 | 1 |

### COMF – Complement Accumulator F

Description

The ones complement of accumulator F is loaded into accumulator F.

Instruction Format: INH

11 0

253h

Operation:

Accf = ~Accf

Flags Affected:

N is set to bit 11 of the result

Z is set if the result is zero

V is cleared

C is set

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 | 1 |

### COMW – Complement Accumulator W

Description

The ones complement of accumulator W is loaded into accumulator W.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

153h

Operation:

Accw = ~Accw

Flags Affected:

N is set to bit 23 of the result

Z is set if the result is zero

V is cleared

C is set

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 | 1 |

### CWAI – Wait For Interrupt

Description

This instruction waits for an interrupt to occur and may be used to clear bits in the condition code register. The condition code register is bitwise anded with an immediate value. The E bit in the condition code register is set and the entire machine state is stored on the stack.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 03Ch |

Flags Affected:

Flags for which the immediate constant has a zero bit will be cleared, other flags will not be affected.

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### DAA – Decimal Adjust after Addition

Description

The value in accumulator A is adjusted after an addition to be consistent with a BCD number.

Instruction Format: INH

11 0

019h

Operation:

Acca = 0

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero, otherwise cleared

V is undefined

C is set if there is a carry out from bit 11

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ? | ↕ |

### DEC – Decrement Memory

Description

Memory is read, decremented and written.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 00Ah |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 06Ah |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 07Ah |

Operation:

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $800

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### DECA – Decrement Accumulator A

Description

Accumulator A is decremented by one.

Instruction Format: INH

11 0

04Ah

Operation:

Acca = Acca - 1

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $800

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### DECB – Decrement Accumulator B

Description

Accumulator B is decremented by one.

Instruction Format: INH

11 0

05Ah

Operation:

Accb = Accb - 1

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $800

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### DECD – Decrement Accumulator D

Description

Accumulator D is decremented by one.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

14Ah

Operation:

Accd = Accd - 1

Flags Affected:

N is set to the value of bit 23 of the result

Z is set if the result is zero.

V is set if the original value was $800000

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### DECE – Decrement Accumulator E

Description

Accumulator E is decremented by one.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

24Ah

Operation:

Acce = Acce - 1

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $800

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### DECF – Decrement Accumulator F

Description

Accumulator F is decremented by one.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

25Ah

Operation:

Accf = Accf - 1

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $800

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### DECW – Decrement Accumulator W

Description

Accumulator W is decremented by one.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

15Ah

Operation:

Accd = Accd - 1

Flags Affected:

N is set to the value of bit 23 of the result

Z is set if the result is zero.

V is set if the original value was $800000

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### DIVD – Divide Accumulator D by Memory

Description

Divide 24-bit accumulator D by a 12-bit value from memory. Both values are treated as signed values. If overflow occurs and the result will not fit into 12-bits the overflow flag is set.

If the divisor is zero a divide-by-zero interrupt will occur unless the address mode is immediate for which no interrupt occurs. The divide-by-zero interrupt can be tested in the mode register using the BITMD instruction.

Clock Cycles: approximately 28

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 28Dh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 29Dh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2ADh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2BDh |

Flags Affected:

N set equal to bit 11 of the result in accumulator B

Z set if accumulator B value is zero, otherwise cleared

V set if an overflow occurred, otherwise cleared

C set if the quotient in accumulator B is odd, otherwise cleared if even

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### DIVQ – Divide Accumulator D by Memory

Description

Divide 48-bit accumulator Q by a 24-bit value from memory. Both values are treated as signed values. If overflow occurs and the result will not fit into 24-bits the overflow flag is set.

Clock Cycles: approximately 56

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed lo | Immed hi | 28Eh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 29Eh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2AEh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2BEh |

Flags Affected:

N set equal to bit 11 of the result in accumulator B

Z set if accumulator B value is zero, otherwise cleared

V set if an overflow occurred, otherwise cleared

C set if the quotient in accumulator B is odd, otherwise cleared if even

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### EORA – Bitwise Exclusive ‘Or’ to Accumulator A

Description

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 088h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 098h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A8h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B8h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### EORB – Bitwise Exclusive ‘Or’ to Accumulator B

Description

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0C8h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D8h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E8h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F8h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### EORD – Bitwise Exclusive ‘Or’ to Accumulator D

Description

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 188h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 198h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1A8h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1B8h |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### EORR – Bitwise Exclusive ‘or’ Register to Register

Description

Exclusive or register to register.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

| 23 20 | 19 16 | 15 12 | 11 0 |
| --- | --- | --- | --- |
| ~ | r0 | r1 | 136h |

| r0/r1 |  | r0/r1 |  |
| --- | --- | --- | --- |
| 0 | D | 8 | A |
| 1 | X | 9 | B |
| 2 | Y | 10 | CC |
| 3 | U | 11 | DP |
| 4 | S | 12 | 0 |
| 5 | PC | 13 | 0 |
| 6 | W | 14 | E |
| 7 | resv | 15 | F |

Flags Affected:

N set equal to the most significant bit of the result

Z set if result value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### EXG – Exchange Registers

Description

Exchange two registers.

Instruction Format: INH

| 23 20 | 19 16 | 15 12 | 11 0 |
| --- | --- | --- | --- |
| ~ | r0 | r1 | 01Eh |

| r0/r1 |  | r0/r1 |  |
| --- | --- | --- | --- |
| 0 | D | 8 | A |
| 1 | X | 9 | B |
| 2 | Y | 10 | CC |
| 3 | U | 11 | DP |
| 4 | S | 12 | 0 |
| 5 | PC | 13 | 0 |
| 6 | W | 14 | E |
| 7 | resv | 15 | F |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### INC – Increment Memory

Description

Memory is read, incremented and written.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 00Ch |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 06Ch |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 07Ch |

Operation:

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $7FF

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### INCA – Increment Accumulator A

Description

Accumulator A is incremented by one.

Instruction Format: INH

11 0

04Ch

Operation:

Acca = Acca + 1

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $7FF

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### INCB – Increment Accumulator B

Description

Accumulator B is incremented by one.

Instruction Format: INH

11 0

05Ch

Operation:

Accb = Accb + 1

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $7FF

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### INCD – Increment Accumulator D

Description

Accumulator D is incremented by one.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

14Ch

Operation:

Accd = Accd + 1

Flags Affected:

N is set to the value of bit 23 of the result

Z is set if the result is zero.

V is set if the original value was $7FFFFF

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### INCE – Increment Accumulator E

Description

Accumulator E is incremented by one.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

24Ch

Operation:

Acce = Acce + 1

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $7FF

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### INCF – Increment Accumulator F

Description

Accumulator F is incremented by one.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

25Ch

Operation:

Accf = Accf + 1

Flags Affected:

N is set to the value of bit 11 of the result

Z is set if the result is zero.

V is set if the original value was $7FF

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### INCW – Increment Accumulator W

Description

Accumulator W is incremented by one.

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

15Ch

Operation:

Accw = Accw + 1

Flags Affected:

N is set to the value of bit 23 of the result

Z is set if the result is zero.

V is set if the original value was $7FFFFF

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ |  |

### JMP – Unconditional Jump

Description

Load the program counter with the source operand.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 00Eh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 06Eh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 07Eh |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### JSR –Jump to Subroutine

Description

Push the address of the next instruction on the stack, then perform a jump operation.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 09Dh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0ADh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0BDh |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBCC – Long Branch if Carry Clear

Description

LBCC performs a PC relative branch using a 24-bit sign extended displacement if the carry flag bit is clear in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 124h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBCS – Long Branch if Carry Set

Description

LBCS performs a PC relative branch using a 24-bit sign extended displacement if the carry flag bit is set in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 125h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBEQ – Long Branch if Equal

Description

LBEQ performs a PC relative branch using a 24-bit sign extended displacement if the zero-flag bit is set in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 127h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBGE – Long Branch if Greater or Equal

Description

LBGE performs a PC relative branch using a 24-bit sign extended displacement if the negative-flag bit and overflow flag bit are both clear, or are both set in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 12Ch |

Operation:

if ((cc.n = 1 and cc.v = 1) or (cc.n = 0 and cc.v = 0))

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBGT – Long Branch if Greater Than

Description

LBGT performs a PC relative branch using a 24-bit sign extended displacement if the negative-flag bit and overflow flag bit are both clear, or are both set and the zero-flag bit is clear in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 12Eh |

Operation:

if )((cc.n = 1 and cc.v = 1) or (cc.n = 0 and cc.v = 0)) and cc.z = 0)

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBHI – Branch if Higher

Description

LBHI performs a PC relative branch using a 24-bit sign extended displacement if the zero-flag bit and carry flag bit are both clear in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 122h |

Operation:

if (cc.z = 0 and cc.c = 0)

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBHS – Long Branch if Higher or Same

Description

LBHS performs a PC relative branch using a 24-bit sign extended displacement if the carry flag bit is clear in the condition codes register.

This is an alternate mnemonic for the BCC instruction.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 124h |

Operation:

if (cc.c = 0)

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBLE – Branch if Less or Equal

Description

LBLE performs a PC relative branch using a 24-bit sign extended displacement if the negative-flag bit and overflow flag bit are different or the zero-flag bit is set in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 12Fh |

Operation:

if ((cc.n <> cc.v) or (cc.z))

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBLO – Long Branch if Lower

Description

LBLO performs a PC relative branch using a 24-bit sign extended displacement if the carry-flag bit is set in the condition codes register.

This is an alternate mnemonic for the LBCS instruction.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 125h |

Operation:

if (cc.c)

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBLS – Long Branch if Lower or the Same

Description

LBLS performs a PC relative branch using a 24-bit sign extended displacement if the carry-flag bit is set or the zero-flag bit is set in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 123h |

Operation:

if (cc.c or cc.z)

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBLT – Long Branch if Less Than

Description

LBLT performs a PC relative branch using a 24-bit sign extended displacement if the negative-flag bit is not equal to the overflow-flag bit in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 12Dh |

Operation:

if (cc.n <> cc.v)

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBMI – Long Branch if Minus

Description

LBMI performs a PC relative branch using a 24-bit sign extended displacement if the negative-flag bit is set in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 12Bh |

Operation:

if (cc.n)

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBNE – Long Branch if Not Equal

Description

LBEQ performs a PC relative branch using a 24-bit sign extended displacement if the zero-flag bit is clear in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 126h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBPL – Long Branch if Plus

Description

LBPL performs a PC relative branch using a 24-bit sign extended displacement if the negative-flag bit is clear in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 12Ah |

Operation:

if (cc.n = 0)

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBRA – Long Branch Always

Description

LBRA always performs a PC relative branch using a 24-bit sign extended displacement.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 016h |

Operation:

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBRN – Long Branch Never

Description

LBRN never performs a PC relative branch using a 24-bit sign extended displacement. It is effectively a three-byte NOP instruction. The displacement may contain any useful value.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 121h |

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBSR – Long Branch To Subroutine

Description

LBSR performs a PC relative branch using a 24-bit sign extended displacement after pushing the address of the next instruction on the stack.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 017h |

Operation:

SP = SP - 2

Memory[SP] = PC

PC = PC + sign extend(disp24)

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBVC – Long Branch if Overflow Clear

Description

LBVC performs a PC relative branch using a 24-bit sign extended displacement if the overflow flag bit is clear in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 128h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LBVS – Long Branch if Overflow Set

Description

LBVS performs a PC relative branch using a 24-bit sign extended displacement if the overflow flag bit is set in the condition codes register.

Instruction Format: REL

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Disp lo | Disp hi | 129h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LDA – Load Accumulator A

Description

The source operand is loaded into accumulator A.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 086h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 096h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A6h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B6h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDB – Load Accumulator B

Description

The source operand is loaded into accumulator B.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0C6h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D6h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E6h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F6h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDD – Load Accumulator D

Description

The source operand is loaded into accumulator B.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 0CCh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0DCh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0ECh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0FCh |

Flags Affected:

N set equal to bit 23 of the accumulator (bit 11 of accumulator A)

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDE – Load Accumulator E

Description

The source operand is loaded into accumulator E.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 286h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 296h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2A6h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2B6h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDF – Load Accumulator F

Description

The source operand is loaded into accumulator F.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 2C6h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 2D6h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2E6h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2F6h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDMD – Load Mode Reg

Description

This instruction loads the processor mode register. The mode register is write-only.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 23Dh |

Flags Affected: none

Mode Register:

|  |  |  |  |  |  |  |  |  | I | F | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

N: 1 = native mode, Acce, accf stacked during interrupts.

F: 1 = FIRQ stacks all registers, 0 = FIRQ stacks only CCR and PC

I: 1 = interrupt lines act as priority encoder, 0 = interrupt lines operate normally

### LDS – Load Stack Pointer

Description

The source operand is loaded into the stack pointer.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 1CEh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 1DEh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1EEh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1FEh |

Flags Affected:

N set equal to bit 23 of the stack pointer

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDU – Load User Stack Pointer

Description

The source operand is loaded into the user stack pointer.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 0CEh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0DEh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0EEh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0FEh |

Flags Affected:

N set equal to bit 23 of the user stack pointer

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDW – Load Accumulator W

Description

The source operand is loaded into accumulator W.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 186h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 196h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1A6h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1B6h |

Flags Affected:

N set equal to bit 23 of the accumulator (bit 11 of accumulator E)

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDX – Load X Index Register

Description

The source operand is loaded into the X index register.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 08Eh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 09Eh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0AEh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0BEh |

Flags Affected:

N set equal to bit 23 of the index register

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LDY – Load Y Index Register

Description

The source operand is loaded into the Y index register.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 18Eh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 19Eh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1AEh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1BEh |

Flags Affected:

N set equal to bit 23 of the index register

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### LEAS – Load Effective Address Into S

Description

The address of the source operand is loaded into the stack pointer.

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 032h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LEAU – Load Effective Address Into U

Description

The address of the source operand is loaded into the user stack pointer.

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 033h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### LEAX – Load Effective Address Into X

Description

The address of the source operand is loaded into the stack pointer.

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 030h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ↕ |  |  |

### LEAY – Load Effective Address Into Y

Description

The address of the source operand is loaded into the stack pointer.

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 031h |

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ↕ |  |  |

### LSL – Logical Shift Left Memory

Description

Memory is read, bits are shifted to the left by one bit, then the result is written back to memory. A zero is shifted into the least significant bit and the most significant bit is captured in the carry result flag.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 008h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 068h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 078h |

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the result

Z set if result value is zero, otherwise cleared

V set to the exclusive or of bits 10 and 11

C set to the original value of bit 11

| E |  | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### LSLA – Logical Shift Left Accumulator A

Description

Bits in the accumulator A are shifted once to the left. A zero is shifted into the least significant bit and the most significant bit is captured in the carry result flag.

Instruction Format: INH

11 0

048h

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 10 and 11

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### LSLB – Logical Shift Left Accumulator B

Description

Bits in the accumulator B are shifted once to the left. A zero is shifted into the least significant bit and the most significant bit is captured in the carry result flag.

Instruction Format: INH

11 0

058h

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 10 and 11

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### LSLD – Logical Shift Left Accumulator D

Description

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

148h

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 22 and 23

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### LSR – Logical Shift Right Memory

Description

Memory is read, bits are shifted to the right by one bit, then the result is written back to memory. A zero is shifted into the most significant bit and the least significant bit is captured in the carry result flag.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 004h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 064h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 074h |

Operation:

Flags Affected:

N set equal to bit 11 of the result

Z set if result value is zero, otherwise cleared

C set to the original value of bit 0

| E |  | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ↕ | ↕ |  | ↕ |

### LSRA – Logical Shift Right Accumulator A

Description

Bits in the accumulator A are shifted once to the right. A zero is shifted into the most significant bit and the least significant bit is captured in the carry result flag.

Instruction Format: INH

11 0

044h

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

C set if there is a carry out of bit 0, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ |  | ↕ |

### LSRB – Logical Shift Right Accumulator B

Description

Bits in the accumulator B are shifted once to the right. A zero is shifted into the most significant bit and the least significant bit is captured in the carry result flag.

Instruction Format: INH

11 0

054h

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

C set if there is a carry out of bit 0, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ |  | ↕ |

### LSRD – Logical Shift Right Accumulator D

Description

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

144h

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

C set if there is a carry out of bit 0, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ |  | ↕ |

### MUL – Multiply

Description

Accumulators A and B are multiplied, and the resulting product is placed in D. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: INH

11 0

03Dh

Operation:

Accd = Acca * Accb

Flags Affected:

Z is set if the result is zero, otherwise cleared

C is set to the new value of bit 11 of accumulator B

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ↕ |  | ↕ |

### MULD – Multiply Accumulator D by Memory

Description

Multiply 24-bit accumulator D by a 24-bit value from memory. Both values are treated as signed values.

Clock Cycles: approximately 10

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 28Fh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 29Fh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 2AFh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 2BFh |

Flags Affected:

N set if result is negative

Z set if result value is zero, otherwise cleared

| M | D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ↕ | ↕ |  |  |

### NEG – Negate Memory

Description

Memory is read, negated, then written.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 000h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 060h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 070h |

Operation:

Flags Affected:

N set equal to bit 11 of memory

Z set if value is zero, otherwise cleared

V set if the original value is $800

C cleared if the original value was zero

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### NEGA – Negate Accumulator A

Description

Accumulator A is negated. If the decimal mode flag bit is set in the condition code register then the operand is treated as a BCD number and the result is a BCD result. Otherwise, the operand is treated as a signed twos complement number.

Instruction Format: INH

11 0

040h

Operation:

acca = 0-acca

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if the original value is $800

C cleared if the original value was zero

| M | D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### NEGB – Negate Accumulator B

Description

Accumulator B is negated. If the decimal mode flag bit is set in the condition code register then the operand is treated as a BCD number and the result is a BCD result. Otherwise, the operand is treated as a signed twos complement number.

Instruction Format: INH

11 0

050h

Operation:

accb = 0-accb

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if the original value is $800

C cleared if the original value was zero

| M | D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### NEGD – Negate Accumulator D

Description

Accumulator D is negated. If the decimal mode flag bit is set in the condition code register then the operand is treated as a BCD number and the result is a BCD result. Otherwise, the operand is treated as a signed twos complement number.

Instruction Format: INH

11 0

140h

Operation:

accd = 0-accd

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if the original value is $800000

C cleared if the original value was zero

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### NOP – No Operation

Description

This instruction does not perform any operation.

Instruction Format: INH

11 0

012h

Operation:

none

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### ORA – Bitwise ‘Or’ to Accumulator A

Description

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 08Ah |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 09Ah |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0AAh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0BAh |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### ORB – Bitwise ‘Or’ to Accumulator B

Description

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0CAh |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0DAh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0EAh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0FAh |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### ORCC – Bitwise ‘Or’ to Condition Code Reg

Description

This instruction can be used to set bits in the condition code register. A common use is to set the interrupt mask bits.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 01Ah |

Flags Affected:

Flags for which the immediate constant has a one bit will be set, other flags will not be affected.

|  |  | M | D | E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### ORD – Bitwise ‘Or’ to Accumulator D

Description

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 18Ah |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 19Ah |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1AAh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1BAh |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### PSHS – Push onto Stack

Description

This instruction is used to store registers to the stack.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Post-byte | 034h |

Registers are pushed from higher memory addresses to lower memory addresses in the order outlined below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC | higher memory address |

Push / pull post-byte (12-bit bytes)

| 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ~ | ~ | F | E | PC | U or S | Y | X | DP | B | A | CCR |

Flags Affected:

Flags for which the immediate constant has a one bit will be set, other flags will not be affected.

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### PSHU – Push onto User Stack

Description

This instruction is used to store registers to the user stack.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Post-byte | 036h |

Registers are pushed from higher memory addresses to lower memory addresses in the order outlined below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC | higher memory address |

Push / pull post-byte (12-bit bytes)

| 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ~ | ~ | F | E | PC | U or S | Y | X | DP | B | A | CCR |

Flags Affected:

Flags for which the immediate constant has a one bit will be set, other flags will not be affected.

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### PULS – Pull from Stack

Description

This instruction is used to load registers from the stack.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Post-byte | 035h |

Registers are pulled from lower memory addresses to higher memory addresses as in the order outlined below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC | higher memory address |

Push / pull post-byte (12-bit bytes)

| 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ~ | ~ | F | E | PC | U or S | Y | X | DP | B | A | CCR |

Flags Affected:

Flags for which the immediate constant has a one bit will be set, other flags will not be affected.

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### PULU – Pull from User Stack

Description

This instruction is used to load registers from the user stack.

Instruction Format: INH

| 23 12 | 11 0 |
| --- | --- |
| Post-byte | 037h |

Registers are pulled from lower memory addresses to higher memory addresses as in the order outlined below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC | higher memory address |

Push / pull post-byte (12-bit bytes)

| 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ~ | ~ | F | E | PC | U or S | Y | X | DP | B | A | CCR |

Flags Affected:

Flags for which the immediate constant has a one bit will be set, other flags will not be affected.

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### ROL – Rotate Left Memory

Description

Memory is read, bits are shifted to the left by one bit, then the result is written back to memory. The most significant bit is captured in the carry result flag. The original carry bit is shifted into the least significant memory bit.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 009h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 069h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 079h |

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the result

Z set if result value is zero, otherwise cleared

V set to the exclusive or of bits 10 and 11

C set to the original value of bit 11

| E |  | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### ROLA – Rotate Left Accumulator A

Description

Bits in the accumulator A are shifted once to the left. The most significant bit is shifted into the carry and carry shifted into the least significant bit.

Instruction Format: INH

11 0

049h

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 10 and 11

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### ROLB – Rotate Left Accumulator B

Description

Bits in the accumulator B are shifted once to the left. The most significant bit is shifted into the carry and carry shifted into the least significant bit.

Instruction Format: INH

11 0

059h

Operation:

Flags Affected:

H setting is undefined

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 10 and 11

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ? |  | ↕ | ↕ | ↕ | ↕ |

### ROLD – Rotate Left Accumulator D

Description

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

149h

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 22 and 23

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### ROR – Rotate Right Memory

Description

Memory is read, bits are shifted to the right by one bit, then the result is written back to memory. The least significant bit is captured in the carry result flag. The original carry bit is shifted into the most significant memory bit.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 006h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 066h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 076h |

Operation:

Flags Affected:

N set equal to bit 11 of the result

Z set if result value is zero, otherwise cleared

V set to the exclusive or of bits 10 and 11

C set to the original value of bit 0

| E |  | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### RORA – Rotate Right Accumulator A

Description

Bits in the accumulator A are shifted once to the right. The least significant bit is shifted into the carry and carry shifted into the most significant bit.

Instruction Format: INH

11 0

046h

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 10 and 11

C set if there is a carry out of bit 0, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### RORB – Rotate Right Accumulator B

Description

Bits in the accumulator B are shifted once to the right. The least significant bit is shifted into the carry and carry shifted into the most significant bit.

Instruction Format: INH

11 0

056h

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 10 and 11

C set if there is a carry out of bit 0, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### RORD – Rotate Right Accumulator D

Description

- This instruction is available only if 6309 instruction support is configured.

Instruction Format: INH

11 0

146h

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set to the exclusive or of accumulator bits 22 and 23

C set if there is a carry out of bit 0, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### RTF – Return From Far Subroutine

Description

RTF returns from a far subroutine by loading the program bank and program counter from stack. Note that often the program counter may be pulled from the stack at the same time as other registers using the PULS instruction.

Instruction Format: INH

11 0

038h

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### RTI – Return From Interrupt

Description

RTI restores the state of the machine from the stack and is used at the end of an interrupt processing routine to return to the interrupted code.

If 6309 instruction support is enabled and the entire machine state was stacked, then the E, F registers will be restored from the stack.

Registers are restored from lower to higher memory addresses as outlined in the table below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC Bank | higher memory address |
| PC |  |

Instruction Format: INH

11 0

03Bh

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### RTS – Return From Subroutine

Description

RTS returns from a subroutine by loading the program counter from stack. Note that often the program counter may be pulled from the stack at the same time as other registers using the PULS instruction.

Instruction Format: INH

11 0

039h

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### SBCA – Subtract with Carry from Accumulator A

Description

The source operand is subtracted from accumulator A including a carry. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 082h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 092h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A2h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B2h |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### SBCB – Subtract with Carry from Accumulator B

Description

The source operand is subtracted from accumulator B including a carry. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0C2h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D2h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E2h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F2h |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### SBCD – Subtract with Carry from Accumulator D

Description

The source operand is subtracted from accumulator D including a carry. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

*This instruction is available only if 6309 instruction supported is configured.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed Lo | Immed Hi | 182h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 192h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1A2h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1B2h |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 23, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### SEX – Sign Extend

Description

Sign-extend the value from accumulator B into accumulator A.

Instruction Format: INH

11 0

01Dh

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator B

Z set if accumulator value is zero, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ |  |  |  |

### STA – Store Accumulator A

Description

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 097h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A7h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B7h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### STB – Store Accumulator B

Description

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D7h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E7h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F7h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### STD – Store Accumulator D

Description

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0DDh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0EDh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0FDh |

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### STS – Store Stack Pointer

Description

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 1DFh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1EFh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1FFh |

Flags Affected:

N set equal to bit 23 of the stack pointer

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### STU – Store User Stack Pointer

Description

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0DFh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0EFh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0FFh |

Flags Affected:

N set equal to bit 23 of the stack pointer

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### STX – Store X Register

Description

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 09Fh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0AFh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0BFh |

Flags Affected:

N set equal to bit 23 of the X register

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### STY – Store Y Register

Description

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 19Fh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 1AFh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 1BFh |

Flags Affected:

N set equal to bit 23 of the Y register

Z set if accumulator value is zero, otherwise cleared

V always cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### SUBA – Subtract from Accumulator A

Description

The source operand is subtracted from accumulator A.  Carry is not included in the subtraction but is still generated as a result flag. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 080h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 090h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A0h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B0h |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### SUBB – Subtract from Accumulator B

Description

The source operand is subtracted from accumulator B.  Carry is not included in the subtraction but is still generated as a result flag. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 23 12 | 11 0 |
| --- | --- |
| Immed12 | 0C0h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 0D0h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0E0h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0F0h |

Flags Affected:

H set if there is a carry out of bit 4, otherwise cleared

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | ↕ |  | ↕ | ↕ | ↕ | ↕ |

### SUBD – Subtract from Accumulator D

Description

The source operand is subtracted from accumulator D.  Carry is not included in the subtraction but is still generated as a result flag. If the decimal mode flag bit is set in the condition code register then the operands are treated as BCD numbers and the result is a BCD result. Otherwise, the operands are treated as signed twos complement numbers.

Instruction Format: IMM

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Immed24 |  | 083h |

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 093h |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 0A3h |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 0B3h |

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V set if overflow occurred, otherwise cleared

C set if there is a carry out of bit 11, otherwise cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | ↕ | ↕ |

### SWI – Software Interrupt

Description

SWI stores the entire state of the machine onto the stack then vectors to the SWI processing routine. Interrupts are masked by the SWI instruction.

If 6309 instruction support is enabled and the entire machine state was stacked, then the E, F registers will be restored from the stack.

Registers are restored from lower to higher memory addresses as outlined in the table below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC Bank | higher memory address |
| PC |  |

Instruction Format: INH

11 0

03Fh

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  | 1 |  |  |  |  |

### SWI2 – Software Interrupt

Description

SWI stores the entire state of the machine onto the stack then vectors to the SWI2 processing routine.

If 6309 instruction support is enabled and the entire machine state was stacked, then the E, F registers will be restored from the stack.

Registers are restored from lower to higher memory addresses as outlined in the table below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC Bank | higher memory address |
| PC |  |

Instruction Format: INH

11 0

13Fh

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### SWI3 – Software Interrupt

Description

SWI stores the entire state of the machine onto the stack then vectors to the SWI3 processing routine.

If 6309 instruction support is enabled and the entire machine state was stacked, then the E, F registers will be restored from the stack.

Registers are restored from lower to higher memory addresses as outlined in the table below.

| CCR | Lower memory address |
| --- | --- |
| A |  |
| B |  |
| E |  |
| F |  |
| DP |  |
| X |  |
| Y |  |
| U or S |  |
| PC Bank | higher memory address |
| PC |  |

Instruction Format: INH

11 0

23Fh

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### SYNC – Halt and Wait for Interrupt

Description

SYNC activates the address and data bus tristate controls while waiting for an interrupt.

Instruction Format: INH

11 0

013h

Operation:

Flags Affected:

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### TFR – Transfer Registers

Description

Transfer register to register.

If a 24-bit register is transferred to a twelve-bit one the twelve-bit register is set to the lower order 12-bits of the 24-bit register.

If accumulator A or B is transferred to a 24-bit register, the most significant 12-bits of the destination are set to $FFF, while the low order byte of the destination is set to the value of the accumulator register.

For other twelve-bit registers (CC or DP) the twelve-bit register value is copied to both the upper and lower bytes of the 24-bit register.

Transfers involving the PC use only the two low order bytes of the PC.

| Transfer Type | Register |  |
| --- | --- | --- |
| 24 to 12 | Any | Low order 12-bits from source copied to destination |
| 12 to 24 | A, B, E, or F | Lower order 12-bits set to accumulator, high order bits set to $FFF |
| 12 to 24 | CCR, DP | Source copied to both high and low order bytes of destination. |

Instruction Format: INH

| 23 20 | 19 16 | 15 12 | 11 0 |
| --- | --- | --- | --- |
| ~ | r0 | r1 | 01Fh |

| r0/r1 |  | r0/r1 |  |
| --- | --- | --- | --- |
| 0 | D | 8 | A |
| 1 | X | 9 | B |
| 2 | Y | 10 | CC |
| 3 | U | 11 | DP |
| 4 | S | 12 | 0 |
| 5 | PC | 13 | 0 |
| 6 | W | 14 | E |
| 7 | resv | 15 | F |

Flags Affected:

No flags are affected unless the transfer is into the CCR register.

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |

### TST – Test Memory

Description

Memory is tested against the value zero.

Instruction Format: DP

| 23 12 | 11 0 |
| --- | --- |
| Offset12 | 00Dh |

Instruction Format: NDX

|  |  | 23 12 | 11 0 |
| --- | --- | --- | --- |
| As needed |  | Ndx12 | 06Dh |

Instruction Format: EXT

| 35 24 | 23 12 | 11 0 |
| --- | --- | --- |
| Address Lo | Address Hi | 07Dh |

Operation:

Flags Affected:

N set equal to bit 11 of memory

Z set if value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### TSTA – Test Accumulator A

Description

Accumulator A is tested against the value zero.

Instruction Format: INH

11 0

04Dh

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### TSTB – Test Accumulator B

Description

Accumulator B is tested against the value zero.

Instruction Format: INH

11 0

05Dh

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### TSTD – Test Accumulator D

Description

Accumulator D is tested against the value zero.

*This instruction is available only if 6309 instruction supported is configured.

Instruction Format: INH

11 0

14Dh

Operation:

accd = -accd

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### TSTE – Test Accumulator E

Description

Accumulator E is tested against the value zero.

*This instruction is available only if 6309 instruction supported is configured.

Instruction Format: INH

11 0

24Dh

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### TSTF – Test Accumulator F

Description

Accumulator F is tested against the value zero.

*This instruction is available only if 6309 instruction supported is configured.

Instruction Format: INH

11 0

25Dh

Operation:

Flags Affected:

N set equal to bit 11 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |

### TSTW – Test Accumulator W

Description

Accumulator W is tested against the value zero.

*This instruction is available only if 6309 instruction supported is configured.

Instruction Format: INH

11 0

15Dh

Operation:

Flags Affected:

N set equal to bit 23 of the accumulator

Z set if accumulator value is zero, otherwise cleared

V cleared

| E | F | H | I | N | Z | V | C |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ↕ | ↕ | 0 |  |