Random Number Generator Library

User Guide

Author: Geir Drange

gedra@opencores.org

Rev. 1.0

July 30, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 02/09/04 | Geir Drange | First version |

Contents

1 Introduction	1

2Using the library	2

2.1 Seeds	2

3 Example test	3

4 Known issues	6

Introduction

When writing test benches, it is often useful to include a degree of randomness in the simulation. Typical examples are:

- Random data generation

- Adding noise to signals

- Generate random delays

- Clock jitter generation

- Etc

VHDL does not have any built-in functions for random numbers (yet). The Random Number Generator Library can easily be included in a test bench to supply random number functions.

Note that this library will NOT synthesize. It is intended for test bench use only.

Using the library

To use the library, add the following line to the test bench:

use work.rng_lib.all;

A random variable is defined by a record, and must be initialized before use. There is a separate initialization function for each type of distribution. Example:

r_uni := init_uniform(0, 0, 0, 0.0, 200.0);

will initialize a uniform distributed variable with values between 0 and 200. The random variable (real) is accessed by the .rnd member of the record:

r := r_uni.rnd; -- r is a real

i := integer(r_uni.rnd); -- i is an integer

Generate a new number with the rand() function:

r_uni := rand(r_uni);

## Seeds

The first 3 parameters (a, b, c) for the initialization functions are seeds for the Tausworthe generators. If a seed is set to zero, a predefined seed will be used. Predefined seeds will also be used if a < 3, b < 9 or c < 17.

Example test

The file tb_rng.vhd will generate 10000 numbers of each of the distributions, and make simple plots in the log window:

Uniform distribution:

*************************************

****************************************

*************************************

***********************************

****************************************

***************************************

*************************************

**************************************

*************************************

***********************************

****************************************

*************************************

************************************

**************************************

*************************************

Gaussian distribution:

*

**

******

************

*******************

***************************

*************************************

****************************************

************************************

****************************

********************

************

*****

**

*

Exponential distribution:

****************************************

******************************

**********************

****************

************

*********

******

*****

****

***

**

*

*

*

Know issues

The library will not work with Symphony EDA Sonata simulator. Due to bad floating point support?