rtfSimpleUart

# Table of Contents

Table of Contents	2

Overview	3

Register Description	4

Baud Rate Generation	6

Core Parameters	6

pClkFreq	6

pBaud	6

pRts	6

pDtr	6

System Interface	7

Signal Description	7

SAMPLE SOFTWARE USAGE:	9

# Overview

A UART component (Universal Asynchronous Transmitter / Reciever) is used for the asynchronous transmission and reception of data. Asynchronous referring to the lack of a clock signal during transmission or reception.

rtfSimpleUart is a simplified UART component organized to be compatible with a more advanced UART component. Simplifications for the rtfSimpleUart include support for only a single transmission / reception format which is 1 start bit, 1 stop bit, 8 data bits without parity. The rtfSimpleUart is also contains only a single transmit / receive buffer, the fifo present in the more advanced UART is missing.

# Register Description

| Reg | Name |  |  |  |
| --- | --- | --- | --- | --- |
| 0 | TRB | RW | Transmit receive buffer - write to transmit buffer - read from receive buffer |  |
| 1 | LS | RO | Line Status register Bit. 0; = receiver not empty, this bit is set if there is any data available in the receiver. 1; overrun, this bit is set if receiver overrun occurs. 3; Framing error, this bit is set if there was a framing error with the current byte in the receiver. 5; Transmitter not full, this bit is set if the transmitter can accept more data. 6; Transmitter empty, this bit is set if the transmitter is completely empty. |  |
| 2 | MS | RO | Modem Status register Writing to the modem status register clears the change indicators and also clears a modem status interrupt Bit. 3; Change on dcd signal. 4; Cts signal level. 5; Dsr signal level. 6; Ri signal level. 7; Dcd signal level. |  |
| 3 | IS | RO | Interrupt Status register Bit. 0-4; Mailbox number. 0-1; Contain the value “00”. 2-4; Encoded interrupt level. 5-6; Not used. 7; 1 = interrupt pending, 0 = no interrupt. |  |
| 4 | IE | RW | Interrupt Enable register Bit. 0; Receiver interrupt – interrupt on data present. 1; Transmitter interrupt- interrupt on buffer empty. 3; Modem status interrupt – dcd changed. 5-7; Unused, reserved. |  |
| 5 | FF | RW | Frame Format register This register is reserved for compatibility with the advanced UART |  |
| 6 | MC | RW | Modem Control register Bit. 0; DTR signal level output. 1; RTS signal level output. |  |
| 7 |  |  | Control Register Bit. 0; Hardware flow control, when this bit is set, the transmitter output is controlled by the CTS signal automatically. |  |
| 8 | CM0 | RW | Clock multiplier LSB - unimplemented |  |
| 9 | CM1 | RW | Clock multiplier |  |
| 10 | CM2 | RW | Clock Multiplier |  |
| 11 | CM3 | RW | Clock Multiplier MSB |  |
| 12 | FC | RW | Fifo control register – this register is reserved for compatibility with the advanced UART |  |
| 13 | RES1 |  | This is a reserved register |  |
| 14 | RES2 |  | This is a reserved register |  |
| 15 | SPR | RW | Scratchpad register |  |

# Baud Rate Generation

In a small SOC (system on chip) there may not be many options as to the clock frequencies available. rtfSimpleUart contains baud rate generation circuit based on a harmonic frequency synthesizer that can work with a wide variety of clock frequencies.

The harmonic synthesizer acts like a device that multiplies a low frequency clock up to the desired operating frequency.

In this case the input bus clock is used as a frequency source. The baud rate clock is generated as a multiple of the input clock divided by 2 ^ 32. A 32 bit multiplier value is placed into register CM3(MSB) to CM0 (LSB).

For example, assume a SOC uses the standard VGA frequency 25.175 MHz as a master clock. Assume a 9600 baud RS232 connection is desired. A 153.6 kHz clock is required as the 16x clock for 9600 baud. The harmonic synthesizer acts as if the 25.175 MHz clock were divided by 2 ^ 32, giving a bottom frequency clock of 5.86 milli-hertz. In order to get 153.6 kHz from this clock we need to multiply by : 153.6 kHz / 5.86 mHz =

26204845.15. This is 018FDAAD in hexadecimal. So the value is placed into the clock multiplier registers as follow: 01 >= CM3, 8F >= CM2 and DA =>CM1, the least significant clock multiplier register, CM0, is not set, because it is not implemented. If it were implemented, it would be set to AD.

# Core Parameters

pClkFreq – This is the frequency of the input clock in Hertz. 25.175MHz would be specified as 25175000. This parameter must be specified or the core won’t produce proper baud rate.

pBaud – This parameter specifies the desired baud rate. The default value is 19200 baud. This is a convenience parameter that attempts to automatically calculate the required clock multiplier values. Note that this parameter does not work correctly for all baud rates. It invokes an automatic calculation of clock multiplier values which is subject to numeric overflow in some circumstances. It’s best to set the rate with the clock multiplier registers.

pRts – This is the default reset value of the RTS signal. It set to active (1) by default.

pDtr – This is the default reset value for the DTR signal. It is set to active (1) by default.

# System Interface

The core interfaces to the system through a WISHBONE compatible bus. Note that the sel_i input is missing, since the core is byte wide only.

The core uses internal address decoding that place it at the address $FFDC_0A0x.

# Signal Description

| Name | Bits | Description |
| --- | --- | --- |
| WISHBONE Bus Signals |  |  |
| rst_i | 1 | Reset signal, active high |
| clk_i | 1 | Bus clock signal |
| cyc_i | 1 | Cycle valid |
| stb_i | 1 | data strobe |
| we_i | 1 | write enable |
| adr_i | 32 | 32 bit address bus input |
| dat_i | 8 | input data bus |
| dat_o | 8 | output data bus |
| ack_o | 1 | acknowledge back to bus master |
| Other bus Signals |  |  |
| vol_o | 1 | volatile register is being accessed |
| irq_o | 1 | interrupt request |
| Modem Control |  |  |
| cts_ni | 1 | clear to send input, active low |
| rts_no | 1 | request to send output, active low |
| dsr_ni | 1 | data set ready input, active low |
| dcd_ni | 1 | data carrier detect input, active low |
| dtr_no | 1 | data terminal ready, output active low |
| rxd_i | 1 | receive data in |
| txd_o | 1 | transmit data out |
| data_present | 1 | data present in receiver output |

|  | Please refer to the WISHBONE spec available at OpenCores.org for a more complete description of the WISHBONE compatible signals. |
| --- | --- |
| rst_i | This is the synchronous reset active high reset signal. Registers are reset to their default values, and transmitter and receiver state machines are reset. |
| clk_i | This is the bus clock for timing bus transactions, it is the only clock input. The baud rate clock is generated from this clock via harmonic synthesis. |
| vol_o | This signal indicates to the system that the registers being accessed during a read cycle are ‘volatile’ registers whose state may be changed by hardware. This signal may be used to disable I/O register shadowing. |
| cts_ni | This input signal is ignored unless hardware flow control is turned on. By default hardware flow control is turned off. When used, CTS (clear to send) indicates that the other device is ready to receive data. This signal should be connected to ground if not used. |
| rts_no | This signal is active by default. The default setting may be specified with the pRts parameter |
| dtr_no | This signal is active by default. The default setting may be specified with the pDtr parameter. |

# SAMPLE SOFTWARE USAGE:

This is an extract of code from Tiny Basic 68000. The UART is in use as the auxilliary

port for tiny basic. The sample is in 68000 assembly language. The sample uses default

settings of the UART, which is 19.2k baud, so there is no initialization required.

;==============================================================================

;==============================================================================

UART		EQU		0xFFDC0A00

UART_LS		EQU		UART+1

UART_CTRL	EQU		UART+7

;*

;* ===== Output character to the host (Port 2) from register D0

;*	(Preserves all registers.)

;*

AUXOUT:

BTST	#5,UART_LS	;is port ready for a character?

BEQ		AUXOUT		;if not, wait for it

MOVE.B	D0,UART		;out it goes.

RTS

;*

;* ===== Input a character from the host into register D0 (or

;*	return Zero status if there's no character available).

;*

AUXIN:

BTST	#0,UART_LS	;is character ready?

BEQ		AXIRET		;if not, return Zero status

MOVE.B	UART,D0		;else get the character

AND.B	#0x7F,D0 	;zero out the high bit

AXIRET:

RTS

;==============================================================================

;==============================================================================