Secure Hash Algorithm

IP Core

Author: marsgod

marsgod@opencores.org

Rev 1.0

May 16, 2004

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 05/16/2004 | marsgod | Initial Release |

(This page intentionally left blank)

# 1

Introduction

This is a collection of SHA(Secure Hash Algorithm) cores. These include SHA-1, SHA-2 algorithms. In another word, these cores can perform following operations:

| Core | Type | Message | Digest | Cycles | Note |
| --- | --- | --- | --- | --- | --- |
| sha1.v | SHA-1 | 512 | 160 | 81 | With NSA fix |
| sha256.v | SHA-256 | 512 | 256 | 65 |  |
| sha512.v | SHA-384 | 1024 | 384 | 97 |  |
| sha512.v | SHA-512 | 1024 | 512 | 97 |  |

These cores are non-pipelined version of SHA, and have simple interfaces with the host side. Some features of the cores:

- Support SHA-1(160), SHA-2(256/384/512)

- Use a simple 32-bit I/O bus interface

- High performance

- Share hardware between different SHA processing

- Can operate up to 200MHz at 0.18um Standard cell design

These cores have been verified with Shamus Software Ltd’s MIRACL. You can get this software library from http://indigo.ie/~mscott/.

NOTE: The padding are not implemented in these cores, it is the host’s job to do the message padding.