SPDIF
Specification

Author: Geir Drange

gedra@opencores.org

Rev. 1.5

July 29, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 30/4/04 | Geir Drange | First Draft |
| 1.0 | 15/5/04 | Geir Drange | Updated |
| 1.1 | 13/6/04 | Geir Drange | Renamed generics and modified receiver configuration register |
| 1.2 | 26/6/04 | Geir Drange | Corrected a few typo’s. |
| 1.3 | 15/7/04 | Geir Drange | Changed two interrupts in transmitter |
| 1.4 | 19/7/04 | Geir Drange | Corrected some typo’s |
| 1.5 | 21/7/04 | Geir Drange | Final corrections before release. |

Contents

1	1

2	2

2.1 SPDIF Receiver	2

2.2 SPDIF Transmitter	3

3	5

3.1 Receiver	5

3.1.1	Resetting	5

3.1.2	Capture registers	5

3.1.3	Enable receiver	5

3.1.4	Transferring data	5

3.2 Transmitter	5

3.2.1	Resetting	5

3.2.2	Selecting transmit data rate	6

3.2.3	Selecting data format	6

3.2.4	Setting up channel status bits	6

3.2.5	Setting up user data bits	6

3.2.6	Preparing sample buffer	6

3.2.7	Start transmission	6

4	8

4.1 Generics for both transmitter and receiver	8

4.2 Generics for the receiver	8

4.3 Generics for the transmitter	8

5	9

5.1 SPDIF Receiver	9

5.1.1	Receiver registers - overview	9

5.1.2	RxVersion – Description	10

5.1.3	RxConfig – Description	10

5.1.4	RxStatus – Description	11

5.1.5	RxIntMask – Description	11

5.1.6	RxIntStat – Description	12

5.1.7	ChStCap<n> – Description	13

5.1.8	ChStData<n> – Description	13

5.1.9	Receive sample data – Description	13

5.2 SPDIF Transmitter	14

5.2.1	Transmitter registers - overview	14

5.2.2	TxVersion – Description	14

5.2.3	TxConfig – Description	14

5.2.4	TxChStat – Description	15

5.2.5	TxIntMask – Description	16

5.2.6	TxIntStat – Description	16

5.2.7	UserData – Description	17

5.2.8	ChStat – Description	17

5.2.9	Transmit sample data – Description	17

6	19

7	20

7.1 Wishbone interface signals (receiver & transmitter)	20

7.2 SPDIF Receiver signals	21

7.3 SPDIF Transmitter signals	21

Introduction

The SPDIF (or AES/EBU, IEC950 standards) is a point-to-point protocol for serial transmission of digital audio through a single transmission line. The transmission medium can be either electrical or optical (e.g. TosLink). It provides two channels for audio data, a method for communicating control information, and some error detection capabilities. The control information is transmitted as one bit per sample and accumulates in a block structure. The data is bi-phase encoded, which enables the receiver to extract a clock from the data. Coding violations, defined as preambles, are used to identify sample and block boundaries.

SPDIF interfaces are found on most CD/DVD players, audio equipment and computer sound cards.

Architecture

The SPDIF interface consists of two separate cores, a transmitter and a receiver

## SPDIF Receiver

The receiver architecture is shown below.

Wishbone Bus Interrupt SPDIF In Registers Phase detector Data capture registers Frame decoder Sample buffer Sample extractor

Figure 1: Receiver Block Diagram

The bi-phase encoded SPDIF signal is decoded by an oversampling phase detector. The wishbone bus clock is used to sample the input signal, and must be at least 8 times higher than then SPDIF data rate. The lowest SPDIF bit-rate supported is 100kHz.

A frame decoder locks onto the code violations (preambles) that mark the start of frames and sub-frames. Audio data is extracted and placed into the sample buffer. The size of the sample buffer is determined by the Wishbone address bus width. Minimum sample buffer size is 128bytes. The sample buffer is addressed by setting the most significant address bit to ‘1’. The sample buffer is divided in two equal parts, lower and upper, and the user will be notified when either is filled with audio data.

Sub-frame status bits can optionally be included in the sample buffer.

## SPDIF Transmitter

The transmitter architecture is shown below.

Wishbone Bus Interrupt SPDIF out Registers Phase encoder Channel status & user data buffers Frame assembler Sample buffer Sub-frame assembler Buffer copy

Figure 2: Transmitter Block Diagram

The sub-frame assembler creates 32bit data words from sample data, register settings and optionally channel status/user data buffers. A parity bit is added in each sub-frame. The frame assembler adds preambles to create a frame of two sub-frames. 192 frames add up to a block. The block structure is bi-phase encoded before transmitting.

The size of the sample buffer is determined by the Wishbone address bus width. Minimum sample buffer size is 128bytes. The sample buffer is addressed by setting the most significant address bit to ‘1’. The sample buffer is divided in two equal parts, lower and upper, and the user will be notified when either is emptied of audio data.

Channel status can be generated from a dedicated 192bit buffer. Two interrupts can be generated when the transmitter reads from the buffer, one in the middle and one at the end. The user data buffer operates in an identical way.

Operation

This chapter contains operational guidelines for the cores.

## Receiver

Some features are not available when the core is synthesized in 16bit mode.

#### Resetting

Except for the Wishbone reset, the receiver can be disabled by clearing the RXEN bit in the RxConfig register.

#### Capture registers

Before the receiver is enabled, the capture registers should be set up to capture bit-fields of interest, like sample frequency (bit 24-27 in consumer mode).

#### Enable receiver

Set RXEN bit in RxConfig register to enable the receiver. Then wait until the LOCK bit is set in the RxStatus Register. Examine the other status bits in RxStatus register to identify the type of signal being received, and set up the other bits in the RxConfig register. Finally set the SAMPLE bit to start data transfer to the sample buffers.

#### Transferring data

The best way to read out data is to set up an interrupt to be generated when the lower or upper buffer is full, and then execute a block read of the data.

## Transmitter

#### Resetting

Except for the Wishbone reset, the transmitter can be disabled by clearing the TXEN bit in the TxConfig register

#### Selecting transmit data rate

The data rate of the SPDIF signal is a function of the Wishbone bus clock and the RATIO bits in the TxConfig register. The bit rate is 64 times the sampling frequency – each sample is encoded as 32bits and there are two channels. Sample frequency is given by the following equation:

Sfreq=Wishboneclockfrequency128⋅(RATIO+1)

Example: Data rate is 48kHz and Wishbone clock frequency is 12.288MHz. The RATIO bits must then be set to 1. Output data rate is 3.072Mbps.

#### Selecting data format

If the Wishbone data bus is 16bit, it is only possible to send 16bit audio data. In 32bit mode, any sample resolution from 16 to 24bit can be transmitted. Data format is selected by the MODE bits in the TxConfig register.

#### Setting up channel status bits

If output format is standard consumer audio, set CHSTEN to 0 in TxConfig, and set TxChStat to desired format. Otherwise set up the ChStatus buffer with desired channel status data (192bits). If the channel status bits are not changing from block to block, it is only necessary to program the buffer once. Otherwise the HCSBF/LCSBF bits in TxIntMask must be set, and the buffer will need to be updated for every half block transmitted.

#### Setting up user data bits

User data bits are normally set to zero, but if required user data can be transmitted using the UserData buffer. If the user data bits are not changing from block to block, it is only necessary to program the buffer once. Otherwise the HCSBF/LCSBF bits in TxIntMask must be set, and the buffer will need to be updated for every block.

#### Preparing sample buffer

Before the TXDATA bit in TxConfig is set, fill up the complete sample buffer with audio data. The transmitter will generate an interrupt when lower half or upper half of sample buffer is emptied,

#### Start transmission

Transmission of SPDIF signal starts when the TXEN bit in TxConfig is set. If TXDATA bit is not set, the transmitted data will be all zeroes with the sub-frame validity bit set. Once the TXDATA bit is set, audio data from the sample buffer will be transmitted and the validity bit is cleared.

Generics

The SPDIF interface has a number of generics that can be used to tailor the interface for various needs.

## Generics for both transmitter and receiver

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| DATA_WIDTH | Integer | 16/32 | Wishbone data bus width. If using 16bit bus, some functionality is lost. |
| ADDR_WIDTH | Integer | 8 - 64 | Wishbone addresses bus width. The sample buffer occupies half the address range. |

Table 1: Generics for transmitter and receiver

## Generics for the receiver

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| CH_ST_CAPTURE | Integer | 0 – 8 | Specifies the number of channel status capture registers. Only applicable in 32bit mode. |
| WISHBONE_FREQ | Natural | 1 - | WishBone bus frequency in MHz.. This generic is used to optimize the phase detector. |

Table 2: Generics for receiver

## Generics for the transmitter

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| USER_DATA_BUF | Integer | 0 – 1 | 0: No user data buffer is generated 1: User data buffer is generated |
| CH_STAT_BUF | Integer | 0 – 1 | 0: No channel status data buffer is generated 1: Channel status data buffer is generated |

Table 3: Generics for transmitter

Registers

This section specifies all internal registers of the SPDIF interface.

## SPDIF Receiver

#### Receiver registers - overview

| Name |  | Address | Width | Access | Description |
| --- | --- | --- | --- | --- | --- |
| RxVersion |  | 0x00 | 16/32 | R | Version register |
| RxConfig |  | 0x01 | 16/32 | RW | Configuration register |
| RxStatus |  | 0x02 | 16/32 | R | Signal Status Register |
| RxIntMask |  | 0x03 | 16/32 | RW | Interrupt mask register |
| RxIntStat |  | 0x04 | 16/32 | RW | Interrupt status register |
| The following registers are optional depending on the value of CH_ST_CAPTURE |  |  |  |  |  |
| ChStCap0 | 0x10 |  | 32 | RW | Channel status capture register 0 |
| ChStData0 | 0x11 |  | 32 | R | Channel status data register 0 |
| ChStCap1 | 0x12 |  | 32 | RW | Channel status capture register 1 |
| ChStData1 | 0x13 |  | 32 | R | Channel status data data register 1 |
| ChStCap2 | 0x14 |  | 32 | RW | Channel status capture register 2 |
| ChStData2 | 0x15 |  | 32 | R | Channel status data data register 2 |
| ChStCap3 | 0x16 |  | 32 | RW | Channel status capture register 3 |
| ChStData3 | 0x17 |  | 32 | R | Channel status data data register 3 |
| ChStCap4 | 0x18 |  | 32 | RW | Channel status capture register 4 |
| ChStData4 | 0x19 |  | 32 | R | Channel status data data register 4 |
| ChStCap5 | 0x1a |  | 32 | RW | Channel status capture register 5 |
| ChStData5 | 0x1b |  | 32 | R | Channel status data data register 5 |
| ChStCap6 | 0x1c |  | 32 | RW | Channel status capture register 6 |
| ChStData6 | 0x1d |  | 32 | R | Channel status data data register 6 |
| ChStCap7 | 0x1e |  | 32 | RW | Channel status capture register 7 |
| ChStData7 | 0x1f |  | 32 | R | Channel status data data register 7 |

Table 4: Receiver registers

#### RxVersion – Description

The version register allows the SW to read out all the parameter that’s was used to generate the receiver.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-20 | R | - | Unused |
| 19-16 |  | CAPNO | The value of CH_ST_CAPTURE |
| 15-12 |  | - | Unused |
| 11-5 |  | ADRW | The value of ADDR_WIDTH |
| 4 |  | DATW | 0: DATA_WIDTH is 16bit 1: DATA_WIDTH is 32bit |
| 3-0 |  | VER | SPDIF Version number = 1 |

Reset Value:

RxVersion: Depends on generics

#### RxConfig – Description

The configuration register controls the operation of the receiver.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-25 | R | - | Unused |
| 24 | RW | BLKEN | 0: Do not use block boundary marking 1: Mark the first sample in each block with a 1 in bit 27. |
| 23-20 | RW | MODE | 0: Store samples as 16bit 1: Store samples as 17bit 2: Store samples as 18bit 3: Store samples as 19bit 4: Store samples as 20bit 5: Store samples as 21bit 6: Store samples as 22bit 7: Store samples as 23bit 8: Store samples as 24bit 9-15: Reserved |
| 19 |  | PAREN | 0: Do not store parity bit 1: Store parity bit in bit 31 in sample buffer |
| 18 |  | STATEN | 0: Do not store channel status bit 1: Store channel status bit in bit 30 in sample buffer |
| 17 |  | USEREN | 0: Do not store user data bit 1: Store user data bit in bit 29 in sample buffer |
| 16 |  | VALEN | 0: Do not store validity bit 1: Store validity bit in bit 28 in sample buffer |
| 15-5 | R | - | Unused |
| 4 | RW | VALID | 0: Sample data stored in buffers regardless of sub-frame Validity bit 1: Sample data stored only when sub-frame Validity bit is 0 |
| 3 |  | CHAS | 0: RxStatus register holds status from channel B 1: RxStatus register holds status from channel A |
| 2 |  | RINTEN | 0: Interrupt output is disabled 1: Interrupt output is enabled |
| 1 |  | SAMPLE | 0: No data is stored in the sample buffer 1: Data is stored in the sample buffer |
| 0 |  | RXEN | 0: Receiver is disabled 1: Receiver is enabled |

Reset Value:

RxConfig: 0x0000

#### RxStatus – Description

The signal status register holds information about the received signal. When the receiver is disabled, all bits read 0. When lock bit is 0, all other bits are set to 0.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-7 |  | - | Unused |
| 6 |  | COPY | 0: Copy inhibited 1: Copy permitted (copy bit only applicable in consumer mode) |
| 5-3 |  | EMPH | Emphasis code from the channel status block. Note that the interpretation of these bits is different in consumer and professional mode. |
| 2 |  | AUDIO | 0: Signal is non-audio 1: Signal is audio |
| 1 |  | PRO | 0: Signal format is consumer 1: Signal format is professional |
| 0 |  | LOCK | 0: Receiver has no valid input signal 1: Receiver is locked to SPDIF signal |

Reset Value:

RxStatus: 0x0000

#### RxIntMask – Description

Set bit to 1 in the mask register to enable an interrupt source.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 23 | RW | CAP7 | Capture register 7 interrupt mask |
| 22 |  | CAP6 | Capture register 6 interrupt mask |
| 21 |  | CAP5 | Capture register 5 interrupt mask |
| 20 |  | CAP4 | Capture register 4 interrupt mask |
| 19 |  | CAP3 | Capture register 3 interrupt mask |
| 18 |  | CAP2 | Capture register 2 interrupt mask |
| 17 |  | CAP1 | Capture register 1 interrupt mask |
| 16 |  | CAP0 | Capture register 0 interrupt mask |
| 15-5 | R | - | Unused |
| 4 | RW | PARITYB | Parity error channel B interrupt mask |
| 3 |  | PARITYA | Parity error channel A interrupt mask |
| 2 |  | HSBF | Higher sample buffer full interrupt mask |
| 1 |  | LSBF | Lower sample buffer full interrupt mask |
| 0 |  | LOCK | Change in lock bit interrupt mask |

Reset Value:

RxIntMask: 0x0000

#### RxIntStat – Description

A bit in this register is set to 1 when an event occurs. If the corresponding bit in RxIntMask is set to 1, an interrupt is generated (if enabled). Write a 1 to a bit to clear the event. The interrupt signal goes inactive when all events have been cleared.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-24 | R | - | Unused |
| 23 | RW | CAP7 | Capture register 7 changed |
| 22 |  | CAP6 | Capture register 6 changed |
| 21 |  | CAP5 | Capture register 5 changed |
| 20 |  | CAP4 | Capture register 4 changed |
| 19 |  | CAP3 | Capture register 3 changed |
| 18 |  | CAP2 | Capture register 2 changed |
| 17 |  | CAP1 | Capture register 1 changed |
| 16 |  | CAP0 | Capture register 0 changed |
| 15-5 | R | - | Unused |
| 4 | RW | PARITYB | Parity error detected on channel B |
| 3 |  | PARITYA | Parity error detected on channel A |
| 2 |  | HSBF | Higher sample buffer full |
| 1 |  | LSBF | Lower sample buffer full |
| 0 |  | LOCK | Change in lock bit in RxStatus register |

Reset Value:

RxIntStat: 0x0000

#### ChStCap<n> – Description

The channel status capture register can be set up to capture a specified bit-field of the channel status frame or user data frame, and also generate an interrupt when the bit-field changes. The bit-field can be from 1 to 32 bits long. There can be up to eight sets of bit-field capture registers.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-8 | RW | BITPOS | First bit position of bit-field to be captured Valid range is 0 to 191 |
| 7 | RW | CDATA | 0: User data is captured 1: Channel status data is captured |
| 6 | RW | CHID | 0: Source is channel A 1: Source is channel B |
| 5-0 | RW | BITLEN | 0: Capture function disabled 1-32: Length of bit-field to be captured |

Reset Value:

ChStCap: 0x0000

#### ChStData<n> – Description

Captured channel status/user data bits.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-0 | R | DATA | Captured channel status. First bit captured is stored in bit 0. |

Reset Value:

ChStData: 0x0000

#### Receive sample data – Description

Format of data words in receive sample buffer

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31 | R | PARITY | Parity bit (if enabled, otherwise 0) |
| 30 |  | CHSTAT | Channel status bit (if enabled, otherwise 0) |
| 29 |  | USRDAT | User Data bit (if enabled, otherwise 0) |
| 28 |  | VALID | Validity bit (if enabled, otherwise 0) |
| 27 |  | BLKSTART | Start of sample block bit (if enabled, otherwise 0) |
| 26-24 |  | - | Unused |
| 23-16 |  | DATH | Audio data if >16bit resolution. Unused bits are 0 |
| 15-0 |  | DATL | Audio data. Bit 0 is LSB. |

Reset Value:

Sample buffer: undefined

## SPDIF Transmitter

#### Transmitter registers - overview

| Name |  | Address | Width | Access | Description |
| --- | --- | --- | --- | --- | --- |
| TxVersion |  | 0x00 | 16/32 | R | Version register |
| TxConfig |  | 0x01 | 16/32 | RW | Configuration register |
| TxChStat |  | 0x02 | 16/32 | RW | Channel status control register |
| TxIntMask |  | 0x03 | 16/32 | RW | Interrupt mask register |
| TxIntStat |  | 0x04 | 16/32 | RW | Interrupt status register |
| The following registers are optional depending on the value of USER_DATA_BUF |  |  |  |  |  |
| UserData | 0x20-0x37 |  | 16/32 | W | User data buffer |
| The following registers are optional depending on the value of CH_STAT_BUF |  |  |  |  |  |
| ChStatus | 0x40-0x57 |  | 16/32 | W | Channel status buffer |

Table 5: Transmitter registers

#### TxVersion – Description

The version register allows the SW to read out all the parameter that’s was used to generate the transmitter.

| Bit # | Access | Description |
| --- | --- | --- |
| 31-14 | R | Unused |
| 13 | R | 0: Channel status buffer not available 1: Channel status buffer available |
| 12 | R | 0: User data buffer not available 1: User data buffer available |
| 11-5 | R | The value of ADDR_WIDTH |
| 4 | R | 0: DATA_WIDTH is 16bit 1: DATA_WIDTH is 32bit |
| 3-0 | R | SPDIF Version number = 1 |

Reset Value:

TxVersion: -

#### TxConfig – Description

The configuration register controls the operation of the transmitter. When DataWidth=16, only 16bit samples can be transmitted.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-24 | R | - | Unused |
| 23-20 | RW | MODE | Sample format: 0: 16bit 1: 17bit 2: 18bit 3: 19bit 4: 20bit 5: 21bit 6: 22bit 7: 23bit 8: 24bit 9-15: Reserved |
| 19-16 | R | - | unused |
| 15-8 | RW | RATIO | Clock divider for the transmit frequency. The Wishbone bus clock is divided by a factor of (1+RATIO) to generate the serial transmit clock. |
| 7-6 |  | UDATEN | 0: User data A&B set to 0. 1: User data A&B generated from UserData bit 7-0 2: User data A generated from UserData bit 7-0, B generated from UserData bit 15-8 3: reserved |
| 5-4 |  | CHSTEN | 0: Channel status A&B generated from TxChStat 1: Channel status A&B generated from ChStat bit 7-0 2: Channel status A generated from ChStat bit 7-0, B generated from ChStat bit 15-8 3: reserved |
| 3 | R | - | unused |
| 2 | RW | TINTEN | 0: Interrupt output is disabled 1: Interrupt output is enabled |
| 1 |  | TXDATA | 0: Data from buffer not transmitted (validity bit set to 1) 1: Data from buffer transmitted (validity bit set to 0) |
| 0 |  | TXEN | 0: Transmitter is disabled 1: Transmitter is enabled |

Reset Value:

TxConfig: 0x0000

#### TxChStat – Description

The transmit channel status register can be used to set the most important bits of the 192bit channel status frame when the channel status buffer is not enabled. This register only applies to consumer mode. Professional mode requires the channel status buffer.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-8 | R | - | Unused |
| 7-6 | RW | FREQ | Sample frequency: 00: 44.1kHz 01: 48kHz 10: 32kHz 11: Sample rate converter |
| 5-4 | R | - | unused |
| 3 | RW | GSTAT | Generation status: (category code is set to 00) 0: No Indication 1: Original/commercially pre-recoded data |
| 2 | RW | PREEM | Pre-emphasis: 0: None 1: 50/15us |
| 1 | RW | COPY | Copyright: 0: Copy inhibited 1: Copy permitted |
| 0 | RW | AUDIO | Data format: 0: Audio 1: Non-audio |

Reset Value:

TxChStat: 0x0000

#### TxIntMask – Description

Set bit to 1 in the mask register to enable an interrupt source.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-5 | R | - | Unused |
| 4 | RW | HCSBF | Higher channel status/user data buffer empty |
| 3 |  | LCSBF | Lower channel status/user data buffer empty |
| 2 |  | HSBF | Higher sample buffer empty |
| 1 |  | LSBF | Lower sample buffer empty |
| 0 | R | - | Unused |

Reset Value:

TxIntMask: 0x0000

#### TxIntStat – Description

A bit in this register is set to 1 when an event occurs. If the corresponding bit in TxIntMask is set to 1, an interrupt is generated (if enabled). Write a 1 to a bit to clear the event. The interrupt signal goes inactive when all events have been cleared.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | R | - | Unused |
| 15-5 | R | - | Unused |
| 4 | RW | HCSBF | Higher channel status/user data buffer empty |
| 3 |  | LCSBF | Lower channel status/user data buffer empty |
| 2 |  | HSBF | Higher sample buffer empty |
| 1 |  | LSBF | Lower sample buffer empty |
| 0 | R | - | Unused |

Reset Value:

TxIntStat: 0x0000

#### UserData – Description

The user data is 24 bytes per sample block. Bit 0 of byte 0 is transmitted first. Bit 7 of byte 23 is transmitted last.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | - | - | Unused |
| 15-8 | W | CHBUD | Channel B user data |
| 7-0 |  | CHAUD | Channel A user data |

Reset Value:

UserData: undefined

#### ChStat – Description

The channel status data is 24 bytes per sample block. Bit 0 of byte 0 is transmitted first. Bit 7 of byte 23 is transmitted last.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-16 | - | - | Unused |
| 15-8 | W | CHACS | Channel B channel status data |
| 7-0 |  | CHACS | Channel A channel status data |

Reset Value:

ChStat: undefined

#### Transmit sample data – Description

Format of data words in transmit sample buffer. Channel A is transmitted first, and must be stored on even addresses, while channel B is stored on odd addresses in the sample buffer.

| Bit # | Access | Name | Description |
| --- | --- | --- | --- |
| 31-24 | - | - | Unused |
| 23-16 | W | DATH | Audio data if >16bit resolution. Unused bits are 0 |
| 15-0 |  | DATL | Audio data (16bit mode). Bit 0 is LSB. |

Reset Value:

Sample buffer: undefined

Clocks

The SPDIF interface uses only the Wishbone interface clock.

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |
| wb_clk_i | - | - |  | - | Must be at least 8 times higher than SPDIF data rate | System clock. |

Table 6: List of clocks

IO Ports

## Wishbone interface signals (receiver & transmitter)

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| wb_ack_o | 1 | Output | Bus cycle acknowledge |
| wb_adr_i | 8-64 | Input | Address bus |
| wb_bte_i | 2 | Input | Burst type extension |
| wb_clk_i | 1 | Input | Clock |
| wb_cti_i | 3 | Input | Cycle type identifier |
| wb_cyc_i | 1 | Input | Valid bus cycle |
| wb_dat_i | 16/32 | Input | Data to core |
| wb_dat_o | 16/32 | Output | Data from core |
| wb_rst_i | 1 | Input | Asynchronous reset |
| wb_sel_i | 1 | Input | Select Input |
| wb_stb_i | 1 | Input | Strobe |
| wb_we_I | 1 | Input | Write enable |

Table 7: Wishbone signals

| Description | Specification |
| --- | --- |
| General description | 16/32bit slave |
| Supported cycles | SLAVE, READ/WRITE SLAVE, BLOCK READ/WRITE SLAVE, Incrementing burst cycle (linear) |
| Data port, size Data port, granularity Data port, maximum operand size Data transfer ordering Data transfer sequencing | 16 or 32bit 16 or 32bit 16 or 32bit Big/little endian Undefined |

Wishbone properties

## SPDIF Receiver signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| spdif_rx_i | 1 | Input | SPDIF bi-phase encoded input signal |
| rx_int_o | 1 | Output | Interrupt signal (active high) |

Table: Receiver signals

## SPDIF Transmitter signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| spdif_tx_o | 1 | Output | SPDIF bi-phase encoded output signal |
| tx_int_o | 1 | Output | Interrupt signal (active high) |

Table: Transmitter signals