WB2HPI

Authors: 
Gvozden Marinkovic, gvozden@opencores.org 
Dusko Krsmanovic, dusko@opencores.org

Rev. 0.1

July 31, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 12/31/02 | Gvozden Marinkovic Dusko Krsmanovic | First Draft |

Contents

Introduction	1

DSP Host Port Interface	1

Architecture	2

Examples of usage	3

Write data to DSP HPI registers	3

Read data from DSP HPI registers	5

Block transfer from PC system memory to DSP Address Space	7

Block transfer from DSP Address Space to PC system memory	8

Access to DSP Memory Space	9

Interrupts	10

Restart of DSP	10

Registers	11

List of Registers	11

IR	11

IM	12

LED	12

HPI_COM	13

BUF_ADR	13

HPI_READ	14

CONTROL	14

STATUS	15

Clocks	16

IO Ports	17

WISHBONE Common Signals	17

WISHBONE Slave Signals	17

WISHBONE Master Signals	17

DSP HPI Signals	18

LED Signals	18

Appendix A	19

Appendix B	20

Index	21

Introduction

### DSP Host Port Interface

Architecture

Examples of usage

Note 0: After power-on DSP is in reset state. (DSP) Int2 should be connected to the wb2hpi core DSP_INT2_PAD_O  for boot loader activation.

Note 1: Examples below are simplified (assumption is that only one card exists in system) functions for controlling wb2hpi core connected with OpenCores PCI bridge.

Note 2: WriteDword(...) and ReadDword(...) are environment specific functions that implements write/read DWORD to/from PCI card.

Note 3: WriteWord(...) and ReadWord(...) are environment specific functions that implements write/read WORD to/from PCI card.

### Write data to DSP HPI registers

Write WORD to HPI address register

void WriteWordToHPIA(WORD data) { DWORD temp; // data to be written is in lower 16 bits temp = (DWORD) data & 0x0000ffff; // nibble 5 is 2 to indicate that HPIA is destination // nibble 4 is 2 to indicate write operation temp = temp ¦ 0x00220000; // Write command to HPI_COMMAND register WriteDword(BAR1_SPACE,DSP_HPI_COMMAND_OFFSET, temp); }

Write WORD to HPI control register

void WriteWordToHPIC(WORD data) { DWORD temp; // data to be written is in lower 16 bits temp = (DWORD) data & 0x0000ffff; // nibble 5 is 0 to indicate that HPIC is destination // nibble 4 is 2 to indicate write operation temp = temp ¦ 0x00020000; // Write command to HPI_COMMAND register WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, temp); }

Write WORD to HPI data register:

void WriteWordToHPID(WORD data) { DWORD temp; // data to be written is in lower 16 bits temp = (DWORD) data & 0x0000ffff; // nibble 5 is 3 to indicate that HPID is destination // nibble 4 is 2 to indicate write operation temp = temp ¦ 0x00320000; // Write command to HPI_COMMAND register WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, temp); }

### Read data from DSP HPI registers

Read WORD from HPI address register

WORD ReadWordFromHPIA() { DWORD data; // nibble 5 is 2 to indicate that HPIA is destination // nibble 4 is 3 to indicate read operation data = 0x00230000; // Write command to HPI_COMMAND register WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, data); // After completion of read operation data is in // lower 16 bits of HPI_READ register return (WORD) ReadDword(BAR1_SPACE, DSP_HPI_READ_OFFSET); }

Read WORD from HPI control register

WORD ReadWordFromHPIC() { DWORD data; // nibble 5 is 0 to indicate that HPIC is destination // nible 4 is 3 to indicate read operation data = 0x00030000; // Write command to HPI_COMMAND register WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, data); // After completion of read operation data is in // lower 16 bits of HPI_READ register return (WORD) ReadDword(BAR1_SPACE, DSP_HPI_READ_OFFSET); }

Read WORD from HPI data register

WORD ReadWordFromHPID() { DWORD data; // nibble 5 is 3 to indicate that HPID is destination // nible 4 is 3 to indicate read operation data = 0x00330000; // Write command to HPI_COMMAND register WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, data); // After completion of read operation data is in // lower 16 bits of HPI_READ register return (WORD) ReadDword(BAR1_SPACE, DSP_HPI_READ_OFFSET); }

### Block transfer from PC system memory to DSP Address Space

| Parameters | Description |
| --- | --- |
| buffdsp | where to copy block in DSP memory (DSP memory has WORD granularity) |
| buffmemph | PHYSICAL MEMORY pointer to buffer in system memory (!!!!!!!!!!!! NOT VIRTUAL MEMORY) |
| count | number of WORDs to transfer |

void BlockTransferMem2Dsp(DWORD buffmemph,WORD buffdsp, WORD count) { // In autoincrement mode DSP increments HPIA first, // next line is for correction of address DWORD cmd = buffdsp - 1; // Write physical memory address of buffer in BUFFER_ADDRESS_REGISTER WriteDword(BAR1_SPACE, BUFFER_ADDRES_OFFSET, buffmemph); //In next four lines is initiation of OpenCores PCI bridge buffmemph &= 0xffff0000; WriteDword(BAR0_SPACE, PCI_WBA1_OFFSET, buffmemph); WriteDword(BAR0_SPACE, PCI_WAM1_OFFSET, 0xffff0000); WriteDword(BAR0_SPACE, PCI_WIMGCTRL1,0x00000000); //Write corected DSP buffer start address in HPIA cmd = cmd & 0x0000ffff; cmd = cmd ¦ 0x00220000; WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, cmd); //Write number of words to transfer in BUFFER_COUNT register WriteDword(BAR1_SPACE, BUFFER_COUNT_OFFSET, count); //Start transfer DSP->MEM WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, 0x00050000); }

Note 4: After completion of the transfer interrupt will be generated.

### Block transfer from DSP Address Space to
PC system memory

| Parameters | Description |
| --- | --- |
| buffdsp | start of block in DSP memory from where block will be transferred (DSP memory has WORD granularity) |
| buffmemph | PHYSICAL MEMORY pointer to buffer in system memory (!!!!!!!!!!!! NOT VIRTUAL MEMORY) |
| count | number of WORDs to transfer |

void BlockTransferDsp2Mem(WORD buffdsp, DWORD buffmemph, WORD count) { DWORD cmd = buffdsp; //Write physical memory address of buffer in BUFFER_ADDRESS_REGISTER WriteDword(BAR1_SPACE, BUFFER_ADDRES_OFFSET, buffmemph); //In next four lines is initiation of OpenCores PCI bridge buffmemph &= 0xffff0000; WriteDword(BAR0_SPACE, PCI_WBA1_OFFSET, buffmemph); WriteDword(BAR0_SPACE, PCI_WAM1_OFFSET, 0xffff0000); WriteDword(BAR0_SPACE, PCI_WIMGCTRL1, 0x00000000); //Write DSP buffer start address in HPIA cmd = cmd & 0x0000ffff; cmd = cmd ¦ 0x00220000; WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, cmd); //Write number of words to transfer in BUFFER_COUNT register WriteDword(BAR1_SPACE, BUFFER_COUNT_OFFSET, count); //Start transfer DSP->MEM WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, 0x00040000); }

Note 5: Interrupt will be generated after completion of the transfer.

### Access to DSP Memory Space

Read single WORD from DSP memory space

WORD ReadWordFromDSP (WORD address) { DWORD temp; temp =(DWORD) address & 0x0000ffff; // DSP memory has word granularity , // in mapped address space address must be WORD aligned temp = temp << 1; temp = temp ¦ 0x000f0000; return ReadWord(BAR1_SPACE, temp); }

Write single WORD to DSP memory space

void WriteWordToDSP (WORD data, WORD address) { DWORD temp; temp = (DWORD) address & 0x0000ffff; // DSP memory has word granularity , // in mapped address space address must be WORD aligned temp = temp << 1; temp = temp ¦ 0x000f0000; WriteWord(BAR1_SPACE, temp, data); }

### Interrupts

Enabling interrupts

void IntEnable () { // Enabling interrupts in OpenCores PCI bridge WriteDword(BAR0_SPACE, PCI_ICR_OFFSET, 0x00000001); // Enabling interrupts in WB2HPI core WriteDword(BAR1_SPACE, IRQ_IM_OFFSET, 0x00000001); }

Disabling interrupts

void IntDisable () { // Disabling interrupts in OpenCores PCI bridge WriteDword(BAR0_SPACE, PCI_ICR_OFFSET, 0x00000000); // Disabling interrupts in WB2HPI core WriteDword(BAR1_SPACE, IRQ_IM_OFFSET, 0x00000000); }

### Restart of DSP

ActivateBootLoader

void ActivateBootLoader() { //RESET DSP and wb2hpi ON WriteDword(BAR1_SPACE, CONTROL_OFFSET, 0x00000003); //RESET DSP and wb2hpi OFF WriteDword(BAR1_SPACE, CONTROL_OFFSET, 0x00000000); //Initiation of HPIC (byte order on hpi) WriteDword(BAR1_SPACE, DSP_HPI_COMMAND_OFFSET, 0x00020808); }

Note 6: If you wish to load and execute DSP firmware you should:

- Activate BOOT loader

- Write firmware in DSP memory

- Execute firmware by writing start address on DSP location 0x007f

Registers

### List of Registers

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |
| IR | 0x10000 | 32 | R/W | Interrupt Request Register |
| IM | 0x20000 | 32 | R/W | Interrupt Mask Register |
| LED | 0x70000 | 32 | R/W | Test LED Register |
| HPI_COM | 0x80000 | 32 | R/W | HPI Command Register |
| BUF_ADR | 0x90000 | 32 | R/W | Buffer Address (block transfer) |
| HPI_READ | 0xA0000 | 32 | R | Data read from DSP |
| BUF_CNT | 0xB0000 | 32 | R/W | Number of bytes to transfer (block transfer) |
| CONTROL | 0xC0000 | 32 | R/W | DSP Control Register |
| STATUS | 0xD0000 | 32 | R | DSP Status Register |
| DSP Memory Space | 0xFxxxx | 16 | R/W | DSP Memory Space |

Table 1: List of registers

### IR

| Width | Access | Reset | Description |
| --- | --- | --- | --- |
| 32 | R/W | 0000h | Interrupt Request Register |

Register layout:

| 31 |  |  |  |  |  |  | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 |  |  |  |  |  |  | 16 |
| 15 |  |  |  |  |  |  | 8 |
| 7 |  |  |  |  | 2 | 1 | 0 |
|  |  |  |  |  |  | HINT | EBTI |

Bit descriptions:

| Bit # | Name | Description |
| --- | --- | --- |
| 31 - 2 | N/A | Not used |
| 1 | Host Port Interrupt | Interrupt generated directly from DSP |
| 2 | End Block Transfer Interrupt | Interrupt generated when Block Transfer ends |

Table 2: Interrupt Request Register

Write 0x1 to Interrupt Request register to clear End Block Transfer Interrupt.

Write 0x2 to Interrupt Request register to clear Host Port Interrupt.

Write 0x3 to Interrupt Request register to clear both.

### IM

| Width | Access | Reset | Description |
| --- | --- | --- | --- |
| 32 | R/W | 0000h | Interrupt Mask Register |

Register layout:

| 31 |  |  |  |  |  |  | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 |  |  |  |  |  |  | 16 |
| 15 |  |  |  |  |  |  | 8 |
| 7 |  |  |  |  | 2 | 1 | 0 |
|  |  |  |  |  |  | HINTM | EBTIM |

Bit descriptions:

| Bit # | Name | Description |
| --- | --- | --- |
| 31 - 2 | N/A | Not used |
| 1 | Host Port Interrupt Mask | Masks interrupt generated directly from DSP |
| 2 | End Block Transfer Interrupt Mask | Mask interrupt generated when Block Transfer ends |

Table 3: Interrupt Mask Register

### LED

| Width | Access | Reset | Description |
| --- | --- | --- | --- |
| 32 | R/W | 0000h | LED driver register |

Register layout:

| 31 |  |  |  |  |  |  | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 |  |  |  |  |  |  | 16 |
| 15 |  |  |  |  |  |  | 8 |
| 7 |  |  |  |  | 2 | 1 | 0 |
|  |  |  |  |  |  |  | LED |

Bit descriptions:

| Bit # | Name | Description |
| --- | --- | --- |
| 1 | LED | Turns test LED On/Off |

Table 4: LED register

### HPI_COM

| Width | Access | Reset | Description |
| --- | --- | --- | --- |
| 32 | R/W | 0000h | HPI Command Register |

Register layout:

| 31 |  |  |  |  |  |  | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 |  |  | 20 | 19 |  |  | 16 |
| HPI_REGISTER |  |  |  | HPI_COMMAND |  |  |  |
| 15 |  |  |  |  |  |  | 8 |
| DATA |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  | 0 |
| DATA |  |  |  |  |  |  |  |

Bit descriptions:

| Bit # | Name | Description |
| --- | --- | --- |
| 31 – 24 | N/A | Not used |
| 23 - 20 | HPI Registers (DSP) | HPI Register: - 0000 – HPI Control Register - 0001 – HPI Data register - 0010 – HPI Address Register - 0011 – HPI Data Register – Auto Increment |
| 19 - 16 | HPI Command | HPI Command: - 0010 – Write data to DSP - 0011 – Read data from DSP - 0100 – Read block from DSP - 0101 – Write block to DSP |
| 15 - 0 | Data (write) | Data to write |

Table 5: HPI Command Register

### BUF_ADR

| Width | Access | Reset | Description |
| --- | --- | --- | --- |
| 32 | R/W | 0000h | Data Buffer Address |

Register layout:

| 31 |  |  |  |  |  |  | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BUF_ADR |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  | 16 |
| BUF_ADR |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  | 8 |
| BUF_ADR |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  | 0 |
| BUF_ADR |  |  |  |  |  |  | 0 |

Bit descriptions:

| Bit # | Name | Description |
| --- | --- | --- |
| 31 – 0 | Buffer Address | Buffer Address |

Table 6: Buffer Address

### HPI_READ

| Width | Access | Reset | Description |
| --- | --- | --- | --- |
| 32 | R/W | 0000h | HPI Read Register |

Register layout:

| 31 |  |  |  |  |  |  | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 |  |  |  |  |  |  | 16 |
| 15 |  |  |  |  |  |  | 8 |
| DATA |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  | 0 |
| DATA |  |  |  |  |  |  |  |

Bit descriptions:

| Bit # | Name | Description |
| --- | --- | --- |
| 31 – 16 | N/A | Not used |
| 15 - 0 | HPI Read | Data read thought HPI |

Table 7: HPI Read Register

### CONTROL

| Width | Access | Reset | Description |
| --- | --- | --- | --- |
| 32 | R/W | 0002h | DSP Control Register |

Register layout:

| 31 |  |  |  |  |  |  |  |  |  | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 |  |  |  |  |  |  |  |  |  | 16 |
| 15 |  |  |  |  |  |  |  |  |  | 8 |
| 7 |  |  |  |  | 2 |  | 1 |  | 0 |  |
|  |  |  |  |  |  |  | DSP_RST |  | HPI_RST |  |

Bit descriptions:

| Bit # | Name | Description |
| --- | --- | --- |
| 31 – 2 | N/A | Not used |
| 1 | DSP_RST | Reset DSP |
| 0 | HPI_RST | Reset HPI Interface |

Table 8: DSP Control Register

### STATUS

| Width | Access | Reset | Description |
| --- | --- | --- | --- |
| 32 | R/W | 0000h | DSP Status Register |

Register layout:

| 31 |  |  |  |  |  |  |  |  | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 |  |  |  |  |  |  |  |  | 16 |
| 15 |  |  |  |  |  |  |  |  | 8 |
| 7 | 6 | 5 |  |  |  |  |  | 0 |  |
|  |  | DSP_HINT | DSP_RDY | PCI_ERROR |  | READY | END_BT | DATA_RDY |  |

Bit descriptions:

| Bit # | Name | Description |
| --- | --- | --- |
| 31 – 6 | N/A | Not used |
| 5 | DSP_HINT | Host Port Interrupt (from DSP) |
| 4 | DSP_RDY | DSP is Ready |
| 3 | PCI_ERROR | PCI Error (Bus Mustering) |
| 2 | READY | HPI Interface is Ready |
| 1 | END_BT | End of Block Transfer |
| 0 | DATA_RDY | Data is Ready (after read from DSP) |

Table 9: DSP Status Regisatr

Clocks

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |

Table 10: List of clocks

IO Ports

### WISHBONE Common Signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| WB_CLK_I | 1 | Input | WISHBONE Clock |
| WB_RST_I | 1 | Input | WISHBONE Reset |

### WISHBONE Slave Signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| WBS_CYC_I | 1 | Input | Cycle in progress |
| WBS_STB_I | 1 | Input | Strobe |
| WBS_WE_I | 1 | Input | Write enable |
| WBS_CAB_I | 1 | Input | Not used |
| WBS_ADR_I | 32 | Input | Address |
| WBS_DAT_I | 32 | Input | Data input |
| WBS_SEL_I | 4 | Input | Select |
| WBS_ACK_O | 1 | Output | Acknowledge |
| WBS_ERR_O | 1 | Output | Error |
| WBS_RTY_O | 1 | Output | Retry |
| WBS_DAT_O | 32 | Output | Data output |

Table 11: List of IO ports

### WISHBONE Master Signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| WBM_ACK_I | 1 | Input | Acknowledge |
| WBM_ERR_I | 1 | Input | Error |
| WBM_RTY_I | 1 | Input | Retry |
| WBM_DAT_I | 32 | Input | Data input |
| WBM_CYC_O | 1 | Output | Cycle in progress |
| WBM_STB_O | 1 | Output | Strobe |
| WBM_WE_O | 1 | Output | Write enable |
| WBM_CAB_O | 1 | Output | Not used |
| WBM_ADR_O | 32 | Output | Address |
| WBM_DAT_O | 32 | Output | Data output |
| WBM_SEL_O | 4 | Output | Select |

### DSP HPI Signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| DSP_HAD_O | 8 | Output | HPI Address/Data Output |
| DSP_HAD_ENn | 1 | Output | HPI Address/Data Output Enable |
| DSP_RST_PAD_O | 1 | Output | Reset DSP |
| DSP_HCNTL_PAD_O | 2 | Output | HPI Register selection |
| DSP_HCS_PAD_O | 1 | Output | HPI Chip Select |
| DSP_HDS1_PAD_O | 1 | Output | HPI Data Strobe 1 |
| DSP_HDS2_PAD_O | 1 | Output | HPI Data Strobe 2 |
| DSP_HRW_PAD_O | 1 | Output | HPI Read/Write |
| DSP_HBIL_PAD_O | 1 | Output | HPI Byte High/Low |
| DSP_HAS_PAD_O | 1 | Output | HPI HAS |
| DSP_INT2_PAD_O | 1 | Output | HPI Interrupt request 2 |
| DSP_HAD_I | 8 | Input | HPI Address/Data Input |
| DSP_HRDY_PAD_I | 1 | Input | HPI Ready |
| DSP_HINT_PAD_I | 1 | Input | HPI Host Interrupt |

### LED Signals

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| LED | 1 | Output | LED driver |

Name

This section may be added to outline different specifications.

Name

This section may be added to outline different specifications.

This section contains an alphabetical list of helpful document entries with their corresponding page numbers.