Xgate Co-Processor (xgate)
Specification

Author: Robert Hayes

rehayes@opencores.org

Rev. 0.1

July 29, 2026

This page has been intentionally left blank.

Revision History

| Rev. | Date | Author | Description |
| --- | --- | --- | --- |
| 0.1 | 09/29/09 | Robert Hayes |  |

Contents

Introduction	1

FEATURES	1

Architecture	3

2.1 WISHBONE Slave Interface	4

2.2 Control Registers	4

2.3 Interrupt Interface	4

2.4 WISHBONE Master Interface	5

2.5 RISC Processor Core	5

Operation	10

3.1 Software Example	10

3.2 Software Triggers	11

3.3 Semaphore Bits	11

3.4 Program Memory Connection Options	11

3.5 Debug Mode	11

3.6 Instruction Set Summary	11

3.7 Inconsistent Instruction Set Documentation	11

Registers	13

List of Registers	13

4.1 Xgate Module Control Register (XGMCTL)	14

4.2 Xgate Channel ID Register (XGCHID)	17

4.3 Xgate Vector Base Address Register (XGVBR)	18

4.4 Xgate Interrupt Flag Register 7 (XGIFR_7)	18

4.12 Xgate Software Trigger Register (XGSWT)	19

4.13 Xgate Semaphore Register (XGSEM)	19

4.14 Xgate Condition Code Register (XGCCR)	20

4.15 Xgate Program Counter Register (XGPC)	21

4.16 Xgate Register 1 (XGR1)	21

4.23 Interrupt Bypass Register 0 (IRQ_BP0)	21

Clocks	23

IO Ports	24

6.1 WISHBONE Slave Interface	25

6.2 Xgate signals	27

6.3 Xgate Core Parameters	28

Appendix A	30

Instruction Set Details	30

Appendix B	104

Test Bench	104

Test Bench Overview	104

Top Level Test Bench	105

Host	105

Test Bench Regs	105

x.1 Check Point Register (CHECK_POINT)	106

x.2 Channel Acknowledge Register (CHANNEL_ACK)	107

x.3 Check Point Register (CHANNEL_ERR)	107

x.4 Breakpoint Control Register (BRKPT_CNTL)	108

x.5 Breakpoint Address Register (BRKPT_ADDR)	108

x.6 Test Bench Semaphore Register (TB_SEMPHORE)	109

x.7 Channel IRQ Register (CHANNEL_XGIRQ_0)	109

x.8 Channel IRQ Register (CHANNEL_XGIRQ_1)	110

RAM	110

Xgate	110

Arbitration	111

Sample Test Program	111

Index	116

Introduction

The Xgate Co-processor Module, Xgate, is a 16 bit programmable RISC processor that is managed by a host CPU to reduce the host load in handling interrupts.  Because the Xgate is user programmable there is a great deal of user control in how to preprocess data from peripheral modules. This module may be configured as a simple DMA controller to organize data such that the host only operates with whole messages and not individual words or bytes. The Xgate may also handle higher levels of messaging protocols than the peripheral hardware recognizes. Encryption algorithms are also supported by the instruction set.

The ideal application for the Xgate co-processor is in an ASIC environment where a specific host processor is required but the host as a standalone processor lacks sufficient computational resources to service the target application. Such a situation might occur in an existing application that has an extensive software base or the host processor has a significant history in similar applications. In this environment the Xgate provides the additional resources needed for the application while also providing the user programmability that may allow the ASIC to be used in other applications.

The use of an Xgate co-processor in an FPGA may not be as advantageous as in an ASIC environment because of the reprogramability of the FPGA hardware. Because the Xgate co-processor offers a generic solution to a variety of problems and the functionality can be changed by upgrading user software without changing the FPGA hardware implementation it may still be a valuable solution to certain problems. The use of the Xgate module may also reduce development time since the application can begin debug sooner and changes can be prototyped in user software.

### FEATURES

- Instruction set compatible with Freescale XGATE co-processor

- Handles up to 127 interrupt inputs

- Eight software triggerable interrupt channels.

- Eight semaphore registers to coordinate host/Xgate shared memory.

- Static synchronous design

- Fully synthesizable

Architecture

The Xgate core is built around five primary blocks; the WISHBONE Slave Interface, WISHBONE Master Interface, the Control Registers, Interrupt Priority, and RISC Processor Core.

Figure 2-1 Internal Xgate Structural Overview

xgif_o

chan_req_i

WISHBONE Slave Interface

Command Registers

RISC Core

Interrupt Priority

WISHBONE Master Interface

wbm_…

wbs_…

### 2.1 WISHBONE Slave Interface

The host processor uses this bus to access the Xgate control and status registers. The WISHBONE Slave Interface isolates the Xgate functionality from the WISHBONE bus. This interface takes the bus specific signals and generates a generic set of control signals to drive the Xgate control registers. Isolating the WISHBONE bus should help promote Xgate module reusability by localizing the scope of changes needed to retarget the Xgate module to another bus environment.

### 2.2 Control Registers

This module receives the generic write signals from the WISHBONE Slave Interface and applies these signals to the registers that the host processor uses to control Xgate operation.

### 2.3 Interrupt Interface

The Interrupt Interface takes the input interrupts from other slave peripheral modules and prioritizes them for processing by the Xgate RISC Processor Core. . The active interrupt input with the lowest index number becomes the chosen channel select enable.

#### 2.3.1 Simple Interrupt Interface

Previous versions of the Xgate module used this simple interrupt interface because it had the least hardware and least functionality. This module takes the 127 interrupt inputs and does a simple encoding to generate the six bit bus that becomes the active channel select signal

Because the Xgate module is in the signal path from the interrupt sources to the host CPU there must always be always be some code running in the Xgate module to activate the appropriate interrupt output to the CPU.

#### 2.3.2 Bypass Interrupt Interface

The current version of the Xgate module uses this implementation. This module does the same basic interrupt encoding as the “Simple Interrupt Interface” except it includes additional functionality to allow an interrupt input to be bypassed  around the Xgate module and connected directly to the host CPU. This will eliminate the latency and software overhead when it is required for the host CPU to process some interrupts directly while still reserving the option to enable the Xgate module to preprocess those same interrupt sources at some other time. To implement this functionality the IRQ_BP0 – IRQ_BP7 control registers are added to the WISHBONE slave interface.

#### 2.3.3 Programmable Interrupt Interface

This is a proposed functional improvement and is not currently implemented. This module will include the functionality of the “Bypass Interrupt Interface” with additional functionality to allow interrupt priorities to be set by the host CPU. This will eliminate the latency and software overhead when it is required for the host CPU to process some interrupts directly while still reserving the option to enable the Xgate module to preprocess those same interrupt sources at some other time. To implement this functionality additional control registers will need to be added to the WISHBONE slave interface.

### 2.4 WISHBONE Master Interface

The Xgate module uses the WISHBONE Master Interface to access the shared memory space where it’s software code, data, and other slave peripheral registers may be accessed. The WISHBONE Master Interface isolates the Xgate functionality from the WISHBONE bus. This interface takes the bus specific signals and generates a generic set of control signals to drive the Xgate RISC Processor. Isolating the WISHBONE bus should help promote Xgate module reusability by localizing the scope of changes needed to retarget the Xgate module to another bus environment.

### 2.5 RISC Processor Core

The RISC Processor Core is the key element of the Xgate module. This module implements the Xgate instruction set.

#### 2.5.1 Hardware Model

Address

ALU Result

rs1_data

rd_data

rs2_data

Address Reg

WISHBONE Master Bus

Data In

Data Out

Address

Inst Decode

Op Code

PC

R1

R2

R3

R4

R5

R6

R7

R0

RISC SM

#### 2.5.2 State Machine

idle

boot_1

boot_2

w_stall

decode

boot_3

Output address for PC initial address data fetch

Load PC

Output address for initial variable pointer fetch

Load initial variable pointer (R1)

Output PC

Load instruction

Increment PC

Load instruction

Increment PC

RTS

BRK

s_stall

b_stall

break

break_2

ld_inst

debug

Idle: The idle state is the condition where the xgate RISC processor waits to be enabled and an interrupt input to be activated.

Boot_1: State entered after an input interrupt is detected. Output the address stored in XGVBR + 2*XGCHID to fetch PC (Program Counter).

Boot_2: State entered after Boot_1. Load the PC and output the address of the variable pointer, XGVBR + 2*XGCHID + 2.

Boot_3: State entered after Boot_2. Load the variable pointer. Output the PC and prepare to load the first instruction.

Decode: In normal user operation this is the state entered after Boot_3 and the instruction is decoded and executed. For single cycle instructions, if Debug Mode is not active, then the Program Counter is incremented and the next instruction is loaded. For multi-cycle instructions one of the three instruction continue states is selected. If an RTS instruction is decoded then the state machine is returned to the Idle state to wait for the next change in XGCHID.

S_stall: Simple stall is the state that branch instructions use to change the PC when a change of flow is required. This state is also used for Store instructions.

W_stall: Word stall is the state Load instructions use to retrieve a word of RAM memory data.

B_stall: Byte stall is the state Load instructions use to retrieve a byte of RAM memory data.

Break: If the Decode state detects a BRK command then the Break state is entered. The RISC processor remains in the Break state till:

Single Step Command

XGEN is set to zero

XGCHID is set to zero

Break_2: After a Single Step Command is issued in the Break state the PC is incremented in this state.

Ld_inst: The Load Instruction state is used in debug mode to load the next instruction to be executed into the RISC instruction register.

Debug: debug is the state where the RISC processor waits for an input to execute the next command Exit conditions for this state are:

Single Step Command

XGEN is set to zero

XGCHID is set to zero

#### 2.5.3 Semaphore Bits

Each semaphore bit is a simple state machine that is controlled by write commands from either the host or RISC processor. The first write command to set a semaphore bit takes control of the semaphore state until the bit is cleared by the setting processor. The host has priority if the host and RISC processor attempt to write the semaphore bit in the same clock cycle. Each processor has a different read view of the semaphore bit so it can determine if it has successfully set the bit.

// Semaphore states

NO_LOCK =  2'b00

RISC_LOCK = 2'b10

HOST_LOCK = 2'b11

These are the states for the semaphore bits. Only three of the four possible states are required to cover all the real things that can happen. For decoding the state of a semaphore bit the MSB is equivalent to "locked" and the LSB is equivalent to which processor is the one that owns the lock. Therefore state 2'b01 doesn't make sense because it implies there is no "lock" but the bit is owned by the host. In terms of the actual logic it probably makes little if any difference how the states are encoded but from the mental perspective it maybe helpful in understanding and debug to have the individual bits of the state be connected to a unique meaning.

host_lock

risc_lock

no_lock

SSEM

CSEM

wrt_semp=0

wrt_semp=1

Operation

C

Z

V

N

The Xgate Module is a simple RISC CPU with an instruction set that is compatible with the Freescale XGATE module.

R0=0

R1 Variable Address

R2

R3

R4

R5

R6

R7

PC

Programmers Model

15

General Purpose Registers

Program Counter

Condition Code Register

0

0

15

15

0

1

2

3

The COP module also has the capability to generate an interrupt at a programmed number of cycles before the cop_rst_o initiates a system reset. This functionality is primarily intended as a debug feature.

### 3.1 Software Example

The recommended software procedure for using the Xgate Module:

- Initialize Xgate

- Load RAM with Xgate software.

- Set XGVBR register to base address of input interrupt vectors.

- Enable Xgate interrupts by clearing bits in the IRQ_BP0 – IRQ_BP7 control registers.

- Enable Xgate by setting the XGE bit.

- Enable host to accept interrupts.

- Enable peripheral modules to output interrupts as required.

- Normal Operation

- Service Xgate interrupt requests as they are received by the host.

### 3.2 Software Triggers

### 3.3 Semaphore Bits

### 3.4 Program Memory Connection Options

### 3.5 Debug Mode

### 3.6 Instruction Set Summary

The RISC Processor Core is the key element of the Xgate module. This module implements the Xgate instruction set.

| Function | 15 | 14 | 13 |  | 12 |  | 11 |  | 10 |  | 9 |  | 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  | 0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Return to Scheduler and other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BRK | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |
| NOP | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 1 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |

### 3.7 Inconsistent Instruction Set Documentation

The following instructions have problems in the Freescale documentation.

#### 3.7.1 SUBL

Freescale Documentation:

Z:

V: Set if a two’s complement overflow resulted from the 8-bit operation; cleared otherwise.

RD[15]old & !RD[15]new

C: Set if there is a carry from the bit 7 to bit 8 of the result; cleared otherwise.

!RD[15]old & RD[15]new

Xgate implementation:

For signed subtraction the rules for overflow are:

Negative number minus Positive number creates a Positive number OR

Positive number minus Negative number creates a Negative number

So the overflow calculation should be:

V = RD[7] & !IMM8[7] & !result[7]  |  !RD[7] & IMM8[7] & result[7]

The Carry calculation should be:

C = !RD[7] & IMM8[7]  |  !RD[7] & result[7]  |  IMM8[7] & result[7]

These calculations for setting the Carry and Overflow bits are also consistent with the calculations used to set the Carry and Overflow bits using the CMPL instruction.

#### 3.7.2 ADDL

Freescale Documentation:

V: Set if a two’s complement overflow resulted from the 8-bit operation; cleared otherwise.

RD[15]old & RD[15]new

C: Set if there is a carry from the bit 7 to bit 8 of the result; cleared otherwise.

RD[15]old & RD[15]new

Xgate implementation:

For signed addition the rules for overflow are:

Positive number plus Positive number creates a Negative number OR

Negative number plus Negative number creates a Positive number

So the overflow calculation should be:

V = !RD[7] & !IMM8[7] & result[7]  |  RD[7] & IMM8[7] & !result[7]

The Carry calculation should be:

C = !RD[7] & IMM8[7] | RD[7] & !result[7] | IMM8[7] & !result[7]

#### 3.7.2 SUBH

Freescale Documentation:

Z = ????

Registers

The Xgate Module has a WISHBONE slave bus interface configured for a 16-bit bus width with 8-bit granularity.

### List of Registers

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |
| XGMCTL | 0x00 | 16 | RW | Xgate Module Control Register |
| XGCHID | 0x02 | 16 | RW | Xgate Channel ID Register |
|  | 0x04 | 16 | RW | Reserved |
|  | 0x06 | 16 | RW | Reserved |
| XGVBR | 0x08 | 16 | RW | Xgate Vector Base Address |
| XGIF_7 | 0x0A | 16 | RW | Xgate Interrupt Flag Register #7 |
| XGIF_6 | 0x0C | 16 | RW | Xgate Interrupt Flag Register #6 |
| XGIF_5 | 0x0E | 16 | RW | Xgate Interrupt Flag Register #5 |
| XGIF_4 | 0x10 | 16 | RW | Xgate Interrupt Flag Register #4 |
| XGIF_3 | 0x12 | 16 | RW | Xgate Interrupt Flag Register #3 |
| XGIF_2 | 0x14 | 16 | RW | Xgate Interrupt Flag Register #2 |
| XGIF_1 | 0x16 | 16 | RW | Xgate Interrupt Flag Register #1 |
| XGIF_0 | 0x18 | 16 | RW | Xgate Interrupt Flag Register #0 |
| XGSWT | 0x1A | 16 | RW | Xgate Software Trigger Register |
| XGSEM | 0x1C | 16 | RW | Xgate Semaphore Register |
|  | 0x1E | 16 | R | Reserved |
| XGCCR | 0x20 | 16 | RW | Xgate Condition Code Register |
| XGPC | 0x22 | 16 | RW | Xgate Program Counter |
|  | 0x24 | 16 | R | Reserved |
| XGR1 | 0x26 | 16 | RW | Xgate Register 1 |
| XGR2 | 0x28 | 16 | RW | Xgate Register 2 |
| XGR3 | 0x2A | 16 | RW | Xgate Register 3 |
| XGR4 | 0x2C | 16 | RW | Xgate Register 4 |
| XGR5 | 0x2E | 16 | RW | Xgate Register 5 |
| XGR6 | 0x30 | 16 | RW | Xgate Register 6 |
| XGR7 | 0x32 | 16 | RW | Xgate Register 7 |
|  |  | 16 | R | Reserved |
| IRQ_BP0 | 0x40 | 16 | RW | IRQ Bypass Register 0 |
| IRQ_BP1 | 0x42 | 16 | RW | IRQ Bypass Register 1 |
| IRQ_BP2 | 0x44 | 16 | RW | IRQ Bypass Register 2 |
| IRQ_BP3 | 0x46 | 16 | RW | IRQ Bypass Register 3 |
| IRQ_BP4 | 0x48 | 16 | RW | IRQ Bypass Register 4 |
| IRQ_BP5 | 0x4A | 16 | RW | IRQ Bypass Register 5 |
| IRQ_BP6 | 0x4C | 16 | RW | IRQ Bypass Register 6 |
| IRQ_BP7 | 0x4E | 16 | RW | IRQ Bypass Register 7 |

Table 1: List of registers

### 4.1 Xgate Module Control Register (XGMCTL)

- Note: The XGMCTL register can only be written using a word(16 bit) access.

| Bit # | Access | Description |
| --- | --- | --- |
| 15 | W | ###### XGEM, XGE Mask – The XGE bit can only be changed when the XGEM bit is set in the same register access. ###### ‘0’ No change allowed to XGE control bit. ‘1’ Change allowed to XGE control bit. Always reads as ‘0’. |
| 14 | W | ###### XGFRZM, XGFRZ Mask – The XGFRZ bit can only be changed when the XGEM bit is set in the same register access. ###### ‘0’ No change allowed to XGFRZ control bit. ‘1’ Change allowed XGFRZ control bit. Always reads as ‘0’. |
| 13 | W | XGDGBM, XGDGB Mask – The XGDGB bit can only be changed when the XGDGBM bit is set in the same register access. ###### ‘0’ No change allowed to XGDBG control bit. ‘1’ Change allowed to XGDBG Control bit. Always reads as ‘0’. |
| 12 | W | XGSSM, XGSS Mask – The XGSS bit can only be changed when the XGSSM bit is set in the same register access. ###### ‘0’ No change allowed to XGSS control bit. ‘1’ Change allowed to XGSS control bit. Always reads as ‘0’. |
| 11 | W | XGFACTM, XGFACT Mask – The XGFACT bit can only be changed when the XGFACTM bit is set in the same register access. ###### ‘0’ No change allowed to XGFACT control bit. ‘1’ Change allowed to XGFACT control bit. Always reads as ‘0’. |
| 10 |  | BRK_IRQ_ENM, BRK_IRQ_EN Mask – The BRK_IRQ_EN bit can only be changed when the BRK_IRQ_ENM bit is set in the same register access. ###### ‘0’ No change allowed to BRK_IRQ_EN control bit. ‘1’ Change allowed to BRK_IRQ_EN control bit. ###### Always reads as ‘0’. |
| 9 | RW | ###### XGSWEIFM, XGSWEIF Mask – The XGSWEIF bit can only be changed when the XGSWEIFM bit is set in the same register access. ###### ‘0’ No change allowed to XGSWEIF control bit. ‘1’ Change allowed to XGSWEIF control bit. Always reads as ‘0’. |
| 8 | RW | ###### XGIEM, XGIE Mask – The XGIE bit can only be changed when the XGIEM bit is set in the same register access. ###### ‘0’ No change allowed to XGIE control bit. ‘1’ Change allowed to XGIE control bit. Always reads as ‘0’. |
| 7 | RW | ###### XGE, XG Enable – The XGE bit can only be changed when the XGEM bit is set in the same register access. ###### ‘0’ The Xgate module is disabled. The currently active interrupt request will be completed and then no new interrupt processing requests will be accepted. ‘1’ The Xgate module is enabled to start processing channel interrupts. |
| 6 | RW | ###### XGFRZ, XG Freeze Mode – The XGFRZ bit functionality is currently unimplemented. This bit can only be changed when the XGFRZM bit is set in the same register access. ###### ‘0’ The XGFRZ register bit is clear. ‘1’ The XGFRZ register bit is set. |
| 5 | RW | ###### XGDBG, XG Debug Mode – The XGDBG bit can only be changed when the XGDBGM bit is set in the same register access. ###### ‘0’ The Xgate module is not in DEBUG Mode. ‘1’ The Xgate module is in DEBUG Mode. Along with writing to the XGGBG bit from the host interface DEBUG Mode can also be entered by executing the ‘BRK’ instruction by the RISC Processor Core. |
| 4 | RW | ###### XGSS, XG Single Step – The XGSS bit can only be changed when the XGSSM bit is set in the same register access. ###### ‘0’ The Xgate module is disabled. The currently active channel interrupt will be completed and then no new interrupt channel processing requests will be accepted. ‘1’ Write - Xgate module should execute a single RISC instruction. Read – A RISC instruction is being processed. |
| 3 | RW | ###### XGFACT, XG Fake Activity – The XGFACT bit functionality is currently unimplemented. This bit can only be changed when the XGFACTM bit is set in the same register access. ###### ‘0’ The XGFACT bit is clear. ‘1’ The XGFACT is set. |
| 2 | RW | ###### BRK_IRQ_EN, Break Interrupt Enable – The BRK_IRQ_EN bit can only be changed when the BRK_IRQ_EN M bit is set in the same register access. ###### ‘0’ The Xgate module is operating in a mode compatible with the Freescale XGATE module. BRK instructions do not generate an Xgate error interrupt output. ###### ‘1’ The Xgate module is enabled to output an error interrupt whenever a BRK command is encountered. Set this bit whenever no BRK commands are expected to be used in the software. |
| 1 | RW | ###### XGSWEIF, XG Software Error Interrupt Flag – The XGSWEIF bit can only be changed when the XGSWEIFM bit is set in the same register access. ###### ‘0’ The Xgate module is operating correctly. Writing ‘0’ has no effect. ‘1’ The Xgate RISC processor has detected an error condition. Writing ‘1’ will clear a set XGSWEIF bit and terminate the current interrupt process. The Xgate module will go to an IDLE state if no other interrupt inputs are active. |
| 0 | RW | ###### XGIE, XG Interrupt Enable – The XGIE bit can only be changed when the XGEM bit is set in the same register access. ###### ‘0’ All Xgate module interrupt outputs are disabled. ‘1’ All Xgate module interrupt outputs are enabled. |
| Reset Value: |  |  |
|  |  | XGMCTL: 0004h |

Table 2: CNTRL Register Bits

### 4.2 Xgate Channel ID Register (XGCHID)

| Bit # | Access | Description |
| --- | --- | --- |
| 15:7 | R | ###### Reserved, write zeros for future compatibility. |
| 6:0 | RW | ###### XGCHID, XG Channel ID – The XGCHID register reflects the value of the current interrupt channel being processed. ###### ‘0’ No interrupt currently being processed. ‘1’ The encoded value of the input interrupt currently being processed. The XGCHID can only be written in DEBUG Mode. When a value is written to the XGCHID register the RISC Processor Core will start executing the code associated with that interrupt input. |

Reset Value:

XGCHID: 0000h

Table 4: Channel ID Register Bits

### 4.3 Xgate Vector Base Address Register (XGVBR)

| Bit # | Access | Description |
| --- | --- | --- |
| 15:1 | RW | XGVBR, Sets the vector base address that the Xgate RISC Core uses to determine where in the memory map the code and data space is for a specific interrupt is reserved. XGVBR can only be changed when XGE is zero. |

Reset Value:

XGVBR: FE00h

Table 5: XGVBR Register Bits

### 4.4 Xgate Interrupt Flag Register 7 (XGIFR_7)

The bits in this register reflect the state of interrupts that are being output from the Xgate module to the host CPU. A bit is set by the Xgate module with the “SIF” instruction usually at the end of a process and the host CPU clears the XGIF bit as part of its interrupt service routine.

Clear the interrupt output by writing a ‘1’ to the corresponding XGIF register bit.

The highest numbered interrupt is in the MSB position of the register bank.

Some bits in this register may be unimplemented in a specific hardware design based on the setting of the “MAX_CHANNEL” parameter and will always read as “0”.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | RW | XGIF_127 – XGIF_112 ###### ‘0’ No interrupt. ‘1’ Interrupt output active. |

Reset Value:

XGIFR_7: 0000h

Table 5: XGIF_7 Register Bits

### 4.12 Xgate Software Trigger Register (XGSWT)

- Note: The XGSWT register can only be written using a word(16 bit) access.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:8 | W | XGSWTM[7:0], Read always returns zero. ###### XGSWTM, XGSWT Mask – The XGSWT bits can only be changed when the XGSWTM bit is set in the same register access. ###### ‘0’ No change allowed to XGSWT control bit. ‘1’ Change allowed to XGSWT control bit. Always reads as ‘0’. |
| 7:0 | RW | ###### XGSWT[7:0], XG Software Trigger – XGSWT bits can only be changed when the corresponding XGSWTM bit is set in the same register access. ###### ‘0’ The task associated with the XGSWT is disabled. ‘1’ The task associated with the XGSWT bit is queued to start processing when it becomes the highest priority Xgate interrupt input. |

Reset Value:

XGSWT: 0000h

Table 5: XGSWT Register Bits

### 4.13 Xgate Semaphore Register (XGSEM)

- Note: The XGSEM register can only be written using a word(16 bit) access.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:8 | W | ###### XGSEMM, XGSEM Mask – The XGSEM bit can only be changed when the XGSEMM bit is set in the same register access. ###### ‘0’ No change allowed to XGSEM control bit. ‘1’ Change allowed to XGSEM control bit. Always reads as ‘0’. |
| 7:0 | RW | ###### XGSWT[7:0], XG Software Trigger – The XGSWT bit can only be changed when the XGSWTM bit is set in the same register access. ###### ‘0’ The semaphore bit is either unlocked or locked by the Xgate module. If the host attempts to set the bit by writing a ‘1’ and a subsequent read of the bit returns a ‘0’ the bit is locked by the Xgate module. ‘1’ The semaphore bit is locked by the host processor. The host writes a ‘0’ to clear the bit and unlock the semaphore. |

Reset Value:

XGSEM: 0000h

Table 5: XGSEM Register Bits

### 4.14 Xgate Condition Code Register (XGCCR)

- Note: The XGCCR register can only be written when DEBUG mode is active or the XGE bit is clear.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:4 | R | ###### Reserved, write zeros for future compatibility. |
| 3 | RW | ###### XGN, Xgate Negative – Reflects the current state of the “Negative” bit of the Xgate condition code register. |
| 2 | RW | ###### XGZ, Xgate Zero – Reflects the current state of the “Zero” bit of the Xgate condition code register. |
| 1 | RW | ###### XGV, Xgate Overflow – Reflects the current state of the “Overflow” bit of the Xgate condition code register. |
| 0 | RW | ###### XGC, Xgate Carry – Reflects the current state of the “Carry” bit of the Xgate condition code register. |

Reset Value:

XGCCR: 0000h

Table 5: XGCCR Register Bits

### 4.15 Xgate Program Counter Register (XGPC)

- Note: The XGPC register can only be written when DEBUG mode is active or the XGE bit is clear.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | RW | XGPC, Read returns the current state of the Xgate Program Counter. Writes to the XGPC register can only be done in Debug Mode. |

Reset Value:

XGPC: 0000h

Table 5: XGPC Register Bits

### 4.16 Xgate Register 1 (XGR1)

- Note: The XGR1 register can only be written when DEBUG mode is active or the XGE bit is clear.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | RW | XGR1, Read returns the current state of the Xgate General Purpose Register 1. Writes to the XGR1 register can only be done in Debug Mode. |

Reset Value:

XGR1: 0000h

Table 5: XGR1 Register Bits

### 4.23 Interrupt Bypass Register 0 (IRQ_BP0)

The bits in this register are used to control the visibility of interrupts coming from the chan_req_i inputs to the Xgate RISC processor or if they are bypassed to the host processor.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | RW | IRQ_BYPASS[15:0] ###### ‘0’ The input interrupts, chan_reg_i[15:0], are accepted by the Xgate module and blocked from passing through to the xgif_o[15:0] outputs. An internal Xgate processing thread is triggered. The Xgate module triggers the corresponding output interrupt, xgif_o[15:0], as required by the Xgate software. ‘1’ The input interrupts, chan_req_i[15:0] are bypassed directly to the Xgate interrupt outputs, xgif_o[15:0]. The input interrupt is disabled from triggering an Xgate processing thread. |

Reset Value:

IRQ_BP0: FFFFh

Table 5: IRQ_BP0 Register Bits

Clocks

| Name | Source | Rates (MHz) |  |  | Remarks | Description |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Max | Min | Resolution |  |  |
| wbs_clk_i | System | 200 | - | - | Master clock for all Xgate bus registers. Positive edge active. | System clock. |
| risc_clk_i | System |  |  |  |  | RISC Processor clock |

Table 3: List of clocks

The wbs_clk_i has no timing constraints based on the RTL implementation although there may be constrains applied for synthesis results to be compatible with the target physical implementation. If the Xgate is targeted for an ASIC implementation then [wbs_clk_i] should be used as the scan clock, any clock multiplexing required to make [wbs_clk_i] the scan clock should be done at the system level external to the Xgate Module.

The risc_clk_i is the clock used to run the Xgate RISC processor submodule. The frequency of this clock should be the same as or 2X the frequency of the wbs_clk_i input. The risc_clk_i input is assumed to be in phase with the wbs_clk_i input.

IO Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| wbs_clk_i | 1 | Input | WISHBONE Bus Clock Input, Master Clock |
| wbs_rst_i | 1 | Input | WISHBONE Slave Synchronous Reset |
| wbs_adr_i | 5 | Input | WISHBONE Slave Lower address bits |
| wbs_dat_i | 16 | Input | WISHBONE Slave Bus Data |
| wbs_dat_o | 16 | Output | WISHBONE Slave Bus Data |
| wbs_we_i | 1 | Input | WISHBONE Slave Write enable |
| wbs_stb_i | 1 | Input | WISHBONE Slave Strobe signal/Core select |
| wbs_cyc_i | 1 | Input | WISHBONE Slave Valid bus cycle |
| wbs_sel_i | 2 | Input | WISHBONE Slave Data Bus Byte Select |
| wbs_ack_o | 1 | Output | WISHBONE Slave Bus cycle acknowledge |
| wbm_adr_o | 16 | Output | WISHBONE Master address bits |
| wbm_dat_i | 16 | Input | WISHBONE Master Bus Data |
| wbm_dat_o | 16 | Output | WISHBONE Master Bus Data |
| wbm_we_o | 1 | Output | WISHBONE Master Write enable |
| wbm_stb_o | 1 | Output | WISHBONE Master Strobe signal/Core select |
| wbm_cyc_o | 1 | Output | WISHBONE Master Valid bus cycle |
| wbm_sel_o | 2 | Output | WISHBONE Master Data Bus Byte Select |
| wbm_ack_i | 1 | Input | WISHBONE Master Bus cycle acknowledge |
| arst_i | 1 | Input | Asynchronous Reset |
| risc_clk_i | 1 | Input | Clock signal for the RISC Core Processor |
| chan_req_i | 1271 | Input | Xgate Interrupt Request |
| xgif_o | 1271 | Output | Xgate Interrupt Request |
| xg_sw_irq | 1 | Output | Xgate Software Error Interrupt |
| xgswt | 8 | Output | Xgate Software Trigger |
| debug_mode_i | 1 | Input | Force Xgate into debug mode |
| secure_mode_i | 1 | Input | Limit host access to RISC core registers |
| scantestmode_i | 1 | Input | Scan Test Mode Enable |
| 1) The actual width is determined by the parameter MAX_CHANNEL, the default is the maximum value: 127. |  |  |  |

Table 4: List of IO ports

### 6.1 WISHBONE Slave Interface

The core features a WISHBONE RevB.3 compliant WISHBONE Classic interface that operates in SLAVE mode. All output signals are registered. Each access takes 2 clock cycles. To limit a WISHBONE access to just two clock cycles the following synthesis rules should be used:

- Single cycle timing for wbs_cyc_i and wbs_stb_i

- Two cycle timing for wbs_adr_i, wbs_data_i, and wbs_data_o. (Single cycle timing could be used but it would be a waste of resources to meet an over constrained timing path.)

- Note: Use the “SINGLE_CYCLE” parameter to do a WISHBONE bus access in one clock cycle.

| WISHBONE DATASHEET |  |  |
| --- | --- | --- |
| Description | Specification |  |
| General description: | 16-bit SLAVE |  |
| Supported Cycles: | SLAVE, READ/WRITE |  |
| Data port, size: Data port, granularity: Data port, maximum operand size: Data transfer ordering: Data transfer sequencing: | 16 bit word 8 bit byte |  |
| Supported signal list and cross reference to equivalent WISHBONE signals: | Signal Name | WISHBONE Equiv. |
|  | wbs_clk_i | CLK_I |
|  | wbs_rst_i | RST_I |
|  | wbs_adr_i | ADR_I() |
|  | wbs_dat_i | DAT_I() |
|  | wbs_dat_o | DAT_O() |
|  | wbs_we_i | WE_I |
|  | wbs_stb_i | STB_I |
|  | wbs_cyc_i | CYC_I |
|  | wbs_sel_i | SEL_I() |
|  | wbs_ack_o | ACK_O |

#### 6.1.1 wbs_rst_i

The synchronous reset signal has a minimum pulse width requirement of one [wbs_clk_i] clock period. It will take two [wbs_clk_i] clock cycles for all registers in the Xgate Module to initialize. Also see information on pin [arst_i].

#### 6.1.2 wbs_adr_i

The slave WISHBONE address pins are defined to be compatible with a WISHBONE bus that is word addressed and byte accessible. This means that there is no [wbs_adr_i[0]] input. If only word accesses are to be allowed then the LSB of the host address bus can be connected to [wbs_adr_i[1]].

#### 6.1.3 wbs_sel_i

The [wbs_sel_i] are the WISHBONE byte lane select signals. These signals allow selected Xgate registers to be written in byte mode. Note that some registers can only be written in 16 bit word mode. If only word mode accesses are to be done then these signals should be tied hi.

### 6.2 Xgate signals

#### 6.2.1 xgif_o[127:1]

The Xgate output signal. This output is the result of executing the SIF command by the Xgate RISC processor. The normal connection is to the interrupt inputs of the host processor.

#### 6.2.2 chan_req_i[127:1]

These signals are the interrupt inputs from the peripheral modules that the Xgate services. The eight outputs of the xgswt_o bus may also connect to some subset these inputs.

#### 6.2.3 xgswt_o[7:0]

The [xgswt_o] signal is active low. These signals are activated by the host processor writing to the XGSWT register. These signals are connected to a user selected subset of the [chan_req_i] inputs to trigger the Xgate coprocessor to execute a software routine associated with the selected XGSWT register bit. The host should respond the the setting of one of the [xgif_o] outputs by clearing the set bit in the XGSWT register. Priority of the software routine executed is determined by which [chan_req_i] input the [xgswt_o] signal is connected to.

#### 6.2.4 arst_i

The signal [arst_i] is an asynchronous reset signal that goes to all flops in the COP. It is provided for FPGA implementations and test methodologies that require this function for initialization. Using [arst_i] instead of [wb_rst_i] can result in lower cell-usage and higher performance for a FPGAs implementation because the standard FPGA cell already provides a dedicated asynchronous reset path. Using [wb_rst_i] for an ASIC implementation might synthesize to a smaller module because smaller non_reset flops can be used. Use either [arst_i] or [wb_rst_i], tie the other to a negated state. The active level of [arst_i] is determined by the parameter ARST_LVL that defaults to active low.

#### 6.2.6 xg_sw_irq_o

This output signal is activated whenever the Xgate is activated whenever the encounters a software error. These errors are:

- Instruction error

- Instruction Address error

- Load/Store Word Address error

#### 6.2.7 risc_clk_i

The [risc_clk_i] is the clock input signal used by the RISC core processor.

#### 6.2.8 debug_mode_i

The [debug_mode_i] input can be used by an external source such as a debug module to force the Xgate RISC core processor into debug mode. The exact number of clock cycles between the [debug_mode_i] signal going active and the RISC core stopping instruction execution may vary between one and several clock cycles. If no external debug activation is required then the [debug_mode_i] signal should be tied low.

#### 6.2.9 secure_mode_i

The [secure_mode_i] input is used to limit host read access to the RISC core registers for software security reasons. In an ASIC implementation this pin would be tied to a non-volatile memory bit that is only cleared when all of the Xgate program memory is also cleared. In an FPGA implementation the signal may be tied to a level consistent with the desired application security level. If no software security is required then the [secure_mode_i] signal should be tied low.

#### 6.2.10 scantestmode_i

The [scantestmode_i] input is an optional signal used to put the module into scan test mode. When [scantestmode_i] is active the [startup_osc_clk_i] is replaced by the [wb_clk_i] clock so all register are clocked by a common clock source.

### 6.3 Xgate Core Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| ARST_LVL | Bit | 1’b0 | Asynchronous reset level |
| MAX_CHANNEL | Integer | 127 | Number of input an output interrupts |
| SINGLE_CYCLE | Bit | 1’b0 | WISHBONE wait state |
| WB_RD_DEFAULT | Bit | 1’b0 | Default state for the WISHBONE read bus |

#### 6.3.1 ARST_LVL

The asynchronous reset level can be set to either active high (1’b1) or active low (1’b0).

Allowed values: 1’b0, 1’b1

#### 6.3.2 MAX_CHANNEL

The maximun index value for chan_req_i and xgif.

Allowed values: any integer value between 1 and 127

#### 6.3.3 SINGLE_CYCLE

The default operation of the Xgate WISHBONE bus interface is to insert one wait state by delaying the assertion of the wb_ack_o by one wb_clk_i period. Setting the SINGLE_CYCLE parameter generates the wb_ack_o combinationaly  so a WISHBONE bus cycle can be completed in one wb_clk_i period.

Allowed values: 1’b0, 1’b1

#### 6.3.4 WB_RD_DEFAULT

The WB_RD_DEFAULT parameter is used to facilitate the connection of the WISHBONE slave data output bus in a “multiplexer interconnection” scheme on an SoC with a minimum of external glue logic. The value of the parameter determines the state of the wbs_dat_o data bus when the Xgate module is not selected for a read operation.

Allowed values: 1’b0, 1’b1

Instruction Set Details

#### ADC – Add with Carry

Function:

RS1 + RS2 + C => RD

Use binary addition to add RS1, RS2, and Carry and store the sum in the RD register.

Example:

; Do 32 bit addition R7:R6 = R5:R4 + R3:R2

; --- Add least significant 16 bit words

ADD	R6, R4, R2	; R6 = R4 + R2

; --- Add most significant 16 bit words

ADC	R7, R5, R3	; R7 = R5 + R3 + C

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s and Z was set before the operation, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS1[15] & RS2[15] & ~RD[15]new) | (~RS1[15] & ~RS2[15] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(RS1[15] & RS2[15]) | (RS1[15] & ~RD[15]new) | (RS2[15] & ~RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADC RD,RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 1 | RD | RS1 | RS2 | 1 | 1 | P |

#### ADD – Add without Carry

Function:

RS1 + RS2 => RD

Use binary addition to add RS1, RS2, and store the sum in the RD register.

Example:

; Do 16 bit addition R6 = R4 + R2

ADD	R6, R4, R2	; R6 = R4 + R2

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS1[15] & RS2[15] & ~RD[15]new) | (~RS1[15] & ~RS2[15] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(RS1[15] & RS2[15]) | (RS1[15] & ~RD[15]new) | (RS2[15] & ~RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADD RD,RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 1 | RD | RS1 | RS2 | 1 | 0 | P |

#### ADDH – Add Immediate 8-bit constant high byte

Function:

RD + IMM8:$00 => RD

Use binary addition to add RD, and a signed 8-bit constant and store the sum in the high byte of the RD register.

Example:

; --- Add 16 bit immediate

ADDL	R6, #LOWBYTE	; R6 = R6 + 8 bit immediate

ADDH	R6, #HIGHBYTE	; R6 = R6 + 16 bit immediate

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RD[15]old & IMM8[7] & ~RD[15]new) | (~RD[15] & ~IMM8[7] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(RD[15]old & IMM8[7]) | (RD[15]old & ~RD[15]new) | (IMM8[7] & ~RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDH RD,#IMM8 | IMM8 | 1 | 1 | 1 | 0 | 1 | RD | IMM8 | P |

#### ADDL – Add Immediate 8-bit constant low byte

Function:

RD +$00:IMM8 => RD

Use binary addition to add RD, and an unsigned 8-bit constant and store the sum in the RD register.

Example:

; --- Add 16 bit immediate

ADDL	R6, #LOWBYTE	; R6 = R6 + 8 bit immediate

ADDH	R6, #HIGHBYTE	; R6 = R6 + 16 bit immediate

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(~RD[7] & ~IMM8[7] & RD[7]new) || (RD[7] & IMM8[7] & ~RD[7]new)

C: Set if there is a carry from bit 7 to bit 8 of the result, otherwise cleared.

(~RD[7]old & IMM8[7]) | (RD[7] & ~RD[7]new) | (~RD[7]new  & IMM8[7])

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADDL RD,#IMM8 | IMM8 | 1 | 1 | 1 | 0 | 0 | RD | IMM8 | P |

#### AND – Logical AND

Function:

RS1 & RS2 => RD

Do a bit wise logical AND between the RS1 and RS2 registers and store the sum in the RD register.

Example:

; Bitwise 16 bit AND operation

AND	R6, R4, R2	; R6 = R4 & R2

AND	R6, R0, R2	; R6 = $0000

AND	R0, R4, R2	; Set CCR without storing result

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AND RD,RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 0 | RD | RS1 | RS2 | 0 | 0 | P |

#### ANDH – Logical AND Immediate 8-bit constant high byte

Function:

RD.H & IMM8:$FF => RD

Do a bit wise logical AND between the 8-bit constant and the high byte of the RD register and store the sum in the RD register. The low byte of the RD register is unaffected.

Example:

ANDH	R6, #HIGHBYTE	; R6.H = R6.H & 8 bit immediate

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the 8 bit result, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ANDH RD,#IMM8 | IMM8 | 1 | 0 | 0 | 0 | 1 | RD | IMM8 | P |

#### ANDL – Logical AND Immediate 8-bit constant low byte

Function:

RD.L + $FF:IMM8 => RD

Da a logical AND function between the 8-bit constant and the low byte of the RD register and store the result in the RD register. The high byte of the RD register is unchanged.

Example:

ANDL	R6, #LOWBYTE	; R6.L = R6.L & 8 bit immediate

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 7 of the result is set, otherwise cleared.

Z: Set if the 8 bit result is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ANDL RD,#IMM8 | IMM8 | 1 | 0 | 0 | 0 | 0 | RD | IMM8 | P |

#### ASR – Arithmetic Shift Right

RD

D[15]

Q[0]

n bits

C

Function:

n = RS or IMM4

Use binary addition to add RS1, RS2, and Carry and store the sum in the RD register.

Example:

ASR	R6, #3	;

ASR	R6, R1	;

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Set if n > 0 and RD[n-1] = 1, if n = 0 then uneffected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ASR RD,#IMM4 | IMM4 | 0 | 0 | 0 | 0 | 1 | RD | IMM4 |  | 1 | 0 | 0 | 1 | P |
| ASR RD,RS | DYA | 0 | 0 | 0 | 0 | 1 | RD | RS | 1 | 0 | 0 | 0 | 1 | P |

#### BCC – Branch if Carry Cleared

Function:

If C = 0 then PC + $0002 + (REL9 <<1) => PC

Tests if the Carry Flag and branches if C = 0.

Example:

BCC	CARRY_C	;

;... Carry Set Code

CARRY_C

;... Carry Clear Code

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCC REL9 | REL9 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | REL9 | PP/P |

#### BCS – Branch if Carry Set

Function:

If C = 1 then PC + $0002 + (REL9 <<1) => PC

Tests if the Carry Flag and branches if C = 1.

Example:

BCS	CARRY_S	;

;... Carry Clear Code

CARRY_S

;... Carry Set Code

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCS REL9 | REL9 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | REL9 | PP/P |

#### BEQ – Branch if Equal

Function:

If Z = 1 then PC + $0002 + (REL9 <<1) => PC

Tests if the Zero Flag and branches if Z = 1.

Example:

TST	R1, R2

BEQ	A_EQ_B	;

;... Not equal Code

A_EQ_B

;... Equal Code

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BEQ REL9 | REL9 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | REL9 | PP/P |

#### BFEXT – Bit Field Extract

Function:

RS1[(offset+width):offset] => RD[width:0]; 0 => RD[15:width+1)]

width=(RS2[7:4])

offset=(RS2[3:0])

Extracts (width+1) bits from register RS1 starting at index offset and writes them right aligned to register RD. The remaining bits of RD are cleared. If (offset+width)>15 only bits with index [15:offset] get extracted.

Example:

LDL	R5, $5a	;

LDL	R2, $5a	; width = $5, offset = $a

BFEXT	R7, R5, R2	; R5[15:10] => R7[5:0], 0 => R7[15:6]

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not effected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BFEXT RD,RS1,RS2 | TRI | 0 | 1 | 1 | 0 | 0 | RD | RS1 | RS2 | 1 | 1 | P |

#### BFFO – Bit Field Find First One

Function:

FirstOne (RS) => RD;

Searches for the index of the most siginificant “1” in the RS register and puts that index into the RD register. The MSB’s of RD are cleared. If RS is $0000 then RD will be set to $0000 and the carry bit will be set. If the only “1” in RS is at index 0 then RD will be set to $0000 and the carry flag will be cleared.

Example:

; Find MSB of R2

BFFO	R3, R2	; R3 = Index of R2 MSB

; --- Branch if R2 was $0000

BCS	NO_ONES	; R2 had no set bits

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| 0 |  | 0 |  |

N: Always cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Set if RS is $0000, otherwise cleared.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BFFO RD,RS | DYA | 0 | 0 | 0 | 0 | 1 | RD | RS | 1 | 0 | 0 | 0 | 0 | P |

#### BFINS – Bit Field Insert

Function:

RS1[width:0] => RD[width+offset:offset]

width=(RS2[7:4])

offset=(RS2[3:0])

Extracts (width+1) bits from register RS1 starting at index 0 and writes them right into register RD at position offset. The remaining bits of RD are not effected. If (offset+width)>15 the upper bits are ignored. If R0 is used as RS1 then a bit field can be cleared.

Example:

LDL	R5, $5a	;

LDL	R2, $5a	; width = $5, offset = $a

BFINS	R7, R5, R2	; R7[15:10] = R5[5:0]

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not effected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BFINS RD,RS1,RS2 | TRI | 0 | 1 | 1 | 0 | 1 | RD | RS1 | RS2 | 1 | 1 | P |

#### BFINSI – Bit Field Insert and Invert

Function:

~RS1[width:0] => RD[width+offset:offset]

width=(RS2[7:4])

offset=(RS2[3:0])

Extracts (width+1) bits from register RS1 starting at index 0, invert them, and then writes the result into register RD at position offset. The remaining bits of RD are not effected. If (offset+width)>15 the upper bit are ignored. If R0 is used as RS1 then a bit field can be set.

Example:

LDL		R2, $5a	; width = $5, offset = $a

BFINSI	R7, R5, R2	; R7[15:10] = ~R5[5:0]

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not effected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BFINSI RD,RS1,RS2 | TRI | 0 | 1 | 1 | 1 | 0 | RD | RS1 | RS2 | 1 | 1 | P |

#### BFINSX – Bit Field Insert and XNOR

Function:

(~RS1[width:0] ^ RD[width+offset:offset])=> RD[width+offset:offset]

width=(RS2[7:4])

offset=(RS2[3:0])

Extracts (width+1) bits from register RS1 starting at index 0, performs an XNOR with them and the bits of RD[width+offset:offset], and then writes the result into register RD at position offset. If R0 is used as RS1 then a bit field can be toggled.

Example:

LDL		R2, $5a	; width = $5, offset = $a

BFINSX	R7, R5, R2	; R7[15:10] = R5[5:0] ^ R7[15:10]

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not effected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BFINSX RD,RS1,RS2 | TRI | 0 | 1 | 1 | 1 | 1 | RD | RS1 | RS2 | 1 | 1 | P |

#### BGE – Branch if Greater Than or Equal to Zero

Function:

If N ^ V = 0 then PC + $0002 + (REL9 <<1) => PC

Branch instruction to compare signed numbers.

Example:

; Branch if RS1 GTE RS2

SUB	R0, R1, R2	;

BGE	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BGE REL9 | REL9 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | REL9 | PP/P |

#### BGT – Branch if Greater than Zero

Function:

If Z | (N ^ V) = 0 then PC + $0002 + (REL9 <<1) => PC

Branch instruction to compare signed numbers.

Example:

; Branch if RS1 GT RS2

SUB	R0, R1, R2	;

BGT	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BGT REL9 | REL9 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | REL9 | PP/P |

#### BHI – Branch if Higher

Function:

If C | Z = 0 then PC + $0002 + (REL9 <<1) => PC

Branch instruction to compare unsigned numbers.

Example:

; Branch if RS1 GT RS2

SUB	R0, R1, R2	;

BHI	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BHI REL9 | REL9 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | REL9 | PP/P |

#### BHS – Branch if Higher or Same

Function:

If C = 0 then PC + $0002 + (REL9 <<1) => PC

--- Compiler alias to the command  BCC REL9

Branch instruction to compare unsigned numbers.

Example:

; Branch if RS1 GTE RS2

SUB	R0, R1, R2	;

BHS	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BHS REL9 | REL9 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | REL9 | PP/P |

#### BITH – Bit Test Immediate 8 Bit Constant high byte

Function:

RD.H & IMM8 => None

Preforms a bit wise logical AND operation between the IMM8 and the high byte of the  RD register. Used to detect Set bits in the RD register. RD is not changed and only the condition bits are updated after the instruction is completed.

Example:

; --- Test the high byte of R6

LDH	R6, $80		; R6 = $80??

BITH	R6, #HIGHBYTE	; R6 = R6

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the 8 bit result is $00, otherwise cleared.

V: Always set to zero.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BITH RD,#IMM8 | IMM8 | 1 | 0 | 0 | 1 | 1 | RD | IMM8 | P |

#### BITL – Bit Test Immediate 8 Bit Constant low byte

Function:

RD.L & IMM8 => None

Preforms a bit wise logical AND operation between the IMM8 and the low byte of the  RD register. Used to detect set bits in the RD register. RD is not changed and only the condition bits are updated after the instruction is completed.

Example:

; --- Test the low byte of R6

BITL	R6, #LOWBYTE	; R6 = R6

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the 8 bit result is $00, otherwise cleared.

V: Always set to zero.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BITL RD,#IMM8 | IMM8 | 1 | 0 | 0 | 1 | 0 | RD | IMM8 | P |

#### BLE – Branch if Less or Equal Zero

Function:

If Z | (N ^ V) = 1 then PC + $0002 + (REL9 <<1) => PC

Branch instruction to compare signed numbers.

Example:

; Branch if RS1 LTE RS2

SUB	R0, R1, R2	;

BLE	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLE REL9 | REL9 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | REL9 | PP/P |

#### BLO – Branch if Less Than

Function:

If C = 1 then PC + $0002 + (REL9 <<1) => PC

--- Compiler alias to the command  BCS REL9

Branch instruction to compare unsigned numbers.

Example:

; Branch if RS1 LT RS2

SUB	R0, R1, R2	;

BLO	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLO REL9 | REL9 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | REL9 | PP/P |

#### BLS – Branch Lower or Same

Function:

If C | Z = 0 then PC + $0002 + (REL9 <<1) => PC

Branch instruction to compare unsigned numbers.

Example:

; Branch if RS1 LTE RS2

SUB	R0, R1, R2	;

BLS	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLS REL9 | REL9 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | REL9 | PP/P |

#### BLT – Branch Lower Than Zero

Function:

If N ^ V = 1 then PC + $0002 + (REL9 <<1) => PC

Branch instruction to compare signed numbers.

Example:

; Branch if RS1 LT RS2

SUB	R0, R1, R2	;

BLS	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BLT REL9 | REL9 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | REL9 | PP/P |

#### BMI – Branch if Minus

Function:

If N = 1 then PC + $0002 + (REL9 <<1) => PC

Tests the Sign Flag and branch if N = 1.

Example:

; Branch if RS1 is Negative

SUB	R0, R1, R0	;

BMI	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BMI REL9 | REL9 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | REL9 | PP/P |

#### BNE – Branch if Not Equal

Function:

If Z = 0 then PC + $0002 + (REL9 <<1) => PC

Tests the Zero Flag and branch if Z = 0.

Example:

; Branch if RS1 NE RS2

SUB	R0, R1, R2	;

BNE	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BNE REL9 | REL9 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | REL9 | PP/P |

#### BPL – Branch if Plus

Function:

If N = 0 then PC + $0002 + (REL9 <<1) => PC

Tests the Sign Flag and branch if N = 0.

Example:

; Branch if RS1 Postive

ADD	R0, R1, R0	;

BLS	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BPL REL9 | REL9 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | REL9 | PP/P |

#### BRA – Branch Always

Function:

PC + $0002 + (REL10 << 1) => PC

Branch unconditionaly.

Example:

BRA	REL10

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BRA REL10 | REL10 | 0 | 0 | 1 | 1 | 1 | 1 | REL10 | PP |

#### BRK – Break

Function:

Put Xgate into Debug Mode.

Example:

BRK	; Goto debug mode and stop instruction execution

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BRK | INH | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | PAff |

#### BVC – Branch if Overflow Cleared

Function:

If V = 0 then PC + $0002 + (REL9 << 1) => PC

Tests the Overflag Flag and branch if V = 0.

Example:

; Branch if RS1 LTE RS2

SUB	R0, R1, R2	;

BVC	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BVC REL9 | REL9 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | REL9 | PP/P |

#### BVS – Branch if Overflow Set

Function:

If V = 1 then PC + $0002 + (REL9 <<1) => PC

Tests the Overflow Flag and branch if V = 1.

Example:

; Branch if RS1 LTE RS2

SUB	R0, R1, R2	;

BVS	REL9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BVS REL9 | REL9 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | REL9 | PP/P |

#### CMP – Compare

Function:

RS1 - RS2 => None

--- Compiler alias to the command  SUB R0, RS1 RS2

Subtract the contents of RS2 from RS1 using binary subtraction and update the condition code registers.

Example:

; --- Compare two 16 bit words

CMP	R5, R3	; R5 - R3

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS1[15] & ~RS2[15] & ~RD[15]new) | (~RS1[15] & RS2[15] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RS1[15] & RS2[15]) | (~RS1[15] & RD[15]new) | (RS2[15] & RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMP RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | RS1 | RS2 | 0 | 0 | P |

#### CMPL – Compare Immediate 8 Bit Constant Low Byte

Function:

RS.L – IMM8 => None

Subtract the IMM8 value from the low byte of RS using binary subtraction and update the condition code registers.

Example:

; Compare low byte of R7 to $55

CMPL	R7, #$55	; R5.L - $55

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 7 of the result is set, otherwise cleared.

Z: Set if the 8 bit result is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS[7] & ~IMM8[7] & ~result[7]) | (~RS[7] & IMM8[7] & result[15])

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RS[7] & RS2[7]) | (~RS[7] & result[7]new) | (RS[7] & result[7])

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMPL RS,#IMM8 | IMM8 | 1 | 1 | 0 | 1 | 0 | RS | IMM8 | P |

#### COM – One’s Complement

Function:

~RS => RD

~RD => RD

--- Compiler alias to the command  XNOR RD, R0 RS

--- Compiler alias to the command  XNOR RD, R0 RD

Do the one’s complement(bit wise invert) on the RS and store the result in the RD register..

Example:

COM	R6, R4,	; R6 = ~R4

COM	R7		; R7 = ~R7

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| COM RD,RS | TRI | 0 | 0 | 0 | 1 | 0 | RD | 0 | 0 | 0 | RS | 1 | 1 | P |
| COM RD | TRI | 0 | 0 | 0 | 1 | 0 | RD | 0 | 0 | 0 | RD | 1 | 1 | P |

#### CPC – Compare with Carry

Function:

RS1 - RS2 – C => None

--- Compiler alias to the command  SBC R0, RS1 RS2

Subtract the Carry bit and the contents of RS2 from RS1 using binary subtraction, and update the condition code registers.

Example:

; Do 32 bit subtraction R7:R6 = R5:R4 - R3:R2

; --- Subtract least significant 16 bit words

SUB	R6, R4, R2	; R6 = R4 - R2

; --- Subtract most significant 16 bit words

SPC	R5, R3	; R5 - R3 - C

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS1[15] & ~RS2[15] & ~result[15]) | (~RS1[15] & RS2[15] & result[15])

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RS1[15] & RS2[15]) | (~RS1[15] & result[15]) | (RS2[15] & result[15])

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SPC RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | RS1 | RS2 | 0 | 1 | P |

#### CPCH – Compare Immediate with bit constant with Carry high byte

Function:

RS.H – IMM8 - C => None

Subtract the contents of RS2 from RS1 using binary subtraction and update the condition code registers.

Example:

; Do 16 bit compare of constant to R2

CMPL	R6, #LOWBYTE	; Update Condition Code Register

CMPH	R6, #HIGHBYTE	; Update Condition Code Register

BCS	IS_EQUAL

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result is all $00 and Z was set before this instruction, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS[15] & ~IMM8[7] & ~result[15]) | (~RS[15] & IMM8[7] & result[15])

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RS[15] & IMM8[7]) | (~RS1[15] & result[15]) | (IMM8[7] & result[15])

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CPCH RS,#IMM8 | TRI | 1 | 1 | 0 | 1 | 1 | RS | IMM8 | P |

#### CSEM – Clear Semaphore Bit

Function:

0 => XGSEM[IMM3]

0 => XGSEM[RD[2:0]]

Unlocks a semaphore bit that was set by the RISC core.

Example:

; Do 32 bit addition R7:R6 = R5:R4 + R3:R2

; --- Add least significant 16 bit words

ADD	R6, R4, R2	; R6 = R4 + R2

; --- Add most significant 16 bit words

ADC	R7, R5, R3	; R7 = R5 + R3 + C

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CSEM #IMM3 | IMM3 | 0 | 0 | 0 | 0 | 0 | IMM3 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | P |
| CSEM RS | MON | 0 | 0 | 0 | 0 | 0 | RS | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | P |

#### CSL – Logical Shift Left with Carry

RD

Q[15]

D[0]

n bits

Cold

Cnew

…..

Cold

Function:

n bits

n = RS or IMM4; 0 ≤ n ≤ 16

Left shift the contents of the RD register n positions. The lower n bits of RD are filled with the contents of the carry register. At the end of the operation the carry register will be updated with the contents of RD[16-n] when n is greater than zero.

Example:

; Do 16 bit compare of constant to R2

CMPL	R6, #LOWBYTE	; Update Condition Code Register

CMPH	R6, #HIGHBYTE	; Update Condition Code Register

BCS	IS_EQUAL

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result is $0000, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

RD[15]old ^ RD[15]new

C: Set if (n > 0) and RD[16-n] = 1. If (n = 0) then there is no change in C.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CSL RD,#IMM4 | IMM4 | 0 | 0 | 0 | 0 | 1 | RD | IMM4 |  | 1 | 0 | 1 | 0 | P |
| CSL RD,RS | DYA | 0 | 0 | 0 | 0 | 1 | RD | RS | 1 | 0 | 0 | 1 | 0 | P |

#### CSR – Logical Shift Right with Carry

Function:

RD

D[15]

Q[0]

n bits

Cnew

Cold

…..

Cold

n bits

n = RS or IMM4; 0 ≤ n ≤ 16.

Example:

; Do 16 bit compare of constant to R2

CMPL	R6, #LOWBYTE	; Update Condition Code Register

CMPH	R6, #HIGHBYTE	; Update Condition Code Register

BCS	IS_EQUAL

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result is $0000, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

RD[15]old ^ RD[15]new

C: Set if (n > 0) and RD[n-1] = 1. If (n = 0) then there is no change in C.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CSR RD,#IMM4 | IMM4 | 0 | 0 | 0 | 0 | 1 | RD | IMM4 |  | 1 | 0 | 1 | 1 | P |
| CSR RD,RS | DYA | 0 | 0 | 0 | 0 | 1 | RD | RS | 1 | 0 | 0 | 1 | 1 | P |

#### JAL – Jump and LINK

Function:

PC + $0002 => RD; RD => PC

Branches to the address in RD and stores the return address to RD.

Example:

TFR	R5, PC	; Save the return address in R5

BRA	SUB_1		; Goto the subroutine code

; More Code

SUB_1

; Subroutine Code

JAL	RD		; Return to the address after the BRA

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| JAL RD | IMM4 | 0 | 0 | 0 | 0 | 0 | RD | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | PP |

#### LDB – Load Byte From Memory Low Byte

Function:

M[RB, $OFFS5]	=> RD.L; $00 => RD.H  ($OFFS5 = unsigned offset)

M[RB, RI]		=> RD.L; $00 => RD.H

M[RB, RI+]		=> RD.L; $00 => RD.H; RI + 1 => RI (Post increment)

RI – 1 => RI ;M[RS, -RI]	=> RD.L; $00 => RD.H (Pre decrement)

Loads the low byte of RD from memory and clear the high byte of RD.

Example:

LDB	R5, (R1,#9)	; Load R5 from the address at R1+9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LDB RD, (RB, #IMM8) | IDO5 | 0 | 1 | 0 | 0 | 0 | RD | RB | OFFS5 |  |  | PR |
| LDB RD, (RS, RI) | IDR | 0 | 1 | 1 | 0 | 0 | RD | RB | RI | 0 | 0 | PR |
| LDB RD, (RS, RI+)1 | IDR+ | 0 | 1 | 1 | 0 | 0 | RD | RB | RI | 0 | 1 | PR |
| LDB RD, (RS, -RI) | -IDR | 0 | 1 | 1 | 0 | 0 | RD | RB | RI | 1 | 0 | PR |

Note:

- If the RB == RD the final value of RD will be M[RB,RI]; RD will not be incremented.

#### LDH – Load Immediate 8 Bit Constant High Byte

Function:

IMM8 => RD.H

Put an 8 bit constant into the high byte of the RD register. The low byte of RD is unaffected.

Example:

LDL	R5, $55	; R5 = $0055

LDH	R5, $AA	; R5 = $AA55

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LDH #IMM8 | IMM8 | 1 | 1 | 1 | 1 | 1 | RD | IMM8 | P |

#### LDL – Load Immediate 8 Bit Constant Low Byte

Function:

IMM8 => RD.L; $00 => RD.H

Put an 8 bit constant into the low byte of the RD register. The high byte of RD is set to $00.

Example:

LDL	R5, $55	; R5 = $0055

LDH	R5, $AA	; R5 = $AA55

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LDL #IMM8 | IMM8 | 1 | 1 | 1 | 1 | 0 | RD | IMM8 | P |

#### LDW – Load Word From Memory

Function:

M[RB, $OFFS5]	=> RD

M[RB, RI]		=> RD

M[RB, RI+]		=> RD; RI + 2 => RI (Post increment)

M[RS, -RI]		=> RD; RI – 2 => RI (Pre decrement)

Loads the RD register from memory.

Example:

LDW	R5, (R1,#2)	; Load R5 from the address at R1+2

;  Immediate Constant should not exceed 30

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LDW RD, (RB, #IMM8) | IDO5 | 0 | 1 | 0 | 0 | 1 | RD | RB | OFFS5 |  |  | PR |
| LDW RD, (RS, RI) | IDR | 0 | 1 | 1 | 0 | 1 | RD | RB | RI | 0 | 0 | PR |
| LDW RD, (RS, RI+)1 | IDR+ | 0 | 1 | 1 | 0 | 1 | RD | RB | RI | 0 | 1 | PR |
| LDW RD, (RS, -RI) | -IDR | 0 | 1 | 1 | 0 | 1 | RD | RB | RI | 1 | 0 | PR |

Note:

1) If the RB == RD the final value of RD will be M[RB,RI]; RD will not be incremented.

#### LSL – Logical Shift Left

Function:

n bits

RD

Q[15]

D[0]

0

C

…..

0

0

n bits

Use binary addition to add RS1, RS2, and Carry and store the sum in the RD register.

Example:

LSL	R6, #5	; R6 = R6 left shifted by 5 bits

;  R6[4:0] = 0

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: .

RD[15]old ^ RD[15]new.

C: Set if n > 0 and RD[16-n] = 1, if n = 0 then uneffected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LSL RD,#IMM4 | IMM4 | 0 | 0 | 0 | 0 | 1 | RD | IMM4 |  | 1 | 1 | 0 | 0 | P |
| LSL RD,RS | DYA | 0 | 0 | 0 | 0 | 1 | RD | RS | 1 | 0 | 1 | 0 | 0 | P |

#### LSR – Logical Shift Right

Function:

n bits

RD

D[15]

Q[0]

C

0

…..

0

0

n bits

Use binary addition to add RS1, RS2, and Carry and store the sum in the RD register.

Example:

LSR	R6, #5	; R6 = R6 right shifted by 5 bits

;  R6[15:11] = 0

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if RD[15] changed.

RD[15]old ^ RD[15]new.

C: Set if n > 0 and RD[16-n] = 1, if n = 0 then uneffected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LSR RD,#IMM4 | IMM4 | 0 | 0 | 0 | 0 | 1 | RD | IMM4 |  | 1 | 1 | 0 | 1 | P |
| LSR RD,RS | DYA | 0 | 0 | 0 | 0 | 1 | RD | RS | 1 | 0 | 1 | 0 | 1 | P |

#### MOV – Move Register Content

Function:

RS => RD

--- Compiler alias to the command  OR RD, R0 RS

Copies the contents of the RS register to the RD register.

Example:

MOV	R6, R4	; R6 = R4

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MOV RD,RS | TRI | 0 | 0 | 0 | 1 | 0 | RD | 0 | 0 | 0 | RS2 | 1 | 0 | P |

#### NEG – Two’s Complement

Function:

-RS => RD

-RD => RD

--- Compiler alias to the command  SUB RD, R0 RS

--- Compiler alias to the command  SUB RD, R0 RD

Calculate the two’s complement of a general purpose register.

Example:

NEG	R6, R4	; R6 = -R4

NEG	R7		; R7 = -R7

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS1[15] & ~RS2[15] & ~RD[15]new) | (~RS1[15] & RS2[15] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RS1[15] & RS2[15]) | (~RS1[15] & RD[15]new) | (RS2[15] & RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NEG RD, RS | TRI | 0 | 0 | 0 | 1 | 1 | RD | 0 | 0 | 0 | RS | 0 | 0 | P |
| NEG RD | TRI | 0 | 0 | 0 | 1 | 1 | RD | 0 | 0 | 0 | RD | 0 | 0 | P |

#### NOP – No Operation

Function:

PC + $0002 => PC

Consume one cycle of RISC CPU time without doing anything.

Example:

NOP		; Stall for one CPU cycle

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOP | INH | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | P |

#### OR – Logical OR

Function:

RS1 | RS2 => RD

Do a bit wise logical OR operation between the RS1 register and the RS2 register and put the result in the RD register.

Example:

; Do 16 bit logical OR

OR	R6, R4, R2	; R6 = R4 | R2

OR	R6, R0, R4	; R6 = R4

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OR RD,RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 0 | RD | RS1 | RS2 | 1 | 0 | P |

#### ORH – Logical OR 8 Bit Immediate Constant High Byte

Function:

RD.H | IMM8 => RD.H

Do a bit wise logical OR operation between the 8 bit immediate value and the RD high byte and put the result in the RD high byte register.

Example:

; Do 8 bit logical OR

ORH	R6, #$AA	; R6 = R6 | $AA00

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the 8 bit result is $00, otherwise cleared.

V: Always cleared.

C: Not affected

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ORH RD,#IMM8 | IMM8 | 1 | 0 | 1 | 0 | 1 | RD | IMM8 | P |

#### ORL – Logical OR 8 Bit Immediate Constant Low Byte

Function:

RD.H | IMM8 => RD.H

Do a bit wise logical OR operation between the 8 bit immediate value and the RD low byte and put the result in the RD low byte register.

Example:

; Do 8 bit logical OR

ORL	R6, #$AA	; R6 = R6 | $0055

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the 8 bit result is $00, otherwise cleared.

V: Always cleared.

C: Not affected

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ORL RD,#IMM8 | IMM8 | 1 | 0 | 1 | 0 | 0 | RD | IMM8 | P |

#### PAR – Calculate Parity

Function:

RD.H | IMM8 => RD.H

Set the Carry flag if the number of one’s in the RD register is odd..

Example:

; Do 16 bit parity

PAR	R6	; R6 = R6; Carry = Odd or Even

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the RD is $0000, otherwise cleared.

V: Always cleared.

C: Set if RD has an odd number of  1’s, otherwise cleared.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PAR RD | IMM8 | 0 | 0 | 0 | 0 | 0 | RD | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | P |

#### ROL – Rotate Left

Function:

RD

Q[15]

D[0]

n bits

n  = RS or IMM4.

Set the Carry flag if the number of one’s in the RD register is odd..

Example:

ROL	R6, #5	; R6[15:0] = R6[10:0] R6[15:11]

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the RD is $0000, otherwise cleared.

V: Always cleared.

C: No affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROL RD, #IMM4 | IMM4 | 0 | 0 | 0 | 0 | 1 | RD | IMM4 |  | 1 | 1 | 1 | 0 | P |
| ROL RD, RS | DYA | 0 | 0 | 0 | 0 | 1 | RD | RS | 1 | 0 | 1 | 1 | 0 | P |

#### ROR – Rotate Right

RD

D[15]

Q[0]

n bits

Function:

n = RS or IMM4.

Example:

ROR	R6,#3	; R6 = R6 ROR 3 places

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the RD is $0000, otherwise cleared.

V: Always cleared.

C: No affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROR RD, #IMM4 | IMM4 | 0 | 0 | 0 | 0 | 1 | RD | IMM4 |  | 1 | 1 | 1 | 1 | P |
| ROR RD, RS | DYA | 0 | 0 | 0 | 0 | 1 | RD | RS | 1 | 0 | 1 | 1 | 1 | P |

#### RTS – Return to Scheduler

Function:

Completes the current thread of program instructions and remain idle till the hardware scheduler starts a new thread.

Example:

;

RTS		; Channel thread complete

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RTS | INH | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | P |

#### SBC – Subtract with Carry

Function:

RS1 - RS2 – C => RD

Subtract the Carry bit and the contents of RS2 from RS1 using binary subtraction, store the result in RD and update the condition code registers.

Example:

; Do 32 bit subtraction R7:R6 = R5:R4 - R3:R2

; --- Subtract least significant 16 bit words

SUB	R6, R4, R2	; R6 = R4 - R2

; --- Subtract most significant 16 bit words

SBC	R7, R5, R3	; R7 = R5 - R3 - C

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s and Z was set before the operation, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS1[15] & ~RS2[15] & ~RD[15]new) | (~RS1[15] & RS2[15] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RS1[15] & RS2[15]) | (~RS1[15] & RD[15]new) | (RS2[15] & RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SBC RD,RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 1 | RD | RS1 | RS2 | 0 | 0 | P |

#### SSEM – Set Semaphore Bit

Function:

1 => XGSEM[IMM3]

1 => XGSEM[RD[2:0]]

Locks a semaphore bit that was set by the RISC core.

Example:

NOT_MINE

SSEM	R6		; Attempt to set semaphore bit

BCC	NOT_MINE	; --- Wait till RISC has semaphore bit

SSEM	#3		; Attempt to set semaphore bit 3

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - |  |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Set if semaphore is locked by RISC core.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSEM #IMM3 | IMM3 | 0 | 0 | 0 | 0 | 0 | IMM3 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | P |
| SSEM RS | MON | 0 | 0 | 0 | 0 | 0 | RS | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | P |

#### SEX – Sign Extend

Function:

RD.L => RD

The 8 bit two’s complement number stored in RD.L is sign extended to fill the RD register.

Example:

SEX	R6		; --- Add 8 bit number to 16 bit number

ADD	R7, R6, R3	; R7 = R6 + R3

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEX RD | MON | 0 | 0 | 0 | 0 | 0 | RD | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | P |

#### SIF – Set Interrupt Flag

Function:

1 => XGIF[XGCHID]

1 => XGIF[RS[6:0]]

Set the output interrupt flag.

Example:

SIF	R6	; Signal Host to release semaphore register?

SIF		; Signal Host that thread is complete

RTS		; Allow Xgate to accept new input interrupt

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIF | INH | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| SIF RS | MON | 0 | 0 | 0 | 0 | 0 | RS |  |  | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | P |

#### STB – Store Byte To Memory Low Byte

Function:

RS.L		M[RB, $OFFS5]

RS.L		M[RB, RI]

RS.L		M[RB, +RI]; RI + 1 => RI

RS.L		M[RS, -RI]; RI – 1 => RI

Stores the low byte of register RS to memory.

Example:

STB	R5, (R1,#9)	; Store R5[7:0] to the address at R1+9

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STB RD, (RB, #IMM8) | IDO5 | 0 | 1 | 0 | 1 | 0 | RD | RB | OFFS5 |  |  | PR |
| STB RD, (RS, RI) | IDR | 0 | 1 | 1 | 1 | 0 | RD | RB | RI | 0 | 0 | PR |
| STB RD, (RS, RI+) | IDR+ | 0 | 1 | 1 | 1 | 0 | RD | RB | RI | 0 | 1 | PR |
| STB RD, (RS, -RI) | -IDR | 0 | 1 | 1 | 1 | 0 | RD | RB | RI | 1 | 0 | PR |

#### STW – Store Word To Memory

Function:

RS	=> M[RB, $OFFS5]

RS	=> M[RB, RI]

RS	=> M[RB, +RI]; RI + 2 => RI

RS	=> M[RS, -RI]	; RI – 2 => RI

Stores the contents of the RD register to memory.

Example:

STB	R5, (R1,#8)	; Store to the address at R1+8

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| STW RD, (RB, #IMM8) | IDO5 | 0 | 1 | 0 | 1 | 1 | RD | RB | OFFS5 |  |  | PR |
| STW RD, (RS, RI) | IDR | 0 | 1 | 1 | 1 | 1 | RD | RB | RI | 0 | 0 | PR |
| STW RD, (RS, RI+) | IDR+ | 0 | 1 | 1 | 1 | 1 | RD | RB | RI | 0 | 1 | PR |
| STW RD, (RS, -RI) | -IDR | 0 | 1 | 1 | 1 | 1 | RD | RB | RI | 1 | 0 | PR |

#### SUB – Subtract without Carry

Function:

RS1 - RS2 => RD

Subtract the contents of RS2 from RS1 using binary subtraction, store the result in RD and update the condition code registers.

Example:

; Do 32 bit subtraction R7:R6 = R5:R4 - R3:R2

; --- Subtract least significant 16 bit words

SUB	R6, R4, R2	; R6 = R4 - R2

; --- Subtract most significant 16 bit words

SBC	R7, R5, R3	; R7 = R5 - R3 - C

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS1[15] & ~RS2[15] & ~RD[15]new) | (~RS1[15] & RS2[15] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RS1[15] & RS2[15]) | (~RS1[15] & RD[15]new) | (RS2[15] & RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUB RD,RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 1 | RD | RS1 | RS2 | 0 | 0 | P |

#### SUBH – Subtract Immediate 8-bit constant high byte

Function:

RD - IMM8:$00 => RD

Subtract a signed 8 bit immediate constant from the high byte of RD and store the result in the high byte of the RD register.

Example:

; --- Subtract 16 bit immediate

SUBL	R6, #LOWBYTE	; R6 = R6 - 8 bit immediate

SUBH	R6, #HIGHBYTE	; R6 = R6 - 16 bit immediate

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RD[15]old & ~IMM8[7] & ~RD[15]new) | (~RD[15]old & IMM8[7] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RD[15]old & IMM8[7] | (~RD[15]old & RD[15]new) | (IMM8[7] & RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUBH RD,#IMM8 | IMM8 | 1 | 1 | 0 | 0 | 1 | RD | IMM8 | P |

#### SUBL – Subtract Immediate 8-bit constant low byte

Function:

RD - $00:IMM8 => RD

Subtract an 8 bit immediate constant from the RD register and store the result in the RD register. (All bits in the RD register may be affected by this operation.)

Example:

; --- Subtract 16 bit immediate

SUBL	R6, #LOWBYTE	; R6 = R6 - 8 bit immediate

SUBH	R6, #HIGHBYTE	; R6 = R6 - 16 bit immediate

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(~RD[15]old & RD[15]new)

C: Set if there is a carry from bit 7 to bit 8 of the result, otherwise cleared.

(~RD[7]old & IMM8[7]) | (RD[7] & ~RD[7]new) | (~RD[7]new  & IMM8[7])

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUBL RD,#IMM8 | IMM8 | 1 | 1 | 0 | 0 | 0 | RD | IMM8 | P |

#### TFR – Transfer from and To Special Registers

Function:

TFR RD, CCR:	CCR + RD[3:0] => RD; 0 => RD[15:4]

TFR CCR, RD:	RD[3:0] => CCR

TFR RD,PC:		PC + 4 => RD

Copies the contents of one RISC core register to another RISC core register.

Example:

TFR	R5, PC	; Save the return address in R5

BRA	SUB_1		; Goto the subroutine code

; More Code

SUB_1

; Subroutine Code

JAL	RD		; Return to the address after the BRA

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
| - | - | - | - |

N: Not affected.

Z: Not affected.

V: Not affected.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TFR RD,CCR | MON | 0 | 0 | 0 | 0 | 0 | RD | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | PP |
| TFR CCR,RS | MON | 0 | 0 | 0 | 0 | 0 | RS | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |  |
| TFR RD,PC | MON | 0 | 0 | 0 | 0 | 0 | RD | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |  |

#### TST – Test Register

Function:

RS - 0 =>  None

--- Compiler alias to the command  SUB R0, RS, R0

Subtract zero from the contents of the RS register and update the condition code registers.

Example:

TST	R6

BEQ	IS_ZERO 	; R6 == 0

BMI	IS_NEG	; R6 is a negative number

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  |  |  |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Set if there is a two’s complement overflow, otherwise cleared.

(RS1[15] & ~RS2[15] & ~RD[15]new) | (~RS1[15] & RS2[15] & RD[15]new)

C: Set if there is a carry from bit 15 of the result, otherwise cleared.

(~RS1[15] & RS2[15]) | (~RS1[15] & RD[15]new) | (RS2[15] & RD[15]new)

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TST RS | TRI | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | RS | 0 | 0 | 0 | 0 | 0 | P |

#### XNOR – Logical Exclusive NOR

Function:

~(RS1 ^ RS2) => RD

Do a bit wise logical XNOR between the RS1 register and the RS2 register and store the result in the RD register.

Example:

XNOR	R6, R0, R6	; R6 = One’s complement (invert) R6

XNOR	R6, R0, R0	; R6 = $FFFF

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XNOR RD,RS1,RS2 | TRI | 0 | 0 | 0 | 1 | 0 | RD | RS1 | RS2 | 1 | 1 | P |

#### XNORH – Logical Exclusive NOR Constant High Byte

Function:

~(RD.H ^ IMM8) => RD.H

Do a bit wise logical XNOR between the 8-bit constant and the high byte of the RD register and store the result in the RD register. The low byte of RD is not affected.

Example:

XNORH	R6, #0	; R6[15:8] = One’s complement (invert) R6

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XNORH RD,#IMM8 | IMM8 | 1 | 0 | 1 | 1 | 1 | RD | IMM8 | P |

#### XNORL – Logical Exclusive NOR Constant Low Byte

Function:

~(RD.L ^ IMM8) => RD.L

Do a bit wise logical XNOR between the 8-bit constant and the low byte of the RD register and store the result in the RD register. The high byte of RD is not affected.

Example:

XNORL	R6, #0	; R6[7:0] = One’s complement (invert) R6

CCR Effect:

| N | Z | V | C |
| --- | --- | --- | --- |
|  |  | 0 | - |

N: Set if bit 15 of the result is set, otherwise cleared.

Z: Set if the result register, RD, is all zero’s, otherwise cleared.

V: Always cleared.

C: Not affected.

Code and Cycles

| ASM Code | Address Mode | Machine Code |  |  |  |  |  |  | Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| XNORL RD,#IMM8 | IMM8 | 1 | 0 | 1 | 1 | 0 | RD | IMM8 | P |

Test Bench

Test Bench Overview

Host

(WISHBONE Master)

Xgate

(WISHBONE Master)

Test Bench Regs

(WISHBONE Slave)

Peripheral Bus

Arbitration

RAM

(Xgate Program Memory)

WB Master Bus

WB Master Bus

ack

ack

WB Slave Bus

xgif[127:1]

This is a simplified drawing of the test bench and how the various components are connected. The intent is to place the Xgate into a system that would be somewhat similar to an actual application system that might use the Xgate.

Top Level Test Bench

Tasks:

Host

The Host module provides a wrapper for the test bench tasks which access the WISHBONE slave bus from the top level test bench code. The basic Host module is the same as used on the PIT (Programmable Interrupt Module), COP (Computer Operating Properly) and was originally copied from the “I2C Controller Core” written by Richard Herveille. Modifications include the addition of a global error counter, improved error messages and the control of the WISHBONE byte select signals for 16/8 bit  bus access.

These tasks are:

wb_write – Activates the WISHBONE Slave bus to complete a write  operation. The write can be ether a 8 or 16 bit operation. Inputs are d, Delay – a, Address – d, Data – s, Size.

wb_read – Cycles the WISHBONE Slave bus to complete a read operation. . The read can be ether a 8 or 16 bit operation. Inputs are d, Delay – a, Address – d, Size. The task returns a data item d, Data which is the value read from the address location.

wb_cmp – This task contains a nested call to the wb_read task and does the additional operation of comparing the value read from the address to a value input to the wb_cmp task. If the data does not match the data read from the addressed location an error signal is generated and the global error counter is incremented. A display message is also added to the simulation run log. The read can be ether a 8 or 16 bit operation. Inputs are d, Delay – a, Address – d, Data  for compare reference – s, Size.

Test Bench Regs

This is a fully functional WISHBONE slave module.

| Name | Address | Width | Access | Description |
| --- | --- | --- | --- | --- |
| CHECK_POINT | 0x00 | 16 | RW | Check Point register used by the Xgate module software to communicate test progress. |
| CHANNEL_ACK | 0x02 | 16 | RW | Channel ack register used by the Xgate module software to indicate the start of an Xgate software thread. Used by the test bench to clear the channel input interrupt chan_req_i |
| CHANNEL_ERROR | 0x04 | 16 | RW | Channel error register used by the Xgate module software to indicate an error condition has been detected while executing the test code. |
| BRKPT_CNTL | 0x06 | 16 | RW | Break point control resister used by the test bench to enable external hardware breakpoints. |
| BRKPT_ADDR | 0x08 | 16 | RW | Breakpoint Address register used by the test bench to set the hardware breakpoint address. |
| TB_SEMAPHR | 0x0A | 16 | RW | Test Bench Semaphore register used by both the Xgate module software or test bench program to cause a change of flow. |
|  |  |  |  | Reserved |
| XGIF_0 | 0x10 | 16 | R | Xgate Interrupt Flag Register #0 xgif[15:1] |
| XGIF_1 | 0x12 | 16 | R | Xgate Interrupt Flag Register #1 xgif[31:16] |
| XGIF_2 | 0x14 | 16 | R | Xgate Interrupt Flag Register #2 xgif[47:32] |
| XGIF_3 | 0x16 | 16 | R | Xgate Interrupt Flag Register #3 xgif[63:48] |
| XGIF_4 | 0x18 | 16 | R | Xgate Interrupt Flag Register #4 xgif[79:64] |
| XGIF_5 | 0x1A | 16 | R | Xgate Interrupt Flag Register #5 xgif[95:80] |
| XGIF_6 | 0x1C | 16 | R | Xgate Interrupt Flag Register #6 xgif[111:96] |
| XGIF_7 | 0x1E | 16 | R | Xgate Interrupt Flag Register #7 xgif[127:112] |

x.1 Check Point Register (CHECK_POINT)

The CHECK_POINT is normally used by software running on the Xgate module to communicate test progress to the test bench. Whenever the CHECK_POINT register is written a message is printed by the test bench:

("\nSoftware Checkpoint #%h -- at vector=%d\n", check_point_reg, vector)

where “check_point_reg” is the value written to the CHECK_POINT register and “vector” is current value of the test bench clock counter.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | R/W | check_point_reg[15:0] |

Reset Value:

CHECK_POINT: 0000h

Table 1: CHECK_POINT Register Bits

x.2 Channel Acknowledge Register (CHANNEL_ACK)

The CHANNEL_ACK is normally used by software running on the Xgate module to communicate to the test bench that a specific software thread has started executing. Whenever the CHANNEL_ACK register is written a test bench task, “clear_channel”, is activated to remove the input interrupt signal.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | R/W | channel_ack_reg [15:0] |

Reset Value:

CHANNEL_ACK: 0000h

Table 1: CHANNEL_ACK Register Bits

x.3 Check Point Register (CHANNEL_ERR)

The CHANNEL_ERR is normally used by software running on the Xgate module to communicate test progress to the test bench. Whenever the CHANNEL_ERR register is written a message is printed by the test bench:

("\n ------ !!!!! Software Checkpoint Error #%d -- at vector=%d\n	 -------", channel_err_reg, vector)

where “channel_err_reg” is the value written to the CHANNEL_ERR register and “vector” is current value of the test bench clock counter. A signal is also sent to increment the global error counter.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | R/W | channel_err_reg [15:0] |

Reset Value:

CHANNEL_ERR: 0000h

Table 1: CHANNEL_ERR Register Bits

x.4 Breakpoint Control Register (BRKPT_CNTL)

The BRKPT_CNTL register will normally used by the test bench to enable hardware breakpoints. This functionality is not currently supported by the test bench or the Xgate module. There is a watch point function supported by the test bench debug module that can be used for software analysis.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:8 | R/W | TRG_ENA[7:0]. ###### Trigger Enable Mask – The TRG_ENA bits are used to individually enable or disable watch point address triggers. ###### ‘0’ Triggers are blocked for these watch points. ‘1’ Triggers are enabled for these watch points. |
| 7:1 | R/W | Reserved for future use. |
| 1 | R/W | DEBUG_ENA. ###### Debug Enable Mask – This bit is the master enable for all debug functions. ###### ‘0’ All debug functions are disabled. ‘1’ All debug functions are enabled. |

Reset Value:

BRKPT_CNTL: 0000h

Table 1: BRKPT_CNTL Register Bits

x.5 Breakpoint Address Register (BRKPT_ADDR)

The BRKPT_ADDR register will normally used by the test bench to enable hardware breakpoints. This functionality is not currently supported by the test bench or the Xgate module.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | R/W | brkpt_addr_reg [15:0] Reserved for future use. |

Reset Value:

BRKPT_ADDR: 0000h

Table 1: BRKPT_ADDR Register Bits

x.6 Test Bench Semaphore Register (TB_SEMPHORE)

The TB_SEMPHORE is used by software running on the Xgate module to communicate to the test bench that a certain point in the code execution has been reached. The Xgate code may be in “infinite loop” waiting for the test bench to respond by writing a new value to the TB_SEMPHORE register to cause an exit from the loop.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | R/W | tb_semaphr_reg [15:0] |

Reset Value:

TB_SEMPHORE: 0000h

Table 1: TB_SEMPHORE Register Bits

x.7 Channel IRQ Register (CHANNEL_XGIRQ_0)

The CHANNEL_XGIRQ_0 is used by the test bench to monitor the state of the xgif_o[15:1] output signals. By using the CHANNEL_XGIRQ_0 register to observe the xgif_o signals the resources of the Host task “wb_cmp” can be used to make the test comparison  for correct state of these pins.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:1 | R | xgif[15:1] |
| 0 | R | Always reads “0” |

Reset Value:

CHANNEL_XGIRQ_0: 0000h

Table 1: CHANNEL_XGIRQ_0 Register Bits

x.8 Channel IRQ Register (CHANNEL_XGIRQ_1)

The CHANNEL_XGIRQ_1 is used by the test bench to monitor the state of the xgif_o[31:16] output signals. By using the CHANNEL_XGIRQ_0 register to observe the xgif_o signals the resources of the Host task “wb_cmp” can be used to make the test comparison  for correct state of these pins.

| Bit # | Access | Description |
| --- | --- | --- |
| 15:0 | R | xgif[31:16] |

Reset Value:

CHANNEL_XGIRQ_1: 0000h

Table 1: CHANNEL_XGIRQ_1 Register Bits

RAM

The RAM module is a generic single port RAM. The bus interface logic is actually contained in the Arbitration module. A key test bench parameter that effects RAM functionality is “RAM_WAIT_STATES” which determines how many bus wait states are required for a RAM access.

The RAM array is 64K x 8bits to simplify loading the program data that was created in a byte wise S-Record format. The RAM output is 16 bits wide and is created by reading the input address and address+1.

The RAM module also contains a task “dump_ram” which can be used to display RAM contents.

Xgate

The Xgate is the DUT (Device Under Test)

Arbitration

The Arbitration module brings together the hardware to interface the WISHBONE master bus signals from the Host and Xgate module to a single WISHBONE slave bus.

Arbitration

State

Machine

RAM

Interface

Address

Decode

RISC

Interface

Sel

RISC

Timeout

Counter

Host

Interface

Host

Timeout

Counter

The main function of the arbitration module is to provide a multiplexer to route the WISHBONE master signals of the selected source to the WISHBONE slave bus. The arbitration state machine takes requests from the possible bus masters and selects which bus master is granted control for the slave bus. Because the Xgate module could possibly take total control of the slave bus while it is running a program to get continuous access to the RAM module to fetch instructions, a timeout counter is included. The timeout counter forces the current bus master to release the slave bus after a number of counts if another master is requesting slave bus access.

The module also functions as a wrapper module for the RAM interface to the WISHBONE slave bus. Finally some additional glue logic is included to decode the slave bus address to form module selects for the various slave bus peripherals. The RAM is the default access decode for any address space that is not specifically decoded for a peripheral.

DEBUGGER

To aid in software development a simple debug module is built into the test bench. The debugger loads watch point addresses stored in RAM after the first RAM initialization. The debugger generates trigger signals that can be watched in the waveform viewer and captures a copy of the CPU registers at each trigger event. The TRG_ENA[7:0] bits in the BRKPT_CNTL register controls an enable for each watch point address. The watch point addresses are generated by the software assembler program when the code is compiled for simulation and stored in the S-Record file. Up to eight addresses can be saved in defined RAM locations for the debugger to use.

Sample Test Program

The following is a segment of ASM test code used to test the Xgate instruction set operation. (The code fragment is from the file /trunk/sw/xgate_test_code/inst_test/inst_test.s) The listed code does a simple test of the BCC (Branch Carry Clear) instruction and a test of the BRA (Branch Always) instruction.

The BCC test has two parts, the first test is structured so that the code should do the required change of flow and the second test is coded so that proper execution will not do a change of flow. If either test fails the code should branch to the error exit and the testbench will log a failed test. This is only a simple test of the BCC bit decoding of the condition code registers, NZOC, a more exhaustive test would try all 16 values of the register to fully verify the instruction functionality.

The BRA instruction is tested in a similar manner. The first test checks the instruction will always cause a change of flow when there is a forward branch (PC + OFFSET) and the second test checks for a backward change of flow (PC – OFFSET).

The sample code shows the basic setup for a test program:

- Setup jump table for each interrupt input

- Define the interrupt code

- Send Checkpoint back to Verilog testbench for logging to follow test progress

- Send message back to Verilog testbench to acknowledge the interrupt and cause the testbench to clear the interrupt input

- Do the targeted instruction test.

- Complete the interrupt code by sending a message to the Verilog testbench to log code segment completion

- Set the Xgate output interrupt flag.

- Define an alternate code ending for an error detect condition that will notify the Verilog testbench to log a failed test.

The ASM code accesses the Verilog testbench resources in the WISHBONE Slave module. These are:

- CHECK_POINT (Address $8000)

- CHANNEL_ACK (Address $8002)

- CHANNEL_ERROR (Address $8004)

- Note: The code is pasted from another file and there is line wrap on some of the longer lines.  Also most of the test code from the file has been deleted for simplicity and to highlight what has been described in the above narrative.

; 345678901234567890123456789012345678901234567890123456789012345678901234

; Instruction set test for xgate RISC processor core

; Bob Hayes - Sept 1 2009

;  Version 0.1 Basic test of all instruction done. Need to improve

;    Condition Code function testing.

CPU	XGATE

ORG	$fe00

DS.W	2 	; reserve two words at channel 0

; channel 1

DC.W	_START  ; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 2

DC.W	_START2	; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 3

DC.W	_START3	; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 4

DC.W	_START4	; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 5

DC.W	_START5	; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 6

DC.W	_START6	; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 7

DC.W	_START7	; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 8

DC.W	_START8	; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 9

DC.W	_START9	; point to start address

DC.W	V_PTR   ; point to initial variables

; channel 10

DC.W	_START10	; point to start address

DC.W	V_PTR   	; point to initial variables

; channel 11

DC.W	_ERROR		; point to start address

DC.W	V_PTR   	; point to initial variables

; channel 12

ORG	$2000 ; with comment

V_PTR	EQU	123

DC.W	BACK_

DS.W	8

DC.B	$56

DS.B	11

ALIGN	1

;-----------------------------------------------------------------------

;   Place where undefined interrupts go

;-----------------------------------------------------------------------

_ERROR:

LDL	R2,#$04    ; Sent Message to Testbench Error Register

LDH   R2,#$80

LDL   R3,#$ff

STB   R3,(R2,#0)

SIF

RTS

;-----------------------------------------------------------------------

;   Test Branch instructions

;-----------------------------------------------------------------------

_START5:

LDL	 R2,#$00    ; Sent Message to Testbench Check Point Register

LDH    R2,#$80

LDL    R3,#$09    ; Checkpoint Value

STB    R3,(R2,#0)

LDL    R3,#$05    ; Thread Value

STB   R3,(R2,#2) ; Send Message to clear Testbench interrupt register

;Test BCC instruction  C = 0

LDL   R2,#$00

TFR   CCR,R2   ; Negative=0, Zero=0, Overflow=0, Carry=0

BCC	_BCC_OK1 ; Take Branch

BRA	_BR_ERR

_BCC_OK1:

LDL   R2,#$01

TFR   CCR,R2   ; Negative=0, Zero=0, Overflow=0, Carry=1

BCC	_BR_ERR  ; Don't take branch

BRA	BRA_FWARD

_BR_ERR:

LDL	R2,#$04    ; Sent Message to Testbench Error Register

LDH   R2,#$80

LDL   R3,#$0a

STB   R3,(R2,#0)

SIF

RTS

_BRA_OK:

LDL	R2,#$00    ; Sent Message to Testbench Check Point Register

LDH   R2,#$80

LDL   R3,#$0a

STB   R3,(R2,#0)

SIF

RTS

BRA_FWARD:

BRA	_BRA_OK    ; Test backward branch calculation

[This section contains an alphabetical list of helpful document entries with their corresponding page numbers.]

Error! No index entries found.